3GPP TSG-SA2 Meeting #161	S2-2402348

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0427 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on Layer-2 link management over PC5 for 5G ProSe Layer-2 UE-to-UE Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to S2-2311980(R2-2311566), it was decided to introduce Local ID in a SRAP header for communication between two end UEs via Layer-2 UE-to-UE Relay. Per RAN2 conclusion, in Data packet for communication between source END UE and target END UE, Layer2 ID is used for PC5 connection between End UE and UE-to-UE Relay and Local IDs used at SRAP header for indicating end to end connection between source End UE and target END UE. There is a E.N. relating to Layer-2 link management over PC5 for Layer-2 UE-to-UE Relay. Editor's note: Whether the LIU between peer Layer-2 End UEs has same message contents as direct PC5 LIU messages is FFS. Link Identifier update procedure is used to update layer 2 ID and other information of End UE for privacy protection. And as Local IDs are sent in the packet without protection (i.e. in cleartext), Local ID pair should be updated together with Layer 2 ID and other identifiers in Link Identity Update procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | LIU procedure between peer Layer-2 End UEs is clarified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete LIU procedure between peer Layer-2 End UEs with Layer2 UE-to-UE Relay. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 6.4.3.7.2	Layer-2 link management over PC5 reference point for 5G ProSe Layer-2 UE-to-UE Relay

For the 5G ProSe Communication via 5G ProSe Layer-2 UE-to-UE Relay as described in clause 6.7.2, the description in clause 6.4.3.7.1 applies.

The message contents over PC5 reference point for unicast mode 5G ProSe Direct Communication as depicted from clause 6.4.3.1 to clause 6.4.3.5 are same for the end-to-end connection between peer 5G ProSe End UEs.

When identifiers used for the unicast mode of 5G ProSe communication over PC5 reference point (e.g. Layer-2 ID and IP address/prefix) of an End UE are updated via LIU procedure with Layer-2 UE-to-UE Relay for privacy requirements, Local IDs pair for the End UE shall be updated at the same time.

# * * * * End of change * * * *