SA WG2 Meeting #S2-161	S2-2402589

26 February - 1 March, 2024, Athens, Greece	(revision of S2-240xxxx)

Source:	ZTE

Title:	KI#1, new solution on the 4G CSG and 5G CAG interworking

Document for:	Approval

Agenda Item:	19.12

Work Item / Release:	FS_5G_Femto / Rel-19

Abstract of the contribution: This paper proposes a new solution to key issue#1 on the 4G CSG and 5G CAG interworking

1. Proposal

It is proposed to accept the changes to TR 23.700-45.

>>>> Start of Change <<<<

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> |
| #1 |  |  |
| #2 |  |  |
| #X | x |  |

>>>> Next Change <<<<

## 6.X	Solution #X: The connected mode mobility between 4G CSG cell and 5G CAG cell

### 6.X.1	Description

This solution is to address the connected mode mobility part of KI#1, investigates any needed enhancements to support the UE moving between CAG cell of 5G Femto and CSG cell.

The high level principle/assumption of this solution can be summarized as follows:

-	The 4G and 5G interworking architecture with N26 interface defined in the 23.501 [2] clause 4.3.1 is applied.

-	There is UE allowed 4G CSG list and 5G CAG list subscription in the UDM/HSS.

-	When the UE registers to 5GS, the UDM/HSS provides the UE allowed 4G CSG list and 5G CAG list to AMF. The AMF create the Mobility restriction list accordingly.

-	When the UE attach to EPS, the UDM/HSS combines the allowed 4G CSG list and 5G CAG list and provides to MME as allowed CSG list.

-	When UE access the 5G CAG cell, it may measure and report the neighbouring CSG cell as CAG cell to NG-RAN.

-	When UE access the 4G CSG cell, it may measure and report the neighbouring CAG cell as CSG cell to eNB.

NOTE:	Alternatively, if the UE does not measure and report the neighbouring CSG/CAG cell as CAG/CSG cell as above bulletin, the normal 4G-5G Handover is performed. After the handover, the UE perform the TAU or registration, the MME/AMF perform the Access control for CSG or CAG.

### 6.X.2	Procedures

The clause 4.11.1.2 of TS 23.502 [3] (4G-5G handover procedure using N26 interface) is re-used with following additional handling.

For the 5GS to EPS Handover:

-	In step 1 in clause 4.11.1.2.1 (Preparation phase), the AMF check whether the target CSG cell is allowed for the UE.

For the EPS to 5GS Handover:

Editor's note:	Additional change is FFS.

### 6.X.3	Impacts on services, entities and interfaces

Editor's note:	This clause captures impacts on existing services, entities and interfaces.

>>>> End of Change <<<<