SA WG2 Meeting #161	S2-2402620

# 26 Feb – 1 Mar 2024, Athens, GR	(was S2-240xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5338 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on expected NG-RAN behaviour when PSIHI is not provided |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 38.415 defines PSIHI as an Optional parameter that can be set to either “true” or “false”: According to the PSIHI definition in 23.501 clause 5.7.7.4: The PDU Set Integrated Handling Information (PSIHI) indicates whether all PDUs of the PDU Set are needed for the usage of the PDU Set by the application layer in the receiver side. PSIHI is an optional parameter. A QoS Flow is associated with at most one PSIHI value per direction. In other words: “true” – means that all PDUs of a PDU Set are needed in the receiver i.e. NG-RAN can drop the remaining PDUs of a PDU Set subsequent to a loss of a PDU from that PDU Set. “false” – means that not all PDUs of a PDU Set are needed in the receiver i.e. NG-RAN should attempt to transmit the remaining PDUs of a PDU Set subsequent to a loss of one or multiple PDUs from that PDU Set. Note that TS 38.413 does not specify the meaning of “true” and “false”, and instead refers to TS 23.501. Clause 5.7.7.4 does not describe the NG-RAN behaviour when the PSIHI is not provided. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 5.7.7.4: clarified that, when PSIHI is not provided, NG-RAN should attempt to transmit the remaining PDUs of a PDU Set subsequent to a loss of one or multiple PDUs from that PDU Set. In other words, NG-RAN should have the same behaviour as when PSIHI is provided and set to “false”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing description of the expected NG-RAN behaviour when PSIHI is not provided to NG-RAN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.7.7.4	PDU Set Integrated Handling Information

The PDU Set Integrated Handling Information (PSIHI) indicates whether all PDUs of the PDU Set are needed for the usage of the PDU Set by the application layer in the receiver side. PSIHI is an optional parameter. When PSIHI is not provided, NG-RAN should attempt to transmit the remaining PDUs of a PDU Set subsequent to loss of PDU(s) from that PDU Set. A QoS Flow is associated with at most one PSIHI value per direction.

# * * * * End of changes * * * *