SA WG2 Meeting #S2-161	S2-2402670

26 February - 3 March, 2024, Athens, Greece

Title:	KI#6 New Sol Support Standalone Data Channel

Source:	Huawei, HiSilicon

Document for:	Approval

Agenda Item:	19.2

Abstract: This paper proposes to provide a solution for KI#6 of FS_NG_RTC_Ph2.

# 1. Introduction/Discussion

The Key Issuse#6 of SA2 R19 FS_NG_RTC_Ph2 depicts as following:

This key issue aims to study enhancements of IMS architecture, interfaces, and procedures to support standalone IMS data channel session. This includes studying following aspects:

-	How to support standalone IMS bootstrap and application data channel sessions without accompanying audio/video/messaging media in an IMS session. This includes studying the establishment and termination of standalone IMS data channel sessions.

-	Study whether and how to negotiate support of standalone IMS data channel between UE and IMS network. The negotiation should include the ability to add audio/video/messaging media to an established standalone IMS data channel.

-	Whether and how to establish an (standalone) application data channel without accompanying bootstrap data channel.

-	Whether and how to establish a (standalone) bootstrap data channel without accompanying application data channel.

-	How to add audio/video/messaging media to an established standalone IMS data channel session.

-	How to remove audio/video/messaging media from an IMS session that also contains IMS data channel media.

-	Whether and how to define new service data in the HSS for support of standalone data channel or extend existing MMTel service data.

This document provides a solution to address the above objective.

# 3. Text Proposal

It is proposed to capture the following changes into TR 23.700-77.

# * * * * First change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | X |  |  |  |  |  |  |  |
| 2 | X |  |  |  |  |  |  |  |
| 3 | X |  |  |  |  |  |  |  |
| 4 | X |  |  |  |  |  |  |  |
| 5 | X |  |  |  |  |  |  |  |
| 6 |  | X |  |  |  |  |  |  |
| 7 | X | X |  |  |  |  |  |  |
| 8 |  |  | X |  |  |  |  |  |
| 9 |  |  | X |  |  |  |  |  |
| 10 |  |  |  | X |  |  |  |  |
| 11 |  |  |  | X |  |  |  |  |
| 12 |  |  |  | X |  |  |  |  |
| 13 |  |  |  | X |  |  |  |  |
| 14 |  |  |  |  |  | X |  |  |
| 15 |  |  |  |  |  | X |  |  |
| 16 |  |  |  |  |  | X |  |  |
| 17 |  |  |  |  |  |  |  | X |
| X |  |  |  |  |  | X |  |  |

# * * * * Second change * * * *

## 6.X	Solution #X: Support Standalone Data Channel

### 6.X.1	Description

This solution addresses the Key Issue#6 "Support of Standalone IMS Data Channel Sessions" in the following aspect:

-	How to support standalone IMS bootstrap and application data channel sessions without accompanying audio/video/messaging media in an IMS session. This includes studying the establishment and termination of standalone IMS data channel sessions.

-	Study whether and how to negotiate support of standalone IMS data channel between UE and IMS network. The negotiation should include the ability to add audio/video/messaging media to an established standalone IMS data channel.

-	Whether and how to establish an (standalone) application data channel without accompanying bootstrap data channel.

-	Whether and how to establish a (standalone) bootstrap data channel without accompanying application data channel.

-	How to add audio/video/messaging media to an established standalone IMS data channel session.

-	How to remove audio/video/messaging media from an IMS session that also contains IMS data channel media.

-	Whether and how to define new service data in the HSS for support of standalone data channel or extend existing MMTel service data.

#### 6.X.1.1	Subscription of Standalone Data Channel

In Rel-18, TS 23.228 [5] has defined the subscription mechanism of IMS Data Channel. The use of standalone IMS Data Channel shall be included as a part of the IMS Data Channel subscription information.

#### 6.X.1.2	Standalone Data Channel Capability Negotiation

It is proposed that the standalone data channel does not need extra capability negotiation. When receiving an SDP offer with standalone data channel, the network or UE that does not support standalone data channel shall return SIP 400 Bad Request response to inform the offerer that it does not recognize the syntax of the standalone DC.

#### 6.X.1.3	Support of Standalone Bootstrap Data Channel

The standalone bootstrap data channel could be used to extract DC application menu, subscriber specific graphical user interface or specific application.

In order to establish standalone bootstrap data channel, the request-URI is set as the PSI of the DCSF. The UE may be configured of the PSI of the DCSF by the HPLMN either via Device Management or in the UICC.

#### 6.X.1.4	Support of Standalone Application Data Channel

The standalone application data channel shall support P2P, P2A/A2P and P2A2P application data channel.

In the P2A/A2P scenario, the request-URI is set as the PSI of the DCSF. DCSF should route the application data channel to the correct DC AS based on the binding information of a DC application.

Standalone application data channel shall only be initiated by DC application. The DC application could be cached on the UE or retrieve from the network through a bootstrap data channel.

The UE shall include version control information (e.g. version number) of a DC application along with binding information. The network, DC AS and terminating UE shall use the version control information to ensure that the both ends of the application data channel are compatible. If the network or terminating UE detects that they are not compatible, it shall reject the application data channel until the both ends have the correct version of the DC application.

NOTE 1:	The version control information will be defined in SA4.

When rejecting the application data channel due to version incompatibility, the network or terminating UE may include  in the SDP indicating the reason why the application data channel is rejected.

NOTE 2:	The format of this tag will be defined in SA4.

The UE may initiate bootstrap data channel along with standalone application data channel. The DCSF will instruct IMS AS to establish bootstrap data channel for both ends in case of version incompatibility. Otherwise the DCSF will instruct IMS AS to establish application data channel and reject the establishment of bootstrap data channel.

#### 6.X.1.5	Adding Audio/Video/Messaging Media to Established Standalone Data Channel

Only the standalone data channel session that contains at least one P2P or P2A2P application data channel could be updated to add audio/video/messaging media through a SIP re-INVITE message.

The standalone data channel session that only contains standalone bootstrap data channel and/or standalone P2A/A2P application data channel cannot be updated to add audio/video/messaging media.

#### 6.X.1.6	Removing Audio/Video/Messaging Media from an IMS Session that Contains IMS Data Channel Media

Only IMS session that contains at least one P2P or P2A2P application data channel could be updated to remove audio/video/messaging media through a SIP re-INVITE message.

The IMS session that only contains bootstrap data channel and/or standalone P2A/A2P application data channel cannot be updated to remove audio/video/messaging media.

### 6.X.2	Procedures

#### 6.X.2.1	Standalone Application Data Channel Establishment when Both Ends have Version Compatible DC App

1.	UE-A starts the standalone application data channel by sending a SIP INVITE/re-INVITE message. The message shall contain the binding information and version control information of the data channel application.

2.	The IMS AS notifies the DCSF when receive the SIP INVITE/re-INVITE message, in the notify the IMS AS shall include the binding information and version control information of the data channel application.

3.	The DCSF compares the binding information and version control information of the data channel application received with those stored in network. The DCSF determines that UE-A has the version compatible data channel application.

NOTE 1:	Whether the binding information and the version control information of the data channel application is stored in DCSF or DCAR is implementation specific and will not be defined in this specification.

4.	The DCSF instructs the IMS AS to continue the standalone application data channel media negotiation.

If UE-A includes the bootstrap data channel media in the SDP, the DCSF will instruct the IMS AS to include a bootstrap data channel media description between originating network and UE-B in the SDP.

5.	IMS AS forwards the SIP INVITE/re-INVITE to the terminating UE.

6.	UE-B compares the binding information and version control information of the data channel application received with those stored in UE-B. UE-B determines that the itself has the version compatible data channel application.

7.	UE-B accepts the standalone application data channel media negotiation.

If UE-B receive the bootstrap data channel media description along with the application data channel media description, it shall reject the bootstrap data channel.

8.	The SIP response forwards to UE-A.

If UE-A includes the bootstrap data channel media in the SDP, the DCSF will instruct the IMS AS to reject the bootstrap data channel.

#### 6.X.2.2 	Standalone Application Data Channel Establishment when Originating UE does not have Version Compatible DC App

1.	UE-A starts the standalone application data channel by sending a SIP INVITE/re-INVITE message. The message shall contain the binding information and version control information of the data channel application.

2.	The IMS AS notifies the DCSF when receive the SIP INVITE/re-INVITE message, in the notify the IMS AS shall include the binding information and version control information of the data channel application.

3.	The DCSF compares the binding information and version control information of the data channel application received with those stored in network. The DCSF determines that UE-A does not have the version compatible data channel application.

4.	The DCSF instructs the IMS AS to reject the standalone application data channel media negotiation.

If UE-A also include the bootstrap data channel media in the SDP, the DCSF may accept the bootstrap data channel establishment so that UE-A may download the latest version DC App. If this is a P2P or P2A2P application data channel establishment request, DCSF shall instruct the IMS AS to also establish the bootstrap data channel between the origination network and UE-B before sending response to UE-A.

5.	IMS AS sends the 200/4xx response to UE-A, and includes the reason why standalone application data channel is rejected.

#### 6.X.2.3 	Standalone Application Data Channel Establishment when Terminating UE does not have Version Compatible DC App

1.	UE-A starts the standalone application data channel by sending a SIP INVITE/re-INVITE message. The message shall contain the binding information and version control information of the data channel application.

2.	The IMS AS notifies the DCSF when receive the SIP INVITE/re-INVITE message, in the notify the IMS AS shall include the binding information and version control information of the data channel application.

3.	The DCSF compares the binding information and version control information of the data channel application received with those stored in network. The DCSF determines that UE-A has the version compatible data channel application.

4.	The DCSF instructs the IMS AS to continue the standalone application data channel media negotiation.

If UE-A includes the bootstrap data channel media in the SDP, the DCSF will instruct the IMS AS to include a bootstrap data channel media description between originating network and UE-B in the SDP.

5.	IMS AS forwards the SIP INVITE/re-INVITE to the terminating UE.

6.	UE-B compares the binding information and version control information of the data channel application received with those stored in UE-B. UE-B determines that the itself does not have the version compatible data channel application.

7.	UE-B sends the SIP 400x response to UE-A, and includes the reason why standalone application data channel is rejected.

If UE-B receive the bootstrap data channel media description along with the application data channel media description, it shall accept the bootstrap data channel.

8.	The SIP response forwards to UE-A.

If the UE-A includes the bootstrap data channel media in the SDP, the DCSF will instruct the IMS AS to accept the bootstrap data channel between DCSF and UE-A during the notification process when IMS received the response.

### 6.X.3	Impacts on Existing Nodes and Functionality

The impacts on the DCSF:

- Support to establish standalone data channel

- Support to compare the version control information send by UE and instruct the IMS AS accordingly.

The impacts on the IMS AS:

- Support to notify DCSF when receive SIP message that only contains IMS data channel media description.

- Support to update SDP to inform UE the reason when standalone application data channel establishment is rejected.

The impacts on the UE:

- Support to establish standalone data channel

- Support to compare the binding information of DC App and version control information to decide whether accept a standalone data channel.

- Support to include the reason in the SDP when rejecting a standalone application data channel.

# * * * * End of changes * * * *