3GPP SA WG2 Meeting #161	S2-2402939

Athens, GREECE, February 26th – March 1st, 2024           	(revision of S2-2401194)

Source:	Nokia, Nokia Shanghai Bell, NTT DOCOMO

Title:	KI#4 Support for multiplexed media traffic

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM Ph2 / Rel-19

Abstract of the contribution: Solution for KI#4 for the case where several RTP media streams and various other protocols are multiplexed into a single end-to-end transport layer traffic flow.

# Discussion

Many XRM applications exchange multiple media streams of various types (video, audio, pose or other sensory information) which may have different QoS requirements. It has become a common practice to multiplex all the streams along with other application traffic into a single (UDP/IP) transport layer traffic flow, i.e., carried in packets sharing the same IP 5-tuple and even the same DiffServ Code Point (DSCP). In this case different QoS treatment cannot be applied to the individual media streams by 5GS, as the packet filtering or detection rules for QoS mapping in both DL and UL direction are only based on the 5-tuple and the DSCP/ToS field.

The SA2 Release 19 XRM Study Item Description introduces this issue in Work Task #2.1 as follows:

WT#2: QoS handling enhancement for XRM service

WT#2.1: Study whether and what enhancements are needed for traffic detection and QoS Flow mapping for 			different media types multiplexed data flows within a single end-to-end transport connection.

In SA2#160 the related Key Issue #4 was agreed to TR 23.700-70:

XR and interactive media services are likely to send data traffic of different media components and with different QoS requirements. Several media streams could be multiplexed on the same end-to-end transport layer connection.

For example, in XR service, several media streams could be multiplexed on a single IP 5-tuple with Transport protocol like IETF QUIC [11], using different QUIC connections or different QUIC streams.

In another example, video and audio RTP streams or different layers of media streams with different QoS requirements are multiplexed into a single transport layer connection with same IP 5-tuple.

Current 5GS QoS Framework does not fit well to support differentiated QoS for the multiplexed traffic flows when they share the same IP 5 tuple.

This key issue proposes study traffic detection and QoS Flow mapping in 5GS for different media streams multiplexed within a single end-to-end transport connection.

- How to identify multiplexed traffic flows with different QoS requirements within a single transport connection.

- How to do QoS Flow mapping for traffic flows with different QoS requirements.

- Whether and what information needs to be provided from AF for traffic detection.

- Whether and how AF provides QoS requirements of different traffic flows to the 5GS.

This document proposes a solution specific for the case where the media streams are carried using RTP or Secure RTP and are multiplexed into the same transport layer traffic flow with each other and with other protocols including RTCP or Secure RTCP, STUN, TURN, DTLS or QUIC according to IETF RFC 5761[a], RFC 5764[b], RFC 7983[c], RFC 8872[d] and RFC 9443[e]. This multiplexing method is commonly supported by RTP-based applications today including those compatible with WebRTC.

The solution allows an AF to provide 5GS identification information and QoS requirements for each RTP-based media stream and also for any of the other protocols within the multiplex, and the 5GS User Plane to use the identification information to provide them the requested QoS treatment. This is achieved by extending the Flow Description information in the AFSessionWithQoS API and the corresponding PCC rule Service Data Flow Filter with the additional identification information. Similar extensions are needed in the Protocol Detection Information (PDI) in the Packet Detection Rules (PDRs) provided by SMF to UPF, and to cover the UL direction also in the UE Packet Filter Set information provided by SMF to the UE within the UE QoS rules.

In addition to RTP-based streams providing specific QoS treatment to DTLS is relevant, as DTLS transports the data sent over the WebRTC data channel, and some XR applications use the data channel for delay- or QoS-critical traffic such as the pose information.

This solution does not provide a way to identify different types of data carried within end-to-end encrypted transport protocols such as QUIC or DTLS, but these protocols are identified just on the protocol level. Solutions to that would belong to Key Issue #2.

# Proposal

A solution is proposed for KI#4 for incorporation in the XRM Ph2 TR23.700-70.

*** First change ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non‑specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policies and Charging control framework for the 5G System; Stage 2".

[5]	IETF RFC 3711: "The Secure Real-time Transport Protocol (SRTP)", March 2004.

[6]	IETF RFC 6904: "Encryption of Header Extensions in the Secure Real-time Transport Protocol (SRTP)".

[7]	IETF RFC 9335: "Completely Encrypting RTP Header Extensions and Contributing Sources".

[8]	IETF draft-ietf-avtcore-rtp-over-quic: "RTP over QUIC (RoQ)".

[9]	IETF draft-ietf-moq-transport: "Media over QUIC Transport".

[10]	IETF experimental draft-ietf-avtext-framemarking: "Frame Marking RTP Header Extension".

[11]	IETF RFC 9000: "QUIC: A UDP-Based Multiplexed and Secure Transport".

[a]	IETF RFC 5761: “Multiplexing RTP Data and Control Packets on a Single Port”.

[b]	IETF RFC 5764:” Datagram Transport Layer Security (DTLS) Extension to Establish Keys for the Secure Real-time Transport Protocol (SRTP)”.

[c]	IETF RFC 7983: “Multiplexing Scheme Updates for Secure Real-time Transport Protocol (SRTP) Extension for Datagram Transport Layer Security (DTLS)”

[d]	IETF RFC 8872: “Guidelines for Using the Multiplexing Features of RTP to Support Multiple Media Streams”.

[e]	IETF RFC 9443: “Multiplexing Scheme Updates for QUIC”.

[f]	IETF RFC 3550: “RTP: A Transport Protocol for Real-Time Applications”.

Editor's note:	References [8], [9] and [10] cannot be formally referenced until published as RFC.

*** END of first change ***

*** Second change (all new text) ***

## 6.X	Solution #X: Support of multiplexed media traffic

### 6.X.1	Key Issue mapping

This solution is for Key Issue #4, which addresses the situation where multiple media streams of the same or different type with potentially different QoS requirements are multiplexed within the same transport layer (L4) traffic flow, meaning they all share the same IP 5-tuple. Applying different QoS treatment to different media streams requires identification of the individual streams and the identification requires inspection of packet header information beyond the 5-tuple.

This solution applies to the case where multiple media streams carried using RTP or Secure RTP are multiplexed into the same transport layer traffic flow potentially with other protocols including RTCP, Secure RTCP, STUN, TURN, DTLS and/or QUIC according to IETF RFC 5761[a], RFC 5764[b], RFC 7983[c], RFC 8872[d] and RFC 9443[e]. This is the approach commonly supported by RTP-based applications today, including those compatible with WebRTC.

In this type of multiplexing, it is possible to identify the different protocols and media streams by specific rules even without inspecting any end-to-end encrypted information or any additional meta-data. DTLS and QUIC can be identified on the protocol level but deeper identification of what they carry is not possible and belongs to Key Issue #2.

Specifically, the solution addresses the following points of KI#4:

XR and interactive media services are likely to send data traffic of different media components and with different QoS requirements. Several media streams could be multiplexed on the same end-to-end transport layer connection.

[…] video and audio RTP streams or different layers of media streams with different QoS requirements are multiplexed into a single transport layer connection with same IP 5-tuple.

- How to identify multiplexed traffic flows with different QoS requirements within a single transport connection.

- How to do QoS Flow mapping for traffic flows with different QoS requirements.

- Whether and what information needs to be provided from AF for traffic detection.

- Whether and how AF provides QoS requirements of different traffic flows to the 5GS.

### 6.X.2	Description

#### 6.x.2.1	Background

Many XRM applications exchange multiple media streams of various types (video, audio, pose or other sensory information) which may have different QoS requirements. It has become a common practice to multiplex all the streams along with other application traffic into a single (UDP/IP) transport layer traffic flow, i.e., carried in packets sharing the same IP 5-tuple and even the same DiffServ Code Point (DSCP).

For applications using RTP or Secure RTP the multiplexing is based on IETF RFC 5761[a], RFC 5764[b], RFC 7983[c], RFC 8872[d] and RFC 9443[e]. These allow any number of (S)RTP streams to be multiplexed over the same UDP/IP traffic flow, along with other protocols specifically including (S)RTCP, STUN, TURN, DTLS and QUIC. DTLS and QUIC may be used for carrying delay critical information alongside with (S)RTP. Especially the WebRTC based applications already today may use the SCTP/DTLS-based data channel for purposes such as transporting pose information from VR glasses to the server performing viewport-dependent rendering.

An example is shown in the Figure below. The Application Server (AS) is sending two RTP video streams and a single RTP audio stream to UE. They are all multiplexed into the same UDP/IP traffic flow along with RTCP, DTLS and STUN. The current, Rel .18 Packet Detection Rules (PDRs) in the UPF only allow identification of the entire UDP/IP flow and mapping it to a single QoS flow. As a result, if PDU Set based QoS handling is beneficial for the RTP video streams it is applied to the QoS flow, and then RTP audio stream and all the other protocols, including lone PDUs (e.g. STUN, RTCP, TURN) are treated using the PDU Set based QoS handling even if that was not desirable.

#### 6.x.2.2	Solution Description

In this solution, the AF can request specific ordinary QoS or PDU Set QoS treatment for traffic flow components (sub-streams) in multiplexed stream it deems require a specified QoS treatment. The sub-streams that require dedicated QoS treatment are mapped into separate QoS Flows. All other PDUs are mapped to a QoS flow providing ordinary QoS. In a basic case for the example below, the AF may send a request for both RTP video streams to be served according to common PDU Set based QoS parameters while the RTP audio stream and the other protocols are served according to common ordinary QoS parameters specified either by the AF or configured in the PCF. In contrast, in an extreme case the AF could send a request for each RTP media stream and also the DTLS-based WebRTC data channel to be served according to their dedicated QoS requirements, with video streams served according to PDU Set based QoS handling.

Figure 6.X.2.2-1: Different protocols and RTP media streams multiplexed into a single UDP/IP traffic flow.

Each UDP/IP datagram only carries one of the protocols and there are rules for how to identify the protocol using the first two bytes of the UDP payload. In case of (S)RTP, it is further possible to identify a specific media stream based on specific RTP header fields. This solution proposes to extend the packet detection information used in the 5G System beyond the IP 5-tuple in such a way that specific protocol(s) and specific RTP media stream(s) can be identified and be mapped to distinct QoS flows according to their QoS requirements. PDU Set based QoS handling may or may not be applied to any of the QoS flows. PDUs not identified as belonging to a specific protocol and hence are unidentified beyond the IP 5-tuple are mapped to a QoS flow providing ordinary QoS as per pre-Rel. 18 QoS.

Any mechanisms for identifying PDU Sets defined in Release 18 such as using the RTP PDU Set Header Extension defined in 3GPP TS 26.522 can be applied.

Note that this solution only applies to applications that use the specific protocols and multiplexing rules standardized in the IETF. These are already in widespread use. This solution does not address how specific information carried within end-to-end encrypted transport protocols such as DTLS or QUIC can be identified. If DTLS or QUIC are multiplexed within the same UDP/IP traffic flow as (S)RTP, distinct QoS handling can however be applied to them on the protocol level.

#### 6.X.2.3	Detection of protocols and media streams

This section explains the protocol and media stream identification rules standardized in the IETF RFCs cited above. The AF may request that one or more of these be detected in the SDF and provide associated QoS or PDU Set QoS requirements. The PCF reflects the AF request in PCC rules provided to the SMF which then determines the QoS flows and reflects the detection parameters in the PDRs.

Detection of the specific protocol

The detection of the specific protocol within a UDP transport flow is possible based on the first byte of UDP payload as specified in RFC 9443. The rules for this are shown in the Figure below taken from the RFC.

Fig 6.X.2.3-1: The first byte of UDP payload is used to identify different protocols.

RTP and RTCP still need to be separated from each other. This is achieved according to the rules provided in RFC 5761 based on the value of the second byte of the UDP payload, which in RTP contains the Marker (M) 1-bit and the Payload Type (PT) 7-bit header fields and in RTCP the 8-bit Packet Type header field. The value of the second byte is guaranteed to be distinct between RTP and RTCP by any implementation compliant with RFC 5761.

Detection of the specific RTP stream

Once the protocol has been determined to be RTP, it is further possible to identify the specific RTP stream it belongs to. This requires the inspection of RTP Synchronization Source (SSRC) and Payload Type (PT) header fields. Depending on the situation, multiple media streams may share the same SSRC value or have the same Payload Type, but each stream multiplexed together is required to always have a unique combination of SSRC and PT values. The Figure below shows these header fields in the baseline RTP header.

Figure 6.X.2.3-2: RTP Headers (IETF RFC 3550)

Summary of the detection rules

In summary the following information is needed to identify the specific protocol and the specific RTP media stream:

- First Byte of the UDP Payload: This identifies the specific protocol, i.e., RTP/RTCP, ZRTP, DTLS, QUIC, STUN or TURN.

- Second Byte of the UDP Payload: In case the first byte identifies the protocol as RTP/RTCP; the second byte is needed to distinguish whether the protocol is RTP or RTCP.

- Synchronization Source (SSRC) and Payload Type (PT) in RTP header: In case the protocol is RTP; SSRC and PT values together always uniquely identify the specific media stream.

### 6.X.2.4	Extensions needed to 3GPP specifications

This section outlines the enhancements needed in the 3GPP specifications to support the identification and differentiated QoS treatment for the different RTP media streams and protocols multiplexed into the same transport layer (UDP/IP) traffic flow. The principle in the solution is to map the multiplexed sub-streams that require differentiated QoS in 5GS into separate QoS Flows and keep rest of the streams in a single QoS Flow. To minimise the impacts to the current procedures, the solution may be built on top of the Rel-18 multi-modal procedures. In the solution, the AF provides the PCF with a partial flow description (e.g. IP 5-tuple) that is common for all multiplexed sub-streams, and additional flow identification information for each single-modal sub-stream that require differentiated handling. The PSA UPF uses these single-modal sub-stream specific flow identification information to identify and map the single-modal sub-streams into QoS Flows. The AF may provide also the PDU Set QoS Parameters separately for each single-modal sub-stream where they apply, in the same way as in Rel-18. The NG-RAN uses the PDU Set QoS Parameters assigned for the QoS Flows as in Rel-18.

With these enhancements the AF can request and 5GS can deliver dedicated QoS for any RTP media stream or any protocol within the multiplexed IP traffic flow.

What is required is:

- To extend the information used to identify IP traffic flows according to the rules outlined in the previous section in the AF, PCF, SMF, UPF and UE. It should be noted that the extensions do not need to change at all how the overall logic of the functions and the procedures work, rather only the identification information is extended.

- Enable the AF to request QoS parameters or PDU Set QoS parameters for each instance of identified protocol/ sub-stream within the multiplexed IP traffic flow. As described above, that may entail extending multi-modal procedures to allow the AF to submit the requirements in a single AF request, or having separate AF requests for each instance.

- PDUs that are not explicitly identified as using the extended identification information and hence are unidentified beyond the IP 5-tuple are mapped to a QoS flow providing either ordinary or PDU Set QoS as per Rel-18 procedures.

The flow identification information is extended by three new fields. These fields may be specified for each RTP media stream and protocol combination for which distinct QoS treatment is required:

- Protocol: This field can have one or multiple of the following values: “RTP”, RTCP”, “ZRTP”, “DTLS”, “STUN”, “TURN” and “QUIC”.

The field denotes the application/transport protocol carried on top of UDP/IP. A UDP/IP packet matches the Flow Description or Packet Filter with this field based on the rules defined in IETF RFC 9443 [e] and RFC 5764 [b].

NOTE: Both RTP and Secure RTP are covered by the Protocol field value “RTP” as their identification rules are identical.

- Rtp-ssrc: This field is only valid if the Protocol field has value “RTP”. It can include one or multiple 32-bit unsigned integer values.

The field denotes the Synchronization source (SSRC) header field value in the RTP header as defined in IETF RFC 3550 [f]. An RTP/UDP/IP packet matches the Flow Description or Packet Filter with this field if the packet’s RTP SSRC header value is equal to one of the field values.

- Rtp-pt: This field is only valid if the Protocol field has value “RTP”. It can include one or multiple 8-bit unsigned integer values.

The field denotes the Payload Type header field value in the RTP header as defined in IETF RFC 3550 [f]. An RTP/UDP/IP packet matches the Flow Description or Packet Filter with this field if the packet’s RTP PT header value is equal to one of the field values.

Examples:

- protocol=“dtls” : identifies DTLS packets

- protocol=["stun”, “turn”, “rtcp”, “dtls”]: identifies these non-RTP packets

- protocol=”rtp”, rtp-ssrc=”1234567890”, rtp-pt=”99”: identifies a specific RTP media stream

The identification information extension needs to be applied to the following interfaces and functions:

- Nnef_AFSessionWithQoS service in AF/NEF interface: The Flow Description information is extended so that the AF can target its QoS requirements to a specific RTP media stream or protocol. The same extension is applied also when the Flow description is used in services exposed by PCF or TSCTSF.

- PCC rules in the PCF/SMF interface: The Service Data Flow Filter is extended to allow the PCC rules to be targeted to a specific RTP media stream or protocol.

- Packet Detection Information (PDI) in the Packet Detection Rules (PDR) in the SMF/UPF interface: The PDI is extended so the SMF can configure the UPF to detect the specific RTP media stream or protocol carried within an IP packet in the DL direction and map that to a specific QoS treatment (QoS flow).

- The Packet Filter Set information in QoS rules provided by SMF to UE is extended so the SMF can configure the UE to map a specific RTP media stream or protocol carried within an IP packet in the UL direction to a specific QoS treatment (QoS flow).

NOTE: The UE QoS rule extension is only relevant if traffic in the UL direction is in scope.

For the above interfaces and procedures the flow identification information with the new fields may be incorporated within a new “Application Layer Packet Filter” parameter that exists in parallel to the IP Packet Filter. The new parameter is used in combination with the existing IP Packet Filter with similar semantics. The new parameter is added to the following main services and interfaces:

  - Nnef_AFSessionWithQoS: Application Layer Flow Description

  - Npcf_SMPolicyControl: Application Layer Service Data Flow Filter

  - N4 (SMF/UPF PFCP): Application Layer Packet Filter (Set)

### 6.X.3	Procedures

Figure 6.X.3-1: Procedure for providing traffic flow identification information and QoS requirements for individual RTP media streams or protocols in case several media streams and protocols are multiplexed within the same UDP/IP traffic flow.

- AF requests QoS for specific traffic flows using the Nnef_AFSessionWithQoS service. In addition to providing for each traffic flow the ordinary Flow Description (IP 5-tuple, IPv4 ToS or IPv6 TC) to identify a particular UDP/IP traffic flow it may include the extended Application Layer Flow Description fields (“protocol” and “rtp-ssrc” and/or “rtp-pt”) to identify a particular set of protocols or RTP media streams within the UDP/IP traffic flow. For each of these “application layer” traffic flows it provides its requested QoS parameters that can be either traditional QoS parameters or PDU Set QoS parameters. For the flows it provides PDU Set QoS parameters it can optionally include a Protocol Description. If Protocol Description is provided, it must match with the information in the Application Layer Flow Description, where applicable. The AF may in addition provide the flow description and QoS requirements without the additional Application Layer Flow description. PDUs that match the flow description but do not match any Application Layer flow descriptions would be handled according to the specified QoS requirements.

Example:

  - Flow Description = {source IP, destination IP, udp, source port, destination port}, Application Layer Flow description = {protocol=”rtp”, pt=”97”}, PDU Set QoS parameters = {PSDB, PSER, PSIHI, …}, Protocol Description. // video

  - Flow Description = {source IP, destination IP, udp, source port, destination port}, Application Layer Flow description = {protocol=”rtp”, pt=”98”}, QoS parameters = {PDB, PER …}. // audio

  - Flow Description = {source IP, destination IP, udp, source port, destination port}, Application Layer Flow description = {protocol=”dtls”}, QoS parameters = {PDB, PER …}. // data channel

- NEF provides the corresponding information to the PCF.

- PCF authorizes the QoS request, generates the PCC rules, and provides them to SMF. The Service Data Flow Filters in the PCC rules include the extended Application Layer Service Data Flow Filter fields (“protocol” and “rtp-ssrc” and/or “rtp-pt”) in addition to the IP packet filters. The PCF also specifies a PCC rule for PDUs that match the flow description but none of the application Layer Flow descriptions.

- Based on the PCC rules, SMF sends QoS rules for the uplink traffic. QoS rules include the extended Application Layer Packet Filters when required, with the new fields (“protocol” and “rtp-ssrc” and/or “rtp-pt”).

- Based on the PCC rules, the SMF creates the PDR, FAR and QER rules and provides them to the UPF. When required, the Packet Detection Information in PDR includes the extended Application Layer Packet Filters with the new fields (“protocol” and “rtp-ssrc” and/or “rtp-pt”).

When UPF receives a DL packet it matches it with the existing IP packet filters in the PDRs. If the IP packet filter matches the packet and the PDR also contains the extended Application Layer Packet Filters, the packet is matched against them. If they also match, the packet matches the PDRs and as currently specified, the UPF treats the packet according to the associated FAR and QER that provide, the information to which QoS flow the packet should be mapped to, and whether marking for PDU Set information is applied or not. If none of the PDRs with Application Layer Packet Filters match the packet, the PDR with the IP packet filter and no Application Layer Filter is matched, and the packet is handled with the associated FAR and QER.

When UE receives a UL packet it matches it with the existing IP packet filters in the QoS rules. If the IP packet filter matches the packet and the QoS rule also contains the extended Application Layer Packet Filters, the packet is matched against them. If they also match, the packet matches the QoS rule and the UE treats the packet accordingly, e.g., the UE maps it to a specific QoS flow. If none of the QoS rules with Application Layer Packet Filters match the packet, the QoS rule with the IP packet filter and no Application Layer Filter is matched and the packet is handled accordingly.

### 6.X.4	Impacts on services, entities and interfaces

The main impacts are:

- 	AF – Provides the QoS requirements for the specific protocols or RTP media streams using AFSessionWithQoS 	by applying the extended Flow Description information.

-	NEF – Receives the extended Flow Description information from AF and passes it to PCF.

-	PCF – Receives the extended Flow Description information from the AF (or NEF) and includes the corresponding extended Service Data Flow Filter information in PCC rules to SMF.

-	SMF – Receives the extended Service Data Flow Filter information in PCC rules from PCF and includes the corresponding extended Packet Detection Information to UPF in Packet Detection Rules and in case of UL direction the extended Packet Filter Set information in QoS Rules to UE.

-	UPF – Receives PDRs with the extended Packet Detection Information from the SMF and applies it for packet detection. The extended information is used only if the IP packet matches with the “ordinary” IP Packet Filter Set in the PDR.

-	UE – Receives QoS rules with the extended Packet Filter Set information in a QoS rule and applies it for QoS flow mapping. The extended information is used only if the IP packet matches with the “ordinary” IP Packet Filter Set in the QoS rule.

-	RAN – No Impact

*** END of second change ***