3GPP SA WG2 Meeting #S2-161	S2-2403040

# Athens, Greece, 26 February - 3 March, 2024		(revision of S2-2402432)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1045 | rev | 2 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding Analytics Information in the ML Model Monitoring service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom, Huawei, HiSilicon, NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When MTLF determines it needs the ML model accuracy information from an AnLF, it should know which Analytics ID and the corresponding Analytics filter that the model is used for. Otherwise, it is difficult for the NWDAF to use the ML model monitoring results due to lack details information about what analytics the model is used for. In the current spec, MTLF may not be able to associate an ML model to a specific analytics filter and analytics target due to the following two reasons: 1, it can happen that one model is used for different analytics targets and filters. For example, an ML model is suitable for both UE A and UE B, if UE A and UE B has similar characteristics. Similarly, an ML model can be suitable for both area A and area B, if two areas have similar characteristics. 2, When AnLF registers its model monitoring capability and action into MTLF, there may not exist a corresponding model subscription from the AnLF. This is because the AnLF gets the model via Nnwdaf_MLModelInfo service, or the ML model subscription is terminated. To tackle the issue in a simple way, we propose to add the analytics filter information and the analytics target in the Nnwdaf_MLModelMonitor Service. Otherwise, it is difficult for the NWDAF to use the ML model monitoring results due to lack details information about what analytics the model is used for. Based on the discussion in SA2#160AHE, most companies can accept AnLF provides more information, e.g. Analytics ID, analytics filter and target information, to MTLF for ML Model monitoring. The only concern is whether the ML model Id is enough for MTLF to point to an ML model. To address this concern, this paper proposes to make Analytics ID optional as well so that the AnLF could provide this kind of information as a reference to MTLF for model monitoring if necessary. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding analytics filter and target information in the Nnwdaf_MLModelMonitor Service. Based on the S2-2400915r02 in SA2#160AHE, this CR changes Analytics ID from required parameter to optioanl parameter. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is difficult for the NWDAF to use the ML model monitoring results due to lack details information about what analytics the model is used for. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1B.4, 6.2E.3.2, 6.2E.3.3, 7.9.2, 7.9.4, 7.9.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.1B.4	Contents of Analytics Context

The Nnwdaf_AnalyticsInfo_ContextTransfer service operation is used to transfer analytics context from a source NWDAF instance to a target NWDAF instance, whereby the target NWDAF consumes the Nnwdaf_AnalyticsInfo_ContextTransfer service operation produced by the source NWDAF instance to request the analytics context as depicted in Figure 6.1B.3-1.

The consumers of the Nnwdaf_AnalyticsInfo_ContextTransfer service operation (as specified in clause 7.3.3) provide the following input parameters:

-	A list of analytics context identifier(s): identify a set of analytics context that are available at the NWDAF instance providing this service and that are requested to be transferred to the consumer NWDAF instance. The analytics context identifier is provided as the following:

-	Subscription Correlation ID: identifies the analytics subscription for which the related analytics context is requested; or

-	A set of SUPI and associated Analytics ID for UE related Analytics; or

-	An Analytics ID for NF related Analytics.

-	[OPTIONAL] Requested Analytics Context Type per analytics context identifier: indicates which part of the analytics context the consumer wishes to receive. Following values are specified:

-	Pending output Analytics;

-	Historical output Analytics;

-	Analytics subscription aggregation information;

-	Data related to Analytics;

-	Aggregation related information;

-	ML Model related information;

-	Analytics accuracy related information.

-	ML Model accuracy related information.

NOTE:	A list of "analytics context identifier(s)" can be provided by the source NWDAF to the target NWDAF in an analytics subscriptions transfer request as described in clause 6.1B.2.2. Information allowing to identify an analytics context can also be provided by the NWDAF consumer to the target NWDAF in the Nnwdaf_AnalyticsSubscription_Subscribe request and based on this information the target NWDAF derives the "analytics context identifier", as defined in clause 6.1B.2.1.

The producer NWDAF provides to the consumer of the Nnwdaf_AnalyticsInfo_ContextTransfer service operation (as specified in clause 7.3.3), the output information listed below:

-	(Set of) Analytics context matching the input parameters of the Nnwdaf_AnalyticsInfo_ContextTransfer request. If no Requested Analytics Context type parameters are available in the request, all available analytics context types are sent. Analytics context includes the following information parts, if available:

-	Analytics related:

-	Pending output analytics (i.e. not yet notified to the consumer).

-	Historical output analytics information. The content of the output analytics is specified in clause 6.1.3 as output information of the Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request service operations.

-	Timestamp(s) of the last output analytics provided to the analytics consumer(s). Value is set to 0 if no output analytics had been sent yet.

-	Analytics subscription aggregation information (only provided when analytics context is related to analytics aggregation): information about the analytics subscriptions that the source NWDAF has with the NWDAFs that collectively serve the transferred analytics subscription, which includes IDs and analytics metadata information of these NWDAFs for specific Analytics ID(s) and optionally input parameters of analytics exposure as defined in clause 6.1.3.

-	Data related to Analytics:

-	Historical data that is available at the source NWDAF and that is related to the analytics to be handed over to the target NWDAF. If available, the time period of the collected data, NF ID(s) of the data source(s) and information (e.g. filter and event reporting parameters) on the subscriptions with those data sources which were used to generate this historical data.

-	Aggregation related information: Related to analytic consumers that aggregate analytics from multiple NWDAF subscriptions:

-	(Set of) NWDAF identifiers of NWDAF instances used by the NWDAF service consumer when aggregating multiple analytic subscriptions.

-	ML Model related information:

-	ID(s) of NWDAF(s) containing MTLF: Instance/Set ID(s) of the NWDAF(s) containing MTLF from which the source NWDAF currently subscribes to the ML Model Information used for the analytics.

-	Optionally, file address(es) of the trained ML model(s), which is included only when the source NWDAF itself provides the trained ML model(s) for the analytics subscription(s) for which the related analytics context is requested.

-	Analytics Accuracy related information: The information is related to the parameters of the Analytics Accuracy Request Information and status of the analytics subscription due to NF consumer and NWDAF interactions pausing or resuming the analytics output. It includes:

-	Timestamp(s) of the last analytics accuracy information provided to the analytics consumer(s). Value is set to 0 if no analytics accuracy information had been sent yet;

-	Indication whether analytics subscription is paused;

-	Remaining time window of paused analytics subscription;

-	Ground truth information: data types and data sources of the ground truth per analytics used for the accuracy information computation.

-	ML Model accuracy related information: The information is related to the parameters of the ML Model Accuracy Subscription Information requested by a NWDAF containing MTLF. It includes:

-	original Subscription Correlation ID for the ML Model accuracy information associated with the ML model and/or analytics ID at the source NWDAF containing AnLF;

-	Analytics ID, Target of Analytics Reporting and the corresponding Analytics filter associated with the original ML Model Monitoring subscription at the source NWDAF containing AnLF;

-	NWDAF containing AnLF NF ID of source NWDAF;

-	The parameters used for the subscription for ML model accuracy information for the given ML Model at the source NWDAF containing AnLF.

# * * * * Second change * * * *

#### 6.2E.3.2	Procedures for registering the monitoring of the analytics accuracy of an ML Model

When an NWDAF containing AnLF starts making use of an ML model and it has the ability either to monitor the analytics accuracy of the ML model, or to deliver Analytics feedback information for the analytics generated by the ML model, it registers with the NWDAF containing MTLF, that is responsible for training/updating this ML model.

When the NWDAF containing AnLF is no longer using the ML model or monitoring the accuracy of the analytics generated by that ML model for the Analytics ID, it de-registers it with the responsible NWDAF containing MTLF.

Figure 6.2E.3.2-1 illustrates the procedure by which an NWDAF containing AnLF registers with an NWDAF containing MTLF that it is starting to make use and monitor the analytics accuracy of an ML model. A new Nnwdaf_MLModelMonitor_Register service operation is used for that purpose.

Figure 6.2E.3.2-1: Procedure for ML Model monitoring registration

An NWDAF containing AnLF may start monitoring the accuracy of an ML model based on local policy or request from its service consumer.

1-2.	The NWDAF containing AnLF sends an Nnwdaf_MLModelMonitor_Register request to an NWDAF containing MTLF (NF ID of the NWDAF containing AnLF, unique identifier of the ML model,  optionally: subscription endpoint of the Nnwdaf_MLModelMonitor_Subscribe service operation at the NWDAF containing AnLF, Analytics ID, Target of Analytics Reporting, Analytics filter). The NWDAF containing MTLF is now aware of the NF ID of the NWDAF containing AnLF that is monitoring the accuracy of that ML model.

If the NWDAF containing AnLF is a target NWDAF in analytics transfer procedure (as defined in clause 6.1B), based on the ML Model accuracy information received from source NWDAF containing AnLF, the NWDAF containing AnLF also includes in the Nnwdaf_MLModelMonitor_Register service request the ML Model accuracy transfer indication, which includes the original Subscription Correlation ID for the ML model accuracy information provided by the source NWDAF containing AnLF and the source NF ID of the NWDAF containing AnLF.

NOTE 1:	These parameters support the NWDAF containing MTLF to map the registration of a new NWDAF containing AnLF with an existing subscription for consumption of ML model accuracy information from a previous NWDAF containing AnLF (i.e. source NWDAF containing AnLF which as described in steps 3-4 may provide a termination indication), enabling NWDAF containing MTLF to reassociate the data from the previous subscription to the new the subscription for ML Model accuracy provisioning at the new NWDAF containing AnLF.

3-4.	When the NWDAF containing AnLF is no longer using the ML model, it sends an Nnwdaf_MLModelMontior_Deregister service operation.

If NWDAF containing AnLF is registered with a NWDAF containing MTLF, is a source NWDAF containing AnLF in an analytics transfer procedure (as defined in clause 6.1B) and is no longer using the ML model, the NWDAF containing AnLF sends Nnwdaf_MLModelMontior_Deregister service operation request including the ML Model accuracy provisioning termination information, which includes: a termination indication, the termination cause set to analytics transfer and optionally the NWDAF containing AnLF NF ID of the target NWDAF.

NOTE 2:	The ML Model accuracy termination information is used by the NWDAF containing MTLF to determine whether the termination request is from the source NWDAF containing AnLF. If so, the NWDAF containing MTLF will not delete any data immediately upon receiving of a de-registration request. Then the NWDAF containing MTLF is able to associate the data from the source NWDAF containing AnLF to the target NWDAF containing AnLF.

#### 6.2E.3.3	Procedures for monitoring the analytics accuracy of an ML model

An NWDAF containing MTLF, due to the registration of monitoring of the analytics accuracy of an ML model received from NWDAF containing AnLF and local policies, subscribes to the NWDAF containing AnLF for receiving notifications of either the accuracy of the ML Model, or Analytics feedback information of the ML model. The NWDAF containing MTLF may get the Subscription endpoint address of the NWDAF containing AnLF from the information received in a previous registration or through a service discovery procedure at the NRF.

Figure 6.2E.3.3-1 illustrates the procedure either for monitoring the analytics accuracy of an ML model or for delivery of Analytics feedback information of an ML model. Nnwdaf_MLModelMonitor_Subscribe and Nnwdaf_MLModelMonitor_Notify service operations are used for the purposes. A service consumer, i.e. an NWDAF containing MTLF, subscribes at a service producer, i.e. an NWDAF containing AnLF, to be notified when either the analytics accuracy of the previously provisioned ML model is not sufficient, or Analytics feedback information is retrieved from analytics consumer NF.

Figure 6.2E.3.3-1: Procedure for monitoring the analytics accuracy of an ML model

0.	Upon the reception of an Nnwdaf_MLModelMonitor_Register request and based on local policy, the NWDAF containing MTLF determines to subscribe to the analytics accuracy monitoring for the ML model as defined in clause 5C.1.

1.	The NWDAF containing MTLF sends an Nnwdaf_MLModelMonitor_Subscribe request (unique identifier(s) of the ML model(s) to be monitored, accuracy metrics to be monitored, optionally Reporting Threshold(s), Analytics ID, Target of Analytics Reporting and Analytics filter for each ML model identifier or Reporting Period) to an NWDAF containing AnLF subscription endpoint.

When the NWDAF containing MTLF determines during the registration process described in clause 6.2E.3.2 that a subscription request for ML model accuracy monitoring to an NWDAF containing AnLF is related to a previous subscription for ML model accuracy information to a different NWDAF containing AnLF (due to changes in the provider of the ML accuracy monitoring for a given ML model, as an effect of analytics transfer among NWDAFs containing AnLF), the NWDAF containing MTLF may use as base for the new subscription request at the new NWDAF containing AnLF the parameters associated with the original subscription identification for the ML model accuracy information that was received in the registration request of the new NWDAF containing AnLF, as described in steps 1-2 of clause 6.2E.3.2.

2.	The NWDAF containing AnLF sends a response to the NWDAF containing MTLF.

3.	The analytics consumer NF may send Analytics feedback information to the NWDAF containing AnLF as described in clause 6.1.1.

4.	When step 1 is triggered, the NWDAF containing AnLF may start monitoring the analytics accuracy of the ML model(s), if it not started yet.

NOTE 1:	The NWDAF containing AnLF can monitor the analytics accuracy in many ways: e.g. comparing predictions of ML model and its corresponding ground truth data, comparing changes in internal configuration for the analytics ID generation, previous existent records of analytics accuracy information etc.

5.	The NWDAF containing AnLF determines whether the analytics accuracy of the ML model is insufficient, i.e. deviation of the output analytics using the trained ML model from ground truth data (which are collected from Data Producer NF corresponding to analytic ID requested at the time which the prediction refers to) is greater than the Reporting Threshold(s) (which are locally configured or received in the Subscribe request), or the Reporting Period indicated in the Subscribe request is reached.

6.	Either the Analytics feedback information is retrieved at step 3 or the NWDAF containing AnLF detects the analytics accuracy of ML model is insufficient at step 5, the NWDAF containing AnLF sends an Nnwdaf_MLModelMonitor_Notify request to the notification endpoint (e.g. the NWDAF containing MTLF). The Notify request includes either Analytics feedback information, or the monitored ML model accuracy information of the ML model (e.g. unique identifier(s) of the ML model(s) to be monitored, Analytics ID, Target of Analytics Reporting and Analytics filter for each ML model identifier, a deviation value which indicates the deviation of the predictions generated using the ML model(s) from the ground truth data and the network data when the deviation occurs (which can be used by the NWDAF containing MTLF for possible ML model retraining) and the number of inferences that were performed during the time interval between Nnwdaf_MLModelMonitor_Register request and the Notify request or between the time of last Notification message and the time of the current Notification message) and optionally an indication that the analytics accuracy of the ML model does not meet the requirement of accuracy for the ML model.

7.	The NWDAF containing MTLF sends a response.

8.	The NWDAF containing MTLF determines whether the ML model is degraded or not based on the notification at step 6. If the notification contains Analytics feedback information, the NWDAF containing MTLF may determine ML model degradation based on the procedures as described in clause 6.2E.2. Otherwise when the NWDAF containing MTLF has received the multiple analytics accuracy information, from one or more NWDAFs containing AnLF, it may consider that the ML model is degraded/to be updated (i.e. enough number analytics accuracy information received from one or more NWDAFs containing AnLF, indicating insufficient analytics accuracy).

NOTE 2:	The actual mechanism for the NWDAF containing MTLF for determining the degradation of the ML model degradation is an internal procedure of the NWDAF containing MTLF, e.g. the NWDAF containing MTLF calculate a global accuracy based on the analytics accuracy information and the number of inferences received from multiple NWDAFs containing AnLF.

9.	When an ML model is considered degraded / to be updated at step 8, the NWDAF containing MTLF re-trains the existing ML model or selects a new ML model. If the network data was not included in the Nnwdaf_MLModelMonitor_Notify request of step 5, the NWDAF containing MTLF may request data from the NWDAF containing AnLF, ADRF and/or other 5GS entities as specified in clause 6.2 and use the collected data for ML model retraining. The NWDAF containing MTLF notifies the NWDAF(s) containing AnLF with the updated trained ML Model Information by invoking Nnwdaf_MLModelProvision_Notify service operation, as described in clause 6.2A.

# * * * * Third change * * * *

### 7.9.2	Nnwdaf_MLModelMonitor_Subscribe service operation

Service operation name: Nnwdaf_MLModelMonitor_Subscribe

Description: Subscribes to NWDAF for the monitored ML model accuracy (i.e. Analytics accuracy for an ML model as described in clause 6.2E.3.3) information and Analytics feedback information for the analytics generated by the NWDAF with specific parameters.

Inputs, Required: (set of) Unique ML Model identifier(s), Notification Target Address (+ Notification Correlation ID).

Inputs, Optional: Subscription Correlation ID (in the case of modification of the ML model monitor subscription), Accuracy metrics to indicate the metrics to calculate the accuracy information, reporting period to indicate the reporting periodicity in which the monitored ML Model accuracy information shall be reported, Accuracy reporting threshold to indicate the reporting condition above which the accuracy information shall be reported, Analytics ID, Target of Analytics Reporting and Analytics filter.

Analytics ID, Target of Analytics Reporting and Analytics filter for each ML model identifier should reflect the corresponding information received in the Nnwdaf_MLModelMonitor_Register request from the NWDAF containing AnLF to which the current NWDAF containing MTLF subscribes.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription), Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs, Optional: None.

# * * * * Fourth change * * * *

### 7.9.4	Nnwdaf_MLModelMonitor_Notify service operation

Service operation name: Nnwdaf_MLModelMonitor_Notify.

Description: NWDAF notifies the monitored ML model accuracy (i.e. Analytics accuracy for an ML model as described in clause 6.2E.3.3) information and Analytics feedback information for the analytics generated by the NWDAF to the consumer instance which has subscribed to the specific NWDAF service.

Inputs, Required: Notification Correlation Information, at least one of the following:

-	the monitored ML model accuracy information which includes:

-	Unique ML model identifier;

-	Monitoring interval: time interval during which the ML model accuracy monitoring was conducted;

-	Monitored Analytics accuracy of the ML Model and a deviation value which indicates the deviation of the predictions generated using the ML model(s) from the ground truth data;

-	Number of inferences that were performed during the monitoring interval;

-	Accuracy metrics as requested in Subscribe service operation.

-	Analytics feedback information: indicates that the consumer NF of the analytics generated by the provisioned ML model has taken an action(s) influenced by the analytics and includes the following parameter(s):

-	Corresponding Analytics ID(s) which has been used for taking an action(s);

-	Corresponding ML Model identifier(s) which has been used for generating Analytics;

-	Indication whether the action will affect on ground truth data (if available);

-	Time stamp(s) when the action(s) are taken.

Inputs, Optional:

-	Input data used for inferencing indicated by DataSetTag with ADRF ID when the prediction generated from the ML Model is not correct (which can be used by the NWDAF containing MTLF for possible ML model retraining);

NOTE:	How MTLF/AnLF determines whether the prediction is correct one is up to implementation.

-	An indication that the analytics accuracy of the ML model does not meet the requirement of accuracy for the ML model.

-    Analytics ID;

-    Target of Analytics Reporting;

-    Analytics filter for which the model is used for.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

### 7.9.5	Nnwdaf_MLModelMonitor_Register

Service operation name: Nnwdaf_MLModelMonitor_Register

Description: The consumer registers the use and monitoring capability for an ML model at an NWDAF containing MTLF.

Inputs, Required: Consumer NF ID, Unique ML Model identifier.

Inputs, Optional: Endpoint address of the Nnwdaf_MLModelMonitor_Subscribe service operation, ML Model accuracy transfer indication as defined in clause 6.2E.3.2, Analytics ID, Target of Analytics Reporting and the Analytics filter for which the model is used for.

Outputs, Required: ML model monitoring registration ID.

Outputs, Optional: None.

# * * * * End of changes * * * *