3GPP TSG-WG SA2 Meeting #161	S2-2403095

Athens, GR, 26th Feb – 1st Mar, 2024	(revision of S2-2401979)

Source:	Huawei, HiSilicon, Lenovo

Title:	KI#5: New Sol: Support of dynamic change in traffic characteristics

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM_Ph2 / Rel-19

Abstract: This paper intends to propose a high-level solution to support the dynamic changes in traffic characteristics.

# 1. Introduction

KI#5 is to study whether and how to support the dynamic changes in traffic characteristics as following:

-	Identify what traffic characteristics are dynamically changed.

-	Whether and how the 5G network can be enhanced to know about dynamic changes in traffic characteristics of GBR and non-GBR flows.

-	What information from the Application in the DN, if any, is needed by the 5G network to be able to know the dynamic changes in traffic characteristics and how this information is provided by the Application in the DN.

-	At what granularity does the 5GS need to know of changes in traffic characteristics (e.g., QoS Flow granularity).

-	What handling may be needed when the 5G network knows about changes in the traffic characteristics.

This paper intends to propose a solution to support the dynamic changes in traffic characteristics, including the pre-authorization of the QoS policy, detection of the changed traffic characteristics and the adaptive QoS adjustments to the dynamic changes in traffic characteristics.

Updated on S2 #161: To clarify the authorized MDBV is a single value.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-70.

# * * * * First change, all new texts * * * *

## 6.X	Solution #X: Support of dynamic change of traffic burst size

### 6.X.1	Key Issue mapping

The solution addresses Key Issue #5: QoS Handling when Traffic Characteristics Change Dynamically.

### 6.X.2	Description

For XR or other interactive media services, the application layer’s network requirements could be quite dynamic. Typically, the size of data burst in XRM services could vary in a wide range. To ensure the occasionally big bursts can be transferred within PDB/PSDB, currently the QoS parametersneed to be set according to the potential maximum burst value. This overprovisioning leads to potential waste of network resource and lower user capacity.

In this solution, a fast adaptation mechanism is proposed to support the dynamic changes in traffic characteristics:

-	The AF provisions the QoS Requirement for a target media flow, and indicates that traffic burst size can change dynamically.

-	The PCF authorizes the service data flow in the PCC rule based on the AF input and/or local operator configuration.

-	Based on the PCC rule from the PCF, the SMF generates and provides the QoS profile with authorized QoS parameter to the NG-RAN,.

-	The SMF instructs the UPF to detect the dynamic change in the burst size) for the target service data flow and to further notify NG-RAN on the changed traffic burst size.

-	The UPF detects the dynamic change of the burst size for the target service data flow and sends the burst size of the data burst to NG-RAN via GTP-U header.

Editor’s Note: How UPF identifies the data burst size is FFS.

Editor’s Note: RAN2 and RAN3 feedback is required on that solution.

-	The NG-RAN can use the received burst size to assist radio resource management.

### 6.X.3	Procedures

Editor's Note: This clause describes high-level procedures and information flows for the solution.

### 6.X.4	Impacts on services, entities and interfaces

Editor's Note: This clause captures impacts on existing 3GPP nodes and functional elements.

# * * * * End of changes * * * *