3GPP TSG-WG SA2 Meeting #161 	S2-2403117

Feb. 26th – Mar. 2nd, 2024, Athens, Greece	(Revision of S2-2402452)

Source:	Intel

Title:	New Solution for KI#2: EAS Discovery taking account of EAS load in EASDF

Document for:	Approval

Agenda Item:	19.9

Work Item / Release:	FS_eEDGE_5GC_Ph3 / Rel-19

Abstract: This paper proposes a new solution for KI#2 on EAS discovery considering EAS load information.

# 1. Introduction

This contribution proposes a new solution for KI#2 on EAS discovery considering EAS load information.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-49.

# * * * * First change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | KI#1 | KI#2 | KI#3 |  |
| #X: EAS Discovery taking account of EAS load in EASDF |  | X |  |  |

# * * * * Second change * * * *

## 6.X	Solution #X: EAS Discovery taking account of EAS load in EASDF

### 6.X.1	Key Issue mapping

This solution addresses EAS Discovery considering EAS load aspects of KI#2.

### 6.X.2	Description

The EAS load information can be known by AF (Application Function), AF can update the EAS load information to EASDF or DNS Server in some interval or range threshold which can be configured in AF. Then during the EAS discovery procedure (in Figure 6.X.2-1), the EASDF can discover and select a suitable EAS by taking account of the EAS load information.

Figure 6.X.2-1: EAS Discovery using EASDF procedure (clause 6.3.2.2.2 in TS 23.548[5])

AF is supposed to store the latest EAS load information, ESADF can subscribe to AF for EAS load information update, then AF can notify the EASDF with the latest EAS load information. The subscription can be per FQDN(s), FQDN range, IP address(es), IP range. The AF can be configured to report in some interval or percentage change range of EAS load.

The EAS load information includes one or more of following:

- load percentage of general resource;

- load percentage of memory resource;

- load percentage of storage resource;

- load percentage of processor resource;

- load percentage of network resource.

### 6.X.3	Procedures

#### 6.X.3.1 EAS load subscription and notification procedure

Figure 6.X.3-1: EAS load subscription and notification procedure

Step 1: EASDF subscribes to AF for EAS load change notification per EAS identifier. The operator can configure the EASDF to serve a list of AFs and EASs, the EASDF subscribes to the selected AFs based on the configured list of AFs.

Step 2: AF is configured with notification condition/trigger which can be time interval or EAS load change range.

Step 3: AF notifies EASDF the latest EAS load per subscribed EAS identifier.

The EAS identifier can be EAS FQDN(s), FQDN range, IP address(es), IP range.

### 6.X.4	Impacts on services, entities and interfaces

Impact to EASDF:

- EASDF subscribes to AF for EAS load notification per EAS identifier;

- EASDF stores the EAS load information and selects a suitable EAS for the received EAS Discovery request from UE taking account of EAS load information.

Impact to AF:

- AF collects and stores EAS load information;

- AF notifies EASDF about latest EAS load with configured notification trigger.

# * * * * End of changes * * * *