SA WG2 Meeting #161  	S2-2403334

# 26 February-1 March 2024, Athens,Greece

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1054 | rev | 3 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of analytics accuracy information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2024-2-9 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When we comparing accuracy between ML models, it always refers to the ML models that serve same analytic ID. It is not helpful and meaningless to compare accuracy ML models that use in different analytic ID. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add half sentence to clarify the analytics accuracy information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some misunderstandings in analytics accuracy information, may cause mis-operation of providing analytic accuracy information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5C.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

# 5C	Analytics/ML Model Accuracy Monitoring Functional Description

## 5C.1	General

A NWDAF may have the accuracy checking capability for Analytics and/or ML Models. The NWDAF may provide the accuracy information to consumers when requested or use it for its internal processes.

Input data is collected from Data Producer NF(s) when there is a request for inference/prediction per analytics ID in NWDAF for a specific time period in future. Ground truth data are collected from those Data Producer NF corresponding to the requested analytic ID at the time to which the prediction refers.

The ground truth data is the actual measured data observed at the time which the prediction refers to.

NOTE 1:	The ground truth data can be impacted when Analytics Feedback Information shows that an action is triggered by the analytics output in the consumer.

Analytics/ML Model Accuracy Monitoring is to be achieved by comparing the predictions using the current trained ML model and its corresponding ground truth data i.e. the corresponding true observed events.

Analytics/ML Model Accuracy information is to represent general performance measurements for analytics and ML Model respectively, which are composed of the number of correct predictions out of all predictions and the corresponding number of samples.

NOTE 2:	How an MTLF/AnLF determines whether a prediction is correct is up to implementation.

The NWDAF (containing AnLF/MTLF) with accuracy checking capability decides to initiate analytics accuracy monitoring based on:

-	A request from an analytics accuracy consumer. The analytics accuracy consumer may be an NWDAF containing AnLF, NWDAF containing MTLF or an analytics consumer NF.

-	Analytics Feedback Information which may be provided by an Analytics Consumer NF.

The AnLF with analytics accuracy checking capability as defined in clause 6.2D is able to provide or notify the accuracy information of Analytics IDs to the analytics consumers of such service and when the analytics accuracy does not meet the analytics consumer's requirements, the analytics consumer may stop using analytics for a period of time or obtain new analytics. In addition, updated analytics for the provided Analytics IDs may be provided to analytics consumers as requested, if the updated analytics is able to be generated within the correction time period. The AnLF with analytics accuracy checking capability is as defined in clause 6.2D.1 is able to determine analytics accuracy information based on e.g.:

-	Comparing predictions and its corresponding ground truth data, which are collected corresponding to the requested analytic ID at the time which the prediction refers to.

NOTE 3:	The ground truth data and the corresponding prediction is to be defined per Analytics ID.

-	Comparing changes in internal configuration for the analytics ID generation (e.g. change of data collection parameters, change in data distribution from a Data Source).

-	Previous existent records of analytics accuracy information.

-	Accuracy feedback information provided by an NF consumer.

-	Determining analytics accuracy by comparing analytics accuracy using multiple ML models that serve the same Analytics ID.

The MTLF with ML Model accuracy checking capability as defined in clause 6.2E is able to determine ML Model degradation based on e.g.:

-	comparing/evaluating the data: including input data, analytics output and the ground truth data either collected from various data source NFs, DCCF, AnLF, ADRF or configured by OAM;

-	or AnLF providing notifications of the analytics accuracy information; or

-	AnLF providing analytics feedback information of the analytics generated by the ML model.

The NWDAF containing MTLF may reselect a new ML model or retrain the existing ML model and consequently notify the ML model accuracy degradation to the ML Model consumer(s). In addition, the NWDAF containing MTLF may consider the rating of untrusted AF(s) when used as data sources.

* * * End of Changes * * *