SA WG2 Meeting #161  	 S2-2403345

26 February-1 March 2024, Athens,Greece

Source:	ZTE

Title:	Solution about LMF-side model training and inference

Document for:	Approval

Agenda Item:	19.15

Work Item / Release:	FS_AIML_CN / Rel-19

Abstract: This contribution proposes a solution for KI#1 about model training of LMF-side model at NWDAF and performing inference at LMF for direct AI/ML positioning

# 1. Discussion

In this contribution, we propose a solution to train and share ML model that outputs the UE locations. This solution suggests the training of the direct positioning AI/ML model is done by NWDAF and the inference of the trained model is done by LMF. By doing so, some data collection can be avoided during the inference phase, because LMF may have some of the inference data. Therefore, the inference speed is faster than performing inference at NWDAF with lower latency for in-time UE positioning task.

# 2. Proposal

This paper proposes to add a solution for Key Issue #1 to TR23.700-84 as follows.

---------- Start of change ----------

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues and Use Cases

|  | Key Issues |  | Use cases (optional) |  |
| --- | --- | --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> | <use case #x> | <use case #y> |
| #1 | X |  |  |  |
| #2 |  |  |  |  |

---------- Next change (all new text) ----------

6.x	Solution #X: Training of the AI/ML positioning model

6.x.1	Functional Description

A request of  LMF-side model that outputs location of a specific UE is provided by the service consumer. After training of the ML model, the NWDAF provides the trained ML model to the service consumer(e.g. LMF) and the service consumer can perform inference afterwards.

Although the service consumer can obtain UE location from AMF/LCS, but the accuracy may not be enough. Therefore, an enhancement of the existing NWDAF is needed, to train the ML model with measurement data and provide a more accurate location estimate of an UE.

6.x.1.1	Input of model training

| Information | Source | Description |
| --- | --- | --- |
| Measurements | TBD |  |
| >Measurement data |  | The measurement data collected by LMF. |
| >Positioning method |  | The positioning method corresponding to the measurement data. |
| >Time stamp |  | Time stamp of measurement data |
| Ground truth data | TBD | The Ground truth data for the LMF-side model training |

Table 6.x.1.1-1: Data Collected for LMF-side model training.

The information may collect by the NWDAF for LMF-side model training is defined in Table 6.x.1.1-1.

Editor’s Note: 	It is FFS to determine what and how data to be collected for LMF-side model training. Coordination with RAN WG is needed.

Editor’s Note: 	The ground truth data for LMF-side model is ground truth UE location, and how to collect the ground truth UE location is FFS.

The trained LMF-side model shall be able to calculate UE location.

6.x.2	Procedures

The NWDAF can provide trained LMF-side model to consumer as follow:

NWDAF

NWDAF consumer

- Nnwdaf_ModelProvision_subscribe

2. Data collection

3. Model training

4.Nnwdaf_ModelProvision_notify

5. Inference

If the consumer request a trained model in step 1, the NWDAF provides the trained model to the consumer with the training input data information to help the consumer perform inference.

- NWDAF consumer(e.g. LMF) subscribe to NWDAF with ML model interoperability information to obtain LMF-side model.

- The NWDAF collects training data specified in the Table 6.x.1.1-1.

- Editor’s Note: 	It is FFS to determine the data collection procedure for model training.

- The NWDAF performs model training base on the collected training data.

- The NWDAF provides the trained model to the consumer with the training input data information to help the consumer perform inference.

- The consumer can perform inference afterwards.

NOTE: Whether and how to trigger inference is not in the scope of this solution.

Editor’s Note: 	Whether the LMF is a standalone NF or co-located with AnLF for model inference is FFS.

6.x.3	Impacts on existing services, entities and interfaces

To implement the proposed solution, the NWDAF should support the proposed model training with input data(listed in table 6.x.1.1-1) collection capability. The trained model shall have the ability to calculate UE location. Besides, the NWDAF shall be able to provide the trained model to consumer.

---------- End of change ----------