3GPP TSG-SA WG2#161	S2-2403469

Athens, GR, January 26 – March 1, 2024	(Revision of S2-2403409 and S2-2402527)

Source:	Sony, Deutche Telekom

Title:	New Solution: AIoT Identifier Control

Document for:	Approval

Agenda Item:	19.14

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This contribution proposed a solution for temporary identifier handling.

# 1.	Discussion

This contribution proposed a solution for temporary identifier handling.

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-13:

# >>>>BEGINNING OF CHANGES<<<<

## 6.X	Solution #X: AIoT Temporary Identifier Control

### 6.X.1	Description

This solution address KI#2. The basic principle in 5GS is that the permanent subscriber identifier shall never be sent in clear text over Uu interface and a UE temporary identifier shall only be sent over Uu interface in clear text once e.g. when the UE is paged. This solution propose that same principle shall apply also for Ambient IoT.

The solution is based on the following assumptions:

-	The AIoT device have higher complexity than a RFID tag that only reflects a preconfigured device ID when exited by RF power, but significantly lower complexity than a 3GPP CIoT device.

-	The AIoT device has a non-volatile storage capability.

-	The Temp ID generation algorithm is light weight but enough to avoid unauthorized AIoT device tracking.

### 6.X.2	Procedures

As the available power in an AIoT device is very limited, the message exchange between the device and the network must be minimized. The solution is based on the following principle:

- The initial temporary identifier (TempID) is known by both the CN NF and the AIoT device. After the AIoT device has been onboarded to the network the CN NF provision the AIoT device with the initial TempID and/or parameters to derive the initial Temp ID i.e.,. parameters for the Temp ID generation algorithm.

NOTE 1: Onboarding procedure will be studied under KI#2 and related conclusions needs to be considered when concluding the AIoT device Identifier control. It is assumed that during the onboarding procedure the CN NF can retrieve information from another NF or AF to onboard the AIoT device. The inital message from the UE during onboarding could e.g. include a URL or FQDN to establishing IP connection to a AF that holds additional onboarding information needed.

- Every time the TempID has been sent over the radio interface and a response from the AIoT device is sent, both CN NF and AIoT device locally generate a new TempID.

Editor’s Note: Details on the algorithm that locally generates a new TempID needs to be defined by SA3.

- In case the CN NF and the AIoT device TempID out of sync is detected, the CN NF and the AIoT re-synchronize the TempID.

Editor’s Note: How the out-of-sync detection and re-synchronization are performed is FFS, e.g. it may be a counter value not matching the expected value. Details depends on the Temp ID generation algortim.

- AIoT device consider and responds to the DT message if the AIoT device can match the TempID used in the DT message.

- AIoT device responds with DO-DTT message if the AIoT device can match the TempID used in the trigger message.

NOTE 2: The CN NF that manage the TempID will be defined together with the system architecture design to support the Ambient IoT in 5GC.

### 6.X.3	Impacts on services, entities and interfaces

Impacts on existing entities:

CN NF:

- Support the AIoT TempID handling including the generation of initial TempID, new TempID and re-synchronization between AIoT device and CN NF.

AIoT device:

- Recieving a initial TempID from the network.

- Generation of new TempID locally, when TempID used over Uu interface.

- re-synchronization between AIoT device and CN NF.

# >>>>END OF CHANGES<<<<