SA WG2 Meeting #161	S2-2403545

26 February - 1 March, 2024, Athens, Greece

Source:	vivo, Charter Communications

Title:	New Sol: Eliminate IPSec tunnel for user plane

Document for:	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This paper proposes a new Solution for PDU Session establishment with non-3GPP access for eliminating IPSec tunnel for UP.

1	Discussion

This paper proposes a new solution for KI#2.2 of the FS_MASSS SID (SP-231802).

2	Proposal

It is proposed to include the following changes in TR 23.700-54 V0.1.0.

* * * * Start of Changes * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of DualSteer Solutions to Key Issues

|  | Key Issues for DualSteer |  |  |  |
| --- | --- | --- | --- | --- |
| Solution# | <Key Issue #1.1> | <Key Issue #1.2> |  |  |
| #X |  |  |  |  |

Table 6.0-2: Mapping of ATSSS_Ph4 Solutions to Key Issues

|  | Key Issues for ATSSS_Ph4 |  |  |  |
| --- | --- | --- | --- | --- |
| Solution# | <Key Issue #2.1> | <Key Issue #2.2> |  |  |
| #Y |  | x |  |  |

* * * * Second Change (all new text) * * * *

### 6.2.Y	Solution #Y: Eliminate IPSec tunnel for UP

6.2.Y.1	Description

This solution addresses the Key Issue#2.2 "Simplified ATSSS architecture over non-3GPP access" that related to eliminate IPSec tunnel encapsulation.

6.2.Y.2	Procedures

Figure 6.2.Y.2-1: PDU Session Establishment for UP w/o IPSec tunnel over non-3GPP access

1.	The UE sends a PDU Session Establishment Request message to AMF as specified in step 1 of clause 4.3.2.2.1 of TS 23.502 [a]. This message shall be sent to N3IWF via the IPsec SA for NAS signalling (established as specified in clause 4.12.2 of TS 23.502 [a]) and the N3IWF shall transparently forward it to AMF in the 5GC..

2.	The AMF invokes Nsmf_PDUSession_CreateSMContext Request service operation towards an SMF.

3.	The SMF reserves UP resources and invokes Namf_Communication_N1N2MessageTransfer Request (N2 Info (no IPSec indication), N1 Info (PDU Session Establishment Accept)) service operation towards the AMF.

4.	The AMF sends N2 message (no IPSec indication, PDU Session Establishment Accept) to the N3IWF/TNGF.

5.	The N3IWF/TNGF stops initiating Child SA creation procedure for the PDU Session.

6.	The N3IWF/TNGF forwards the PDU Session Establishment Accept towards the UE over the IPSec SA for NAS signalling. The UE sends and receives data of the PDU Session without IPSec tunnel according to step 5. The N3IWF/TNGF relays the received data from UE without IPSec encapsulation over the N3 tunnel of the PDU Session.

Editor's note: Security over UP between UE and N3IWF/TNGF is FFS and needs to be evaluated by SA3 WG.

Editor's note: The data exchange over UP between UE and N3IWF/TNGF and how N3IWF/TNGF recognizes the data is from the UE are FFS.

Editor’s note: It is FFS whether and how the core network (e.g. AMF/SMF) knows that the UE and the N3IWF/TNGF support this feature.

6.1.X.3	Impacts on services, entities and interfaces

Editor's note:	This clause captures impacts on existing 3GPP services, entities and interfaces.

SMF:

-	Indicates N3IWF/TNGF to stop IPSec SA creation for PDU Session.

N3IWF/TNGF:

-	Support to stop IPSec SA creation procedure with the UE according to the indication received from SMF.

UE:

-	Support to not use IPSec tunnel for UP of the PDU Session over non-3GPP access.

* * * * End of Changes * * * *