3GPP TSG-WG SA2 Meeting #161	S2-2401927

# Athens, Greece, February 26 - March 1, 2024                (revision of S2-2400086)

Source: 	China Mobile

Title: 	New Solution: Network optimization for energy saving per S-NSSAI

Document for: 	Approval

Agenda Item: 	19.4

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: Propose a solution on KI#3: Network optimization for energy saving per S-NSSAI.

1	Discussion

This paper is to propose a new solution on KI#3.

This solution aims at satisfying energy saving requirement at network slice granularity. The EECF obtains parameters (i.e. target maximum number of UEs, target maximum number of PDU sessions) for energy saving per S-NSSAI from OAM, based on AF request for energy consumption threshold. The EECF indicates NSACF to update the parameters.

2	Proposal

It is proposed to include the below changes into TR 23.700-66 v0.3.0.

- FIRST CHANGE (all the text is new)

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | ... |
| Solution x: Network optimization for energy saving per S-NSSAI |  |  | X |  |
| … |  |  |  |  |

- NEXT CHANGE (all the text is new)

6.x	Solution #X: Network optimization for energy saving per S-NSSAI

6.x.1	Key Issue mapping

This solution is related to the KI#3: 5GS enhancements for network energy saving and efficiency.

6.x.2	Functional Description

This solution The architecture of this solution is described as Figure 6.x.2-1.

AMF

SMF

EECF

AF

PCF

DN

UPF

NEF

(R)AN

UE

Naf

Nnef

Npcf

Neecf

Nsmf

Namf

Figure 6.x.2-1: Architecture assumption

A new NF, Energy Efficiency Control Function (EECF) is introduced. The EECF includes at least the following functionality:

-	Collection of the energy related information that have impact on energy consumption/efficiency per S-NSSAI from OAM.

-	Generating and sending adjustment information to NSACF.

This solution performs energy saving in network slice granularity, by adjusting the following parameters:

-	maximum number of UEs: Reducing maximum number of UEs for a network slice limits the number of UEs registered in this network slice, therefore reduces the energy consumption.

-	maximum number of PDU Sessions: Reducing maximum number of PDU Sessions for a network slice limits the number of PDU Sessions access to this network slice, therefore reduces the energy consumption.

In this solution, the EECF sends the adjustment information to NSACF based on energy consumption threshold and energy saving authorization information provided by AF and energy information from OAM, indicating NSACF to adjust the maximum number of UEs and maximum number of PDU Sessions.

- x.3	Procedures

3. Energy information collection

OAM

1. Nnef_EnergySaving

NEF

AF

4. Nnsacf_NSAC_NumberUpdate

2. Neecf_EnergySaving

EECF

NSACF

Figure 6.x.3-1: Procedure for network optimization for energy saving per S-NSSAI

1.	AF creates a request for energy saving, including S-NSSAI and energy consumption threshold, and sends the request to NEF.

2.	The NEF invokes Neecf_EnergySaving to send the S-NSSAI and energy consumption threshold to EECF.

3.	The EECF collects energy information of the S-NSSAI from OAM, including Energy type information, Carbon emission information, Carbon efficiency information, Energy efficiency information, Energy consumption information. The details can be referred to Solution #1 described in clause 6.1.

4.	If there is only one NSACF of the S-NSSAI or there is a primary NSACF deployed in the PLMN, the following is operated: Based on the energy information and the energy consumption threshold, the EECF generates adjustment information to indicate NSACF to adjust the following parameters of the S-NSSAI to satisfy the energy consumption threshold: maximum number of UEs, maximum number of PDU sessions. The EECF sends the information to NSACF via Nnsacf_NSAC_NumberUpdate service.

6.x.4	Impacts on existing services, entities and interfaces

EECF: New NF. Functionalities are as description of clause 6.x.2.

NEF: New service operation Nnef_EnergySaving.

NSACF: Updating maximum number of UEs and maximum number of PDU sessions per S-NSSAI based on adjustment information from EECF. New service operation: Nnsacf_NSAC_NumberUpdate

- End of CHANGE