3GPP TSG-WG SA2 Meeting #161	S2-2401932

# Athens, Greece, February 26 - March 1, 2024

Source: 	China Mobile

Title: 	New Solution: Edge computing handling by I-SMF

Document for: 	Approval

Agenda Item: 	19.8

Work Item / Release:	FS_EDGE_Ph3 / Rel-19

Abstract of the contribution: Propose a solution on KI#1: Enhancements for EAS (re)discovery and UPF (re)selection with reducing impact on central 5GC NFs.

1	Discussion

This paper is to propose a new solution on KI#1.

In this solution, UL CL/BP controlled by I-SMF is used to offload edge computing related traffic to local DN. The central SMF is not involved with EDI management, DNS message handling rules configuration or update, and UL CL/BP or L-PSA insertion. Therefore, the impact on central SMF is reduced.

2	Proposal

It is proposed to include the below changes into TR 23.700-49 v0.1.0.

FIRST CHANGE

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | ... |
| Solution x: Edge computing handling by I-SMF | X |  |  |  |
| … |  |  |  |  |

NEXT CHANGE (all the text is new)

6.x	Solution #X: Edge computing handling by I-SMF

6.x.1	Key Issue mapping

This solution is related to the KI#1.

6.x.2	Functional Description

The architecture of this solution is described as Figure 6.x.2-1.

Local network

UE

AMF

SMF

I-SMF

PCF

AN

UPF (ULCL/BP)

NEF

PSA UPF

(C-PSA)

UPF(L-PSA)

L-DN

DN

EASDF

UDM

Namf

Npcf

Nudm

Nnef

Nsmf

Nsmf

Neasdf

Figure 6.x.2-1: Architecture assumption

In this solution, I-SMF is used to handle the edge computing related traffic to be offloaded into local DN, EDI management, DNS message handling rules configuration or update, and UL CL/BP or L-PSA insertion. Therefore, the impact on central SMF is reduced.

In this solution, the I-SMF, ULCL/BP UPF and Local PSA UPF are introduced with reusing mechanism described in 5.34 of TS 23.501. The difference from the ETSUN mechanism is that, SMF also sends the address of I-SMF to PCF. When traffic offload policy or edge computing related policy is updated, the PCF sends the policy update to I-SMF, with SMF forwarding transparently, which also reduces the edge computing handling impact to central SMF.

6.x.3	Procedures

UL CL

UE

(R)AN

AMF

SMF

UPF

I-SMF

PCF

UDM

2. Npcf_SMPolicyControl_Update

3. Npcf_PolicyAuthorization_Update

4. Nsmf_PDUSession_Update

1. Steps 1-10 of Addition of PDU Session Anchor and Branching Point or UL CL controlled by I-SMF procedure, clause 4.23.9.1 of TS 23.502

Figure 6.x.3-1: Procedure of edge computing handling by I-SMF

1.	Steps 1-10 of Addition of PDU Session Anchor and Branching Point or UL CL controlled by I-SMF procedure is performed as described in clause 4.23.9.1 of TS 23.502 [3].

2.	SMF sends the endpoint address of I-SMF to PCF via Npcf_SMPolicyControl_Update service.

The following steps 3-4 will be performed when traffic offload policy or edge computing related policy is to be updated.

3.	PCF sends policy container to SMF by invoking Npcf_PolicyAuthorization_Update service. The traffic offload policy or edge computing related policy is encapsulated in the policy container.

4.	The SMF forwards the policy container transparently to I-SMF via Nsmf_PDUSession_Update service. The I-SMF performs EAS (re)discovery and UPF (re)selection based on the traffic offload policy or edge computing.

6.x.4	Impacts on existing services, entities and interfaces

I-SMF:

-	Receiving the policy from PCF.

-	Handling the edge computing related traffic to be offloaded into local DN, EDI management, DNS message handling rules configuration or update, and UL CL/BP or L-PSA insertion. and control the edge computing handling for ULCL/BP,

SMF: Forwarding traffic offload policy or edge computing related policy transparently to I-SMF.

PCF: Providing traffic offload policy or edge computing related policy in policy container and sending to I-SMF.

End of CHANGE