3GPP TSG-SA WG2#161	S2-2401937

Athens, Greece, 26 February-1 March 2024	(was S2-240xxxx)

Source:	Qualcomm Incorporated

Title:	Ambient IoT system architecture

Document for:	Approval

Agenda Item:	19.14

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes a system architecture for Ambient IoT.

# 1.	Discussion

This solution proposes a lightweight Ambient IoT system.

# 2.	Text proposal

It is proposed to agree the following changes to TR 23.700-13.

# >>>>BEGINNING OF CHANGES<<<<

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.369: "Service requirements for Ambient power-enabled IoT".

[3]	3GPP RP-234058: "New SID: Study on solutions for Ambient IoT (Internet of Things) in NR".

Editor's note:	The RAN SID RP-234058 reference is to be updated to RAN TR when available, and the meaning of no mobility is to be clarified by RAN.

[4]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[5]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[6]	3GPP TS 23.503: "Policies and Charging control framework for the 5G System; Stage 2".

[7]	3GPP TR 38.848: "Technical Specification Group Radio Access Network; Study on Ambient IoT (Internet of Things) in RAN".

[8]	3GPP TR 38.xyz: "Study on solutions for Ambient IoT (Internet of Things) in NR".

Editor's note:	The number of TR 38.xyz will be updated when it is available.

[X1]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

# >>>> NEXT CHANGES <<<<

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | Key Issue #1 | Key Issue #2 | Key Issue #3 |
| #1 |  |  |  |
| #2 |  |  |  |
| #X | X | X | X |

# >>>>NEXT CHANGES (all new text)<<<<

## 6.X	Solution #X: Lightweight Ambient IoT system

### 6.X.1	Description

#### 6.X.1.1	Introduction

This solution proposes a lightweight Ambient IoT system.

#### 6.X.1.2	Definitions

-	Command: Refers to an instruction sent by an AF to an AIoT Device. The following instructions may be supported:

-	Read: Reading data from an AIoT Device;

-	Write: Writing data to an AIoT Device;

-	Disable: Disable an AIoT Device temporarily or permanently.

-	Command Response: Refers to the message sent by an AIoT Device in response to a Command. This may include an acknowledgement and optionally data (e.g., in case of the Read operation).

-	Enrichment Data: Additional information that a Reader may include when providing Inventory information or a Command Response to the AIoT Controller. Enrichment Data may include information about the signal strength for each detected Device ID and the location of the Reader (if known).

-	Filter Criteria: Criteria to limit an Inventory or Command to AIoT Devices that match certain criteria.

Editor's note: The details of Filter Criteria are FFS.

-	Inventory: Refers to determining the identity of all or a subset of AIoT Devices in range of a reader.

#### 6.X.1.3	Assumptions

This solution makes the following assumptions:

-	Commands and Command Responses are end-to-end protected between AF and AIoT Device.

NOTE X1:	Details of the end-to-end protection of Commands and Command Results are assumed to be addressed by SA3.

-	Commands and Command Responses are transparent to the AIoT Controller and the Reader, i.e., the AIoT Controller and Reader are not aware of the contents of Commands and Command Responses.

-	AIoT Devices are assumed to be pre-provisioned with a Device ID and the security material for the end-to-end protection of Commands and Command Results.

NOTE X2:	Whether also dynamic (re-)provisioning of Ambient IoT Devices can be supported is up to SA3.

-	The radio configuration (frequency bands, etc.) of the Reader is assumed to be configured through an OAM system, which is beyond the scope of SA2.

#### 6.X.1.4	Reference architecture

This solution proposes the reference architecture depicted in Figure 6.X.1.4-1.

Figure 6.X.1.4-1: Ambient IoT system architecture

The Ambient IoT system according to Figure 6.X.1.4-1 can support different deployment options. For example, the Reader may be co-located with a 3GPP UE so that the communication between Reader and AIoT Controller uses a PDU Session. Alternatively, Readers may be deployed independent of 3GPP UEs, i.e., as stand-alone base stations.

#### 6.X.1.5	Network function description

##### 6.X.1.5.1	Reader

The Reader supports the following functionality:

-	Supports the A-Uu air interface towards Ambient IoT Devices

-	Registers with an AIoT Controller

-	Supports the following functionality based on requests from an AIoT Controller

-	Perform one-time or periodic Inventory, deliver Inventory result to AIoT Controller

-	Delivers Commands from an AIoT controller to an AIoT Device

-	Delivers Command Responses received from an AIoT Device to an AIoT Controller

##### 6.X.1.5.2	AIoT Controller

The AIoT Controller supports the following functionality:

-	Register Readers

-	Authenticate and authorize AFs

-	Based on requests from an AF:

-	 Verify whether an AF is entitled to issue a specific Inventory Request

-	Select Readers to fulfil Inventory or Command request by AFs

-	Forward Inventory request to Readers and deliver Inventory result to AF

-	Forward Command to Readers and Command Responses to AF

-	Optionally collect usage data per AF, e.g., for charging purposes

-	Store last known Reader information for AIoT Devices

##### 6.X.1.5.3	AF

The AF is assumed to support the following functionality:

-	Authenticate towards the AIoT Controller

-	Send Inventory and Command Requests

-	Receive Inventory Responses and Command Responses

#### 6.X.1.6	Protocol Stacks

##### 6.X.1.6.1	Protocol Stack between Reader and AIoT Controller

Legend:

-	Reader Application Protocol (R-AP): Application Layer Protocol between the Reader and the AIoT Controller.

-	Service-based Interface (SBI) Protocol Stack: The protocol for service-based interfaces as defined in TS 29.500 [X1].

Figure 6.X.1.6.1-1: Control Plane between Reader and AIoT Controller

NOTE:	The R-AP protocol is assumed to be defined by RAN3 in coordination with SA2.

##### 6.X.1.6.2	Protocol Stack between AIoT Device and Application Function

Legend:

-	Reader Application Protocol (R-AP): Application Layer Protocol between the Reader and the AIoT Controller.

-	Service-based Interface (SBI) Protocol Stack: The protocol for service-based interfaces as defined in TS 29.500 [X1].

-	AIoT API: The API between AIoT Controller and Application Function to support Inventory and Command Procedures.

-	Command Protocol: Application Layer Protocol between AIoT Device and Application Function to support Commands and Command Responses.

NOTE 1:	The AIoT API is assumed to be defined by SA2 (Stage 2 aspects) and CT3 (Stage 3 aspects).

NOTE 2:	The Command Protocol is assumed to be defined by SA2 (Stage 2 aspects) and CT1 (Stage 3 aspects).

### 6.X.2	Procedures

#### 6.X.2.1	Inventory procedure

Figure 6.X.2.1: Inventory procedure

1.	The AF sends an Inventory Request to the AIoT controller. The AF may optionally include the following information:

-	Filter Criteria to limit the inventory to AIoT Devices matching those criteria;

-	a list of reader IDs to limit the inventory to specific Readers;

-	a periodicity value to request the inventory to be performed periodically.

2.	The AIoT controller verifies whether the AF is entitled to make the received Inventory Request, e.g., the AIoT controller verifies whether the AF is allowed to issue an Inventory Request with the specified Filter Criteria or without any Filter Criteria, without providing reader IDs, etc.

3.	The AIoT controller sends the Inventory Request to the Readers identified by the Reader IDs or to all readers, includes the Filter Criteria and periodicity information, if provided by the AF.

4.	Each Reader that received the Inventory Request from the AIoT Controller performs the Inventory procedure according to the Filter Criteria, if provided. The Readers either perform a one-time inventory or perform the inventory periodically according to the received periodicity.

5.	One or multiple AIoT Devices respond to the Reader and provide their Device ID.

Editor's note: The details of how the Filter Criteria are applied during the Inventory procedure are FFS.

6.	The Readers collects the received Device IDs and provides the Device IDs to the AIoT Controller. The Readers may optionally include Enrichment Data.

7.	For each reported Device ID, the AIoT Controller stores the reader ID that reported the Device ID together with a timestamp. The timestamp enables the AIoT controller to purge outdated last known Reader information; the details of this are up to AIoT Controller implementation.

8.	The AIoT Controller provides the Device IDs and optionally the Enrichment Data to the AF.

#### 6.X.2.2	Command procedure

Figure 6.X.2.2: Command procedure

1.	The AF issues a Send Command request, which includes the Command to be sent and either:

-	a list of Device IDs that the Command is destined to, or;

-	Filter Criteria that identify the AIoT Devices that are supposed to act upon the Command.

In addition, the AF may include a list of reader IDs to use for sending the Command.

2.	The AIoT Controller determines the list of Readers to use for sending the Command taking the Reader IDs (if provided by the AF) and any stored information about the last known Reader for specific Device IDs into account. The details of determining candidate Readers are up to AIoT Controller implementation.

3.	The AIoT Controller provides the Command and the Device IDs or Filter Criteria (whichever has been provided by the AF) to the Readers.

4.	Each Reader that receives the Send Command request from the AIoT Controller, sends the Command to the AIoT Devices identified by the Device IDs or to the AIoT Devices identified by the Filter Criteria (whichever has been provided by the AIoT Controller).

Editor's note: The details of how the Filter Criteria are applied during the Command procedure are FFS.

5.	The AIoT Device(s) respond to the Reader and provide the Command Response.

6.	The Readers send the received Command Response(s) to the AIoT Controller. The Readers may optionally include Enrichment Data.

7.	The AIoT Controller sends the received Command Response(s) and optionally Enrichment Data to the AF.

NOTE:	Support of sending the same Command to multiple AIoT Devices requires support of group security for the Command (e.g., group keys for protecting the Command), which depends on SA3.

### 6.X.3	Impacts on services, entities and interfaces

New network entities and interfaces are proposed.

# >>>>END OF CHANGES<<<<