SA WG2 Meeting #161	S2-2401941

February 26th – March 1st, 2024, Athens, Greece

Source:	OPPO, Xidian University

Title:	FS_UIA_ARC KI#2 New solution for user id authentication

Document for:	Approval

Agenda Item: 	19.8

Work Item / Release:	FS_UIA_ARC /Rel-19

Abstract of the contribution: This paper proposes a new solution for user id authentication for the FS_UIA_ARC TR 23.700-32.

1	Discussion

This paper proposes user id authentication addressing KI#2.

2	Proposal

It is proposed to include the following changes in TR 23.700-32 V0.1.0.

* * * * Start of Changes (All text new) * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  |
| --- | --- | --- |
|  | <Key Issue #1> | <Key Issue #2> |
| #1 |  | x |
| #2 |  |  |

6.X	Solution #X: solution for user id authentication

6.X.1	Key Issue mapping

Editor's Note:	This clause lists the key issue(s) addressed by this solution.

6.X.2	Description

Editor's Note: This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). Sub-clause(s) may be added to capture details.

The solution has the following assumptions:

-  One SUPI is corresponding to one or more User IDs. The value of User IDs has been negotiated and preconfigured in UE, Core network, and AAA-Server.

-  It is assumed each node performs the following functions for User-ID based authentication

UE: Peer

AMF: Authenticator

AAA-Server: EAP server

-  Reusing EAP based authentication (e.g. MD5 challenge, Cogent Systems Biometrics Authentication EAP) to authenticate User-Id by 3rd party’s AAA-Server. The User-id and the authentication result can be aware by 5GC

6.X.3	Procedures

Editor's Note: This clause describes high-level procedures and information flows for the solution.

Figure-6.x.3-1 User ID based authentication after initial registration

(Steps/Parameters for potential impact is in Italic)

- User-A switch on a UE and the UE performs initial registration

- UE send initial registration including SUPI (Concealed as SUCI) and capability of User-Id based authentication

- AUSF triggers the Primary authentication to authenticate the SUPI reported by UE is correct. Specific procedure has been defined in TS 33.501

  - During the registration, AMF retrieves the SUPI’s subscription which indicates whether an User-ID based authentication is required.

NOTE 1: the other omitted messages for 5G registration follows the existing procedure.

5.  Registration accept is sent to UE with AMF allocated 5G-GUTI.

6.  AMF determines to perform User-ID based authentication by sending DL NAS message including EAP-Request (e.g. type=1, identity)

7.  UE sends UL NAS message including EAP-Response (e.g. type=1, identity=user id)

NOTE 2: The step 6 and 7 can be combined into one message

8.  AMF sends AAA Protocol message including GPSI and EAP-Response: identity=user id

9.  More interactions between EAP server and UE may be needed for authentication. E.g., triggers MD5-Challenge to authenticate the user id

10. EAP Server responds authentication result of the user id (success/failure)

11. AMF record the authentication result and send the result to UE

12. AMF sends the authenticated User ID to UDM for storing

6.X.4	Impacts on services, entities and interfaces

Editor's Note: This clause captures impacts on existing 3GPP nodes and functional elements.

UE

Send capability of User-Id authentication in registration request

Support EAP peer and send EAP message via NAS

AMF

Support EAP authenticator

Support determination of performing User-id authentication, sending EAP message via NAS

UDM

SUPI’s Subscription indicates whether User-Id authentication is required

Store the authenticated User ID

* * * * End of Changes * * * *