3GPP TSG-SA2 Meeting #161	S2-2401961

Athens, Greece, Feb. 26- Mar. 1, 2024

Source:	Lenovo

Title:	New solution for KI#1, Establishing backhaul link using a PDU session for N2 interface between MWAB-gNB and AMF

Document for:	Approval

Agenda Item:	19.6

Work Item / Release:	FS_VMR_Ph2 / Rel-19

Abstract of the contribution: establishing backhaul link using a PDU session for N2 interface between MWAB-gNB and AMF

1	Discussion

During SA2#160AdHoc meeting, the Non-Roaming MWAB architecture for 5GS is agreed. Besides, key issue#1 addresses architectural enhancements required to support the MWAB connects to the 5GC with the use of wireless backhauling (for the N2/N3 interfaces) via IP connectivity provided by a PDU session. This will include:

-	Whether and how the MWAB provides service for UEs from the HPLMN and UEs from other PLMN when the MWAB roaming nationally/internationally to another PLMN.

- 	How to provide the backhaul link using a PDU sessions for a MWAB, to support the N2/N3 interfaces and connectivity to an OAM server

-	How to discover and determine the AMF for the MWAB-gNB to connect to.

NOTE 1: The AMF discover and determination needs to be coordinated with RAN WGs.

In this paper, how to provide the backhaul link using a PDU session for a MWAB to support the N2 interface, together with the AMF discovery issue are considered.

2	Proposal

* * * * Start of Change * * * *

## 6.x	Solution #X: Establishing backhaul link using a PDU session for N2 interface between MWAB-gNB and AMF

### 6.x.1	Key Issue mapping

This solution is for Key Issue #1, which addresses the establishment of backhaul link using a PDU sessions to support the N2 interface for a MWAB.

### 6.x.2	Description

Based on the Non-Roaming MWAB architecture for 5GS in Figure 4.1-1, BH PDU session is established between MWAB-UE and UPF for MWAB-UE (i.e., MWAB-UE-UPF) within BH-5GC to transfer the N2 message, where N2 interface is established between MWAB-gNB and AMF within 5GC serving UE. This solution tries to solve the following issues when establishing the backhual link for N2 interface between MWAB-gNB and AMF using a PDU session between MWAB-UE and MWAB-UE-UPF,

- Issue#1: How to discover and determine the AMF for the MWAB-gNB to connect to?

- Issue#2: How MWAB-gNB obtains gNB IP address and AMF IP address before it triggers NG interface setup?

- Issue#3: How AMF obtains the mapping of MWAB-gNB and MWAB-UE?

- Issue#4: How MWAB-UE-UPF performs IP packet filtering for N2 message and forward the N2 message over the backhual PDU session to/from AMF?

The main idea of this solution is as follows,

- For Issue#1, upon receiving the MWAB-Node indication in MSG5 from MWAB-UE, BH gNB selects AMF which supports special handling for MWAB node.

- For Issue#2, MWAB-gNB obtains gNB IP address and AMF IP address from the selected AMF or OAM.

- For Issue#3, Upon receiving the MWAB-gNB information from MWAB-UE, AMF obtains the mapping of MWAB-gNB and MWAB-UE.

- For Issue#4, SMF sets AMF IP address and gNB IP address as source/destination IP address in the packet filter set. Based on the packet filter set, MWAB-UE-UPF is able to forward the N2 message to/from AMF.

### 6.x.3   Procedures

#### 6.x.3.1   Backhaul link establishment using a PDU session to support the N2 interface between MWAB-gNB and AMF

Figure 6.x.3.1-1 illustrates the procedure of establishing backhual link using a PDU session between MWAB-UE and MWAB-UE-UPF to support the N2 interface between MWAB-gNB and AMF. In the following, step 0-4 belong to Registration procedure and step 5-11 belong to PDU session establishment procedure.

Figure 6.x.3.1-1: Backhaul link Establishment using a PDU session to support the N2 interface between MWAB-gNB and AMF

Step 0: MWAB-UE includes MWAB-Node indication in MSG5 towards BH gNB.

Step 1: Upon receiving the MWAB-Node indication in MSG5, BH gNB selects AMF which supports special handling for MWAB node.

Step 2: BH gNB sends N2 message (e.g., Initial UE Message) towards the selected AMF, which includes MWAB-Node indication.

Step 3: Upon receiving the MWAB-Node indication in the N2 Message, AMF may provide gNB IP address and AMF IP address to MWAB-UE via BH gNB. E.g., AMF may provide gNB IP address and AMF IP address in Registration Accept message for MWAB-UE.

NOTE: Similar as IAB, MWAB-gNB may obtain both gNB IP address and AMF IP address from OAM after MWAB-UE established PDU session towards the OAM.

Step 4: MWAB-UE forwards the gNB IP address and AMF IP address to MWAB-gNB.

Step 5: MWAB-UE sends PDU Session Establishment Request message together with MWAB-gNB information to AMF via BH gNB, where PDU Session Establishment Request message includes gNB IP address.

Step 6: Upon receiving MWAB-gNB information from MWAB-UE, AMF preserves the first mapping of MWAB-gNB and MWAB-UE.

Step 7: AMF selects SMF which supports special handling for MWAB node. AMF sends Nsmf_PDUSession_CreateSMContext Request towards SMF, which inlcudes SUPI and PDU Session Establishment Request message from MWAB-UE.

Step 8: Upon receiving the PDU Session Establishment Request message from MWAB-UE, SMF obtains gNB IP address. SMF sets AMF IP address and gNB IP address as source/destination IP address in the packet filter set. It is assumed that SMF is pre-configured with QoS parameters for tranfering N2 message via BH PDU session.

Step 9: SMF triggers N4 Session Establishment Procedure towars MWAB-UE-UPF. MWAB-UE-UPF obtains the packet filter set which contains AMF IP address and gNB IP address.

Step 10: SMF sends N2 PDU Session Request message towards NG-RAN via AMF, which includes NAS message (i.e., PDU Session Establishment Accept) for MWAB-UE.

Step 11: BH gNB triggers AN-specific resource setup towards MWAB-UE, which contains PDU Session Establishment Accept message from SMF.

#### 6.x.3.2   NG Interface setup via BH PDU session

Figure 6.x.3.2-1 illustrates the procedure of NG Interface setup between MWAB-gNB and AMF via the BH PDU session between MWAB-UE and MWAB-UE-UPF.

Figure 6.x.3.2-1: NG interface setup via BH PDU session between MWAB-UE and MWAB-UE-UPF

Step 1: MWAB-gNB sends NG Setup Request message to MWAB-UE.

Step 2: MWAB-UE maps N2 message (i.e., NG Setup Request) to QFI and then to DRB.

Step 3: MWAB-UE sends NG Setup Request message via the DRB to BH gNB.

Step 4: BH gNB forwards the NG Setup Request message to MWAB-UE-UPF via the corresponding BH PDU session.

Step 5: Upon receiving NG Setup Request message from the BH gNB, MWAB-UE-UPF forwards it to AMF based on the destination IP address contained in the IP header.

Step 6: Upon receiving NG Setup Request message from MWAB-UE-UPF, AMF preserves the second mapping of MWAB-gNB and MWAB-UE-UPF.

Step 7: AMF generates NG Setup Response message and forwards it to MWAB-UE-UPF based on the second mapping.

Step 8: MWAB-UE-UPF performs IP filtering based on the packet filter set, maps the NG Setup Response message to the BH PDU session between MWAB-UE and MWAB-UE-UPF for N2 interface.

Step 9: MWAB-UE-UPF forwards the NG Setup Response message to BH gNB through the BH PDU session.

Step 10: BH gNB forwards the NG Setup Response message to MWAB-UE.

Step 11: MWAB-UE checks the destination IP address of the message.

Step 12: MWAB-UE forwards the NG Setup Response message to MWAB-gNB.

NOTE: the interaction between MWAB-UE and MWAB-gNB is out of 3GPP scope.

### 6.x.4	Impacts on services, entities and interfaces

The proposed solution is based on Rel-18 procedures but some enhancements may be needed to make it possible to support establishing backhaul link using a PDU session for N2 interface between MWAB-gNB and AMF, such as:

# MWAB：

-	MWAB-UE includes MWAB-Node indication in MSG5 towards BH gNB

-	MWAB-UE sends PDU Session Establishment Request message and MWAB-gNB information to AMF

# BH-gNB：

-	select AMF which supports special handling for MWAB node

-  include MWAB-Node indication in the Initial UE Message

# AMF：

-	select SMF which supports special handling for MWAB node

-	preserve the first mapping of MWAB-gNB and MWAB-UE

-	preserve the second mapping of MWAB-gNB and MWAB-UE-UPF

# SMF：

-	set AMF IP address and gNB IP address as source/destination IP address in the packet filter set

* * * * End of Change * * * *