3GPP TSG-WG SA2 Meeting #161	S2-2402179

Athens, Greece, 26th February – 1st March 2024	(revision of S2-240xxxx)

Source:	Samsung

Title:	New Sol for KI#1: Local management of EC related information

Document for:	Approval

Agenda Item:	19.9

Work Item / Release:	FS_eEDGE_5GC_ph3 / Rel-19

Abstract: This pCR proposes a new solution for KI#1

1. Introduction/Discussion

This pCR propose a solution KI#1 especially to address the following aspects:

-	Whether and which edge computing related information (e.g. UPF and EAS deployment information, DNAIs) can be locally managed to support EAS (re)discovery and UPF (re)selection.

-	How to manage the above identified edge computing related information, e.g. locally, centrally or in both places.

This pCR proposes procedures to enable the local SMF to locally manage the EAS Deployment Information and configure the EASDF/UPF, accordingly.

2. Text Proposal

It is proposed to introduce the following changes in TR 23.700-49 v0.1.0.

# * * * * First change * * * *All new text

6.X	Solution #X: Local management of EAS Deployment Information

### 6.X.1	Key Issue mapping

Key issue #1

### 6.X.2	Description

This solution is for Key Issue #1, which addresses how to locally manage the edge computing related information.

This solution assumes the deployment scenario where SMF is deployed in a central data center while NEF and a NF (in this solution, the NF refers to local SMF) locally manages edge computing related information is located in a local data center nearby an edge hosting environment.

This solution proposes to handle EAS deployment information among NFs located in the above mentioned local data center as much as possible.

### 6.X.3	Procedures

6.x.3.1	Provisioning EAS Deployment Information to local SMF without involvement of SMF

Figure 6.x.3.1-1: Provisioning EAS Deployment Information to local SMF without involvement of SMF

Precondition: Local SMF and SMF are set up to decide to support local offloading control.

1-2.	Local SMF subscribes EAS Deployment Information Change Notification from the NEF by providing 				information (e.g., DNN and/or S-NSSAI and/or application identifier) received from the SMF.

3.	The AF invokes the Nnef_EASDeployment_Create/Update/Delete service operation.

4.	The NEF checks whether the AF is authorized to perform the request, and authorised to provision the EAS 			Deployment Information based on the operator policies. If the NEF receives the subscription request for the EAS 	Deployment Information Change Notification, the NEF decides not to store the information in the UDR based on the 	locally configured operator policy

5.	The NEF provides response to the AF.

6.	The NEF sends the notification message including EAS Deployment Information to the local SMF.

7.	The local SMF invokes Neasdf_BaselineDNSPattern or Neasdf _DNSContext based on the information received 	from the information obtained from the NEF and the SMF. The local SMF obtains EASDF information from the 	SMF in advance or use the locally configured EASDF information.

8.	The local SMF configures the local PSA UPF to forward DNS message properly to EASDF and between EASDF 	and DN.

6.x.3.2	Provisioning EAS Deployment Information to local SMF with involvement of SMF

Figure 6.x.3.2-1: Provisioning EAS Deployment Information to local SMF with involvement of SMF

Precondition: Local SMF and SMF are set up to decide to support local offloading control.

1-2.	The SMF subscribes EAS Deployment Information Change Notification from the NEF by providing 				information (e.g., DNN and/or S-NSSAI and/or application identifier).

3.	The AF invokes the Nnef_EASDeployment_Create/Update/Delete service operation.

4.	The NEF checks whether the AF is authorized to perform the request, and authorised to provision the EAS 			Deployment Information based on the operator policies.

5.	The NEF invokes the Nudr_DM service operation to store/update/delete EAS Deployment Information.

6.	The NEF provides response to the AF.

7.	The NEF sends the notification message including EAS Deployment Information to the SMF.

8.	The SMF provides the notified EAS Deployment Information to the local SMF if local offloading control is decided. The SMF may provide the local SMF with Indication to modify the subscription related to EAS Deployment Information Notification, and the information required to modify the existing subscription information for the 	notification of the EAS 	Deployment Information (e.g., subscription correlation ID, corresponding NEF 	information).

9.	Option A: The SMF modifies the existing the existing subscription information for the notification of the EAS Deployment Information by replacing the notification target address with the local SMF address.

Option B: The local SMF modifies the existing the existing subscription information for the notification of the EAS Deployment Information by utilizing the information received from the SMF.

10.	The local SMF invokes Neasdf_BaselineDNSPattern or Neasdf _DNSContext based on the information received from the information obtained from the NEF and the SMF. The local SMF obtains EASDF information from the SMF in advance or use the locally configured EASDF information.

11.	The local SMF configures the local PSA UPF to forward DNS message properly to EASDF and between EASDF and DN.

### 6.X.4	Impacts on services, entities and interfaces

SMF

-	delegates EASDF and local UPF configuration related to DNS handling to the local SMF

-	modifies the existing subscription to EDI notification to delegate EDI management to the local SMF

Local SMF newly introduced

-	handles EDI locally, and	configures EASDF and local UPF based on the locally managed EDI

NEF

-	decides whether to store the EDI in the UDR or locally manage and directly provide the EDI to the local SMF

# * * * * End of changes * * * *