SA WG2 Meeting #161	S2-2402214

Athens, Greece, 26 Feburary 26 – March 01, 2024

# Title:	KI#2, New Sol: Enhancement for BDT with Energy related analytics

# Source:	KDDI, TOYOTA MOTOR CORPORATION

# Document for:	Approval

Agenda Item:	19.4 (FS_EnergySys)

Abstract of the contribution: It is proposed a new Solution for KI#2.

1 Discussion

According to the FS_EnergySys (SP-231391), the following Key Issue #2 need to be studied:

The following aspects will be studied for this key issue:

-	Whether and how to enhance the existing subscription and policy control framework to support energy related information as service criteria, including:

-	Whether and what new energy related UE subscription information are to be defined, and whether and how to use the energy related UE subscription information.

-	Whether and what new energy related policies are to be defined, and how to perform energy related policy control, e.g. to determine, provision and enforce energy related policies.

-	At what granularity (e.g., network slice, UE, NF, PDU Session, QoS flow, application ID, etc.) the energy related policy control can be performed.

-	What network energy related information is required for subscription and policy control and how it is obtained.

-	Whether and how the above enhancements on subscription and policy control will impact charging.

The solution for NWDAF-based Energy Analytics (Solution #12) was implemented in TR 23.700-66. This contribution proposes how to enhance the Negotiation for the future BDT and the BDT warning notification considering the Energy related analytics.

2. Proposal

It is proposed to add the following new solution to TR 23.700-66 " Study on Energy Efficiency and Energy Saving".

# *** 1st Changes ***

6.0	Mapping of Solutions to Key Issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | X |
| X |  | X |  |  |
| … |  |  |  |  |

# *** 2nd Change  (all new text)***

6.x	Solution #X: Enhancement for BDT with Energy related analytics

6.x.1	Key Issue mapping

This solution maps to KI#2.

6.x.2		Functional Description

This solution is proposed to address Key Issue #2: 5GS enhancements for network energy saving and efficiency.

Energy related analytics can be generated by a NWDAF and provided to a NF consumer. Then, the NF consumer can take any actions based on the analytics.

In case of negotiation for future BDT (background data transfer), a H-PCF may interact with the NWDAF and request the Network Performance analytics information for the Desired time window and the Network Area Information as defined in TS 23.502 [3]. For BDT warning notification, the H-PCF may subscribe to the analytics on the Network Performance analytics from NWDAF for the area of interest and time window of a background data transfer policy, including a Reporting Threshold in the Analytics Reporting information as defined in TS 23.502 [3]. However, the H-PCF does not consider Energy related information for determination of BDT policies.

It mainly proposes how to enhance the Negotiation for the future BDT and the BDT warning notification considering Energy related analytics (e.g. EE (Energy efficiency), EC (Energy Consumption) and Renewable energy ratio analytics).

6.x.3	Procedure

6.x.3.1	Procedure for Negotiation for future BDT with Energy related analytics

Figure 6.x.3-1: Procedure for Negotiation for future BDT with Energy related analytics

The procedure described in this clause is based on the negotiation for future BDT as defined in clause 4.16.7.2 of TS 23.502[3].

The AF invokes the Nnef_BDTPNegotiation_Create/Update Request. The request may contain Area of interest, Desired time window and one of following Energy related requirements:

‐	Energy related KPIs (e.g. Maximum allowed EE, EC or Renewable energy ratio for NG-RANs and/or UPFs in the Area of interest).

‐	Energy analytics activation flag.

The NEF invokes the Npcf_BDTPolicyControl_Create/Update Request. Examples of actions for the H-PCF are:

-	If the Npcf_BDTPolicyControl_Create/Update Request contains the Energy related KPIs, the H-PCF interacts with the NWDAF and sends a request or subscription for the Energy related analytics (Area of interest, Desired time window, the Energy related KPIs requested by AF).

-	If the Npcf_BDTPolicyControl_Create/Update Request contains the Energy analytics activation flag, the H-PCF interacts with the NWDAF and sends a request or subscription for the Energy related analytics (Area of interest, Desired time window, Energy related KPIs (e.g. Maximum allowed EE, EC or Renewable energy ratio for NG-RANs and/or UPFs in the Area of interest)). The Energy related KPIs for the request or subscription are determined based on operator configuration.

The NWDAF sends a response or notify with Energy related analytics to the H-PCF, if the requested Energy related KPIs are met.

NOTE:	How the NWDAF generates the analytics for Energy related conditions is not addressed in the proposed solution but uses other solutions (e.g. Solution #12).

The H-PCF determines one or more BDT policies, based on information provided by the AF, the NWDAF and other available information. Then it sends the acknowledge message to the NEF with the acceptable BDT policies and Background Data Transfer Reference ID as defined in clause 4.16.7.2 of TS 23.502[3].

6.x.3.2	Procedure for BDT warning notification with Energy related analytics

Figure 6.x.3-2: Procedure for BDT warning notification

The procedure described in this clause is based on the BDT warning notification as defined in clause 4.16.7.3 of TS 23.502[3].

The negotiation for BDT described in clause 6.x.3.1 is completed. In addition, the H-PCF has subscribed to Energy related analytics from the NWDAF for the Area of interest and the Desired time window of a BDT policy.

The H-PCF is notified with the Energy related analytic in the Area of interest from the NWDAF when the NWDAF determines that the requested Energy related KPIs goes below the threshold.

NOTE:	How the NWDAF generates the analytics for Energy related conditions is not addressed in the proposed solution but uses other solutions (e.g. Solution #12).

The H-PCF may retrieve all the BDT Policies together with the relevant information received from the AF and identifies the BDT policies affected by the notification received from the NWDAF. The H-PCF decides based on operator policies, whether a new list of candidate BDT policies can be calculated for the ASP, and sends the BDT warning notification to the AF via the NEF as defined in clause 4.16.7.3 of TS 23.502[3].

6.x.4	Impacts on existing services, entities and interfaces

NWDAF:

-	Support for the Energy related analytics.

NEF:

-	Nnef_BDTPNegotiation service is extended to support the Energy related KPIs and Energy analytics activation flag.

H-PCF:

-	Request/Subscribe to the Energy related analytics and determines BDT policies based on information.

# *** End of Changes ***