3GPP TSG-WG SA2 Meeting #161	S2-2402232

Athens, Greece, 26 February - 01 March, 2024	(revision of S2-2400374)

Source:	Ericsson

Title:	KI #5, New Sol: Signalling of dynamically updated traffic periodicity

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM_Ph2 / Rel-19

Abstract: Introduces solution to KI#5 considering dynamic changes of periodicity.

# 1. Introduction/Discussion

KI#5 addresses following aspects:

-	Identify what traffic characteristics are dynamically changed.

-	Whether and how the 5G network can be enhanced to know about dynamic changes in traffic characteristics of GBR and non-GBR flows.

-	What information from the Application in the DN, if any, is needed by the 5G network to be able to know the dynamic changes in traffic characteristics and how this information is provided by the Application in the DN.

-	At what granularity does the 5GS need to know of changes in traffic characteristics (e.g., QoS Flow granularity).

-	What handling may be needed when the 5G network knows about changes in the traffic characteristics.

In this solution we identify traffic periodicity as a dynamic characteristic that should be signalled in-band, i.e., in the User Plane, and outline the impacts stemming therefrom.

# 2. Text Proposal

It is proposed to introduce the following changes vs. TR 23.700-70.

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policies and Charging control framework for the 5G System; Stage 2".

[5]	IETF RFC 3711: "The Secure Real-time Transport Protocol (SRTP)", March 2004.

[6]	IETF RFC 6904: "Encryption of Header Extensions in the Secure Real-time Transport Protocol (SRTP)".

[7]	IETF RFC 9335: "Completely Encrypting RTP Header Extensions and Contributing Sources".

[8]	IETF draft-ietf-avtcore-rtp-over-quic: "RTP over QUIC (RoQ)".

[9]	IETF draft-ietf-moq-transport: "Media over QUIC Transport".

[10]	IETF experimental draft-ietf-avtext-framemarking: "Frame Marking RTP Header Extension".

[11]	IETF RFC 9000: "QUIC: A UDP-Based Multiplexed and Secure Transport".

[12]	TR 26.926 v18.1.0: "Traffic Models and Quality Evaluation Methods for Media and XR Services in 5G Systems".

[13]	TR 26.925 v18.1.0: "Typical traffic characteristics of media services on 3GPP networks".

[14]	IETF RFC 9330: "Low Latency, Low Loss, and Scalable Throughput (L4S) Internet Service: Architecture

[15]	IETF RFC 9331: "The Explicit Congestion Notification (ECN) Protocol for Low Latency, Low Loss, and Scalable Throughput (L4S)".

[16]	IETF draft-ietf-tsvwg-ecn-encap-guidelines-22: “Guidelines for Adding Congestion Notification to Protocols that Encapsulate IP”.

[17]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[18]	CableLabs DOCSIS MULPI: "Data-Over-Cable Service Interface Specifications DOCSIS 3.1, MAC and Upper Layer Protocols Interface Specification".

[19] 	IETF RFC 9332: " Dual-Queue Coupled Active Queue Management (AQM) for Low Latency, Low Loss, and Scalable Throughput (L4S)".

[20]	IETF TS26.522: "5G Real-time Media Transport Protocol Configurations".

[21] 	IETF ﻿draft-ietf-tsvwg-udp-options: "Transport options for UDP".

[22]	IETF RFC 6363: "Forward Error Correction (FEC) Framework".

[23]	IETF RFC 6364: "Session Description Protocol Elements for the Forward Error Correction (FEC) Framework".

[24]	IETF RFC 6681: "Raptor Forward Error Correction (FEC) Schemes for FECFRAME".

[25]	IETF RFC 6682: "RTP Payload Format for Raptor Forward Error Correction (FEC) ".

[26]	IETF RFC 6695: "Methods to Convey Forward Error Correction (FEC) Framework Configuration Information".

[27]	IETF RFC 6816: "Simple Low-Density Parity Check (LDPC) Staircase Forward Error Correction (FEC) Scheme for FECFRAME".

[28]	IETF RFC 6865: "Simple Reed-Solomon Forward Error Correction (FEC) Scheme for FECFRAME".

[29]	IETF RFC 8680: "Forward Error Correction (FEC) Framework Extension to Sliding Window Codes".

[30]	IETF RFC 8681: "Sliding Window Random Linear Code (RLC) Forward Erasure Correction (FEC) Schemes for FECFRAME".

[31]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[32] 	IETF RFC 8627: " RTP Payload Format for Flexible Forward Error Correction (FEC) ".

[x]	3GPP TS 38.415: "NG-RAN; PDU Session User Plane Protocol".

Editor's note:	References [8], [9] and [10] cannot be formally referenced until published as RFC.

# * * * * Next change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issue # |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | <KI #1> | <KI#2> | <KI#3> | <KI#4> | <KI#5> | <KI#6> | <KI#7> | <KI#8> | <KI#9> |
| #1: PDU Set content ratio awareness at RAN | x |  |  |  |  |  |  |  |  |
| #2: Discarding of redundant PDUs (FEC) and reporting | x |  |  |  |  |  |  |  |  |
| #3: FEC mechanism and PSI based PDU Set QoS Handling Enhancement | x |  |  |  |  |  |  |  |  |
| #4: PDU Set FEC-based PDU Set QoS Handling | x |  |  |  |  |  |  |  |  |
| #5: PDU Set Handling and Information marking …for PSDB/PSER/PSIHI | x |  |  |  |  |  |  |  |  |
| #6: Enhanced Alternative QoS Profiles for PDU set based QoS handling | x |  |  |  |  |  |  |  |  |
| #7: Enhancing alternative QoS profile …PDU set QoS parameters | x |  |  |  |  |  |  |  |  |
| #8: Consistent PDU Set Handling between AF and 5GS | x |  |  | x |  |  |  |  |  |
| #9: PDU Set information identification for encrypted traffic |  | x |  | x |  |  |  |  |  |
| #10: PDU Set information identification based on MoQ |  | x |  |  |  |  |  |  |  |
| #11: RTP over QUIC based Encrypted Traffic …QoS flows mapping |  | x |  |  |  |  |  |  |  |
| #12: Obfuscated Metadata to Classify Payload in Encrypted Media Packets |  | x |  | x | x |  |  |  |  |
| #13: Multiple DSCP markings per QoS Flow |  |  | x |  |  |  |  |  |  |
| #14: Extending Packet Filter … within a single transport connection |  |  |  | x |  |  |  |  |  |
| #15: Traffic Detection and QoS mapping for XR and Media services |  |  |  | x |  |  |  |  |  |
| #16: AS based trigger of data boost handling with reflective QoS |  |  |  |  | x |  |  |  |  |
| #17: L4S in non-3GPP access networks |  |  |  |  |  | x |  |  |  |
| #18: PDU Set handling in wireline/wireless non-3GPP access |  |  |  |  |  |  | x |  |  |
| #n: Signalling of dynamically updated traffic periodicity |  |  |  |  | x |  |  |  |  |

Editor's note:	This clause lists the key issue(s) addressed by this solution.

# * * * * Next change (all new text)* * * *

## 6.X	Solution #n: Signalling of dynamically updated traffic periodicity

### 6.X.1	Key Issue mapping

This solution addresses key issue #5.

### 6.X.2	Description

The need to provide information about traffic periodicity to NG-RAN has been agreed in Rel-18 and its support introduced in the CP over NGAP. This approach is suitable for a static or very infrequent changes of that characteristic. Basically, in Rel-18 it is assumed that traffic periodicity either does not change throughout the session or changes only very infrequently. In Rel-19 additional, typical scenarios where traffic periodicity changes in a dynamic/frequent fashion are proposed to be considered and addressed by a new functionality.

In case of following scenarios, the periodicity of user data generation changes due to:

- Rate adaptation of a media (e.g., video) because of changes to the latency due to increased queue length, e.g., triggered by ECN Marking for L4S.

- Change of media content characteristics during an ongoing session, e.g., application session state transition that results in changing of both frame (PDU Set) sizes and framerate based on media content.

The example scenarios have following characteristics:

- They occur without a prior knowledge, e.g., the need for rate adaptation can be triggered by increased load in a cell.

- They can appear very frequent as the link conditions may vary very frequently, e.g., rate adaptation triggered by L4S.

- They require an immediate notification for proper resource management, e.g., to NG-RAN to reallocate the resources so that delay and UE power consumption can be optimised.

Following current protocol, such frequent changes of the periodicity value will need to be signalled during the setup or the modification of the PDU session procedure from SMF to NG-RAN, which requires to run the PDU Session Management procedure over many network interfaces (e.g, NG, F1, E1, Xn). This is too onerous in terms of signalling resources, especially for frequent changes of the periodicity value.

Given the dynamicity of the changes of the traffic periodicity and its urgency outlined in the scenarios above, this solution introduces support of signalling new periodicity information in the in-band, i.e. in the User Plane on N6 and N3, in order to let NG-RAN be aware rapidly of its change and use it accordingly.

Editor's note:	This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). Sub-clause(s) may be added to capture details.

### 6.X.3	Procedures

The periodicity information is encapsulated inside, e.g., the RTP Header Extension or other protocol information used to carry PDU Set Information that is received and processed on the N6 interface at the UPF. The UPF extracts and forwards the information to NG-RAN within a GTP-U extension header PDU Set Information User Plane protocol on the N3 interface defined in TS 38.415 [x]. The gNB-CU-UP node receives the information also forwards it to relevant nodes in the NG-RAN making use of the information, e.g, to the MAC scheduler in the gNB-DU.

Editor's note:	This clause describes high-level procedures and information flows for the solution.

### 6.X.4	Impacts on services, entities and interfaces

UPF	Identification of the periodicity on N6. Addition of the periodicity identified on N6 to the PDU Set Information on N3.

NG-RAN	Reception of the periodicity in the PDU Set Information on N3.

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

# * * * * End of changes * * * *