SA WG2 Meeting #161	S2-2402253

February 26 - March 1, 2024, Greece

Source:	China Mobile

Title:	New Solution on Network support for NTZ management

Document for:	Approval

Agenda Item:	19.10

Work Item / Release:	FS_UAS_Ph3 / Rel-19

Abstract of the contribution: This contribution proposes a new solution for Key Issue #1 "Support of No Transmit Zones".

1. Discussion

This contribution is related to KI#1: Support of No Transmit Zones where including the content "whether mechanisms are needed to differentiate aerial UEs that support functions defined for NTZs in Rel. 19 and aerial UEs that don't". Here we want to discuss about the network support to manage the aerial UEs those don't support functions defined for NTZs.

Therefore, we should consider the scenario that the UE doesn't support functions defined for NTZs wants to perform flight mission via 5G network in NTZ. There is a need to align NTZ setting with 5G network. so 5G network can manage UE based on the right aerial information(NTZ). Meanwhile, the scenario that some areas are set to NTZ for temporary use should be considered. The 5GS must have solution to support NTZ to guarantee the security of NTZ.

2. Text Proposal

It is proposed to agree the following changes into TR 23.700-59.

# * * * * First change * * * *

6.X	Solution #X: Network support for NTZ management

6.X.1	Key Issue mapping

This solution resolves KI#3.

6.X.2	Description

This solution addresses Key Issue #1 "Support of No Transmit Zones".

Considering the scenario that the UE doesn't support functions defined for NTZs wants to perform flight mission via 5G network in NTZ and the scenario that some areas are set to NTZ for temporary use. There is a need to align NTZ setting with 5G network, so 5G network can manage aerial UEs based on the right aerial information (NTZ). In order to avoid the impact on non-aerial UEs, AMF needs to determine whether the UE is an aerial UE, and if yes, AMF can compare the NTZ information with the UE's local information to determine whether the user can continue communicate service(e.g. register or TAU) via 5G network.

6.X.3	Procedures

Figure 6.X.2-1: Procedure for network support for NTZ

0.	The AMF can store the mobile network cells referring to the NTZ information which is configured locally or obtained from the UTM/USS via UAS-NF/NEF.

1.	The UE sends a Registration request message and, and it shall provide a CAA-level UAV ID of the UAV if configured with one.

2.	AMF could determine whether the UE is an aerial UE according to the CAA-level UAV ID or aerial subscription, and if yes, AMF can compare the NTZ information with the Cell ID or TA where the UE is to determine whether the UE can continue registration.

3.	AMF sends Registration Accept/Registration Reject message to UE.

4.	The AMF can update and store the mobile network cells referring to the new NTZ information configured locally or obtained from the UTM/USS via UAS-NF/NEF, and some NTZ update is for temporary use.

5.	The UE sends a TAU request message due to the UE mobility.

6.	AMF could determine whether the UE is an aerial UE according to the CAA-level UAV ID or aerial subscription, and if yes, AMF can compare the NTZ information with the Cell ID or TA where the UE will move to, to determine whether the UE can continue TAU.

7.	AMF sends TAU success/TAU Failure message to UE.

6.X.4	Impacts on Existing Nodes and Functionality

AMF:

-	Obtain data from UTM/USS referring to NTZ and store it as aerial network cells information.

# * * * * End of changes * * * *