SA WG2 Meeting #161	S2-2402254

February 26 - March 1, 2024, Greece

Source:	China Mobile

Title:	New Solution on USS/UTM relocation

Document for:	Approval

Agenda Item:	19.10

Work Item / Release:	FS_UAS_Ph3 / Rel-19

Abstract of the contribution: This contribution proposes a new solution for Key Issue #1 "Enhancement of NEF services to support service exposure and interactions between MNOs and UTM functions".

1. Discussion

This contribution is related to KI#1: Enhancement of NEF services to support service exposure and interactions between MNOs and UTM functions, which including one detail requirement considering "The scenario of multiple USS/UTM serving different geographical areas corresponding to the UAV flight path". It is to be studied the interaction between multiple USS/UTMs and UAV (e.g. USS/UTM relocation) if multiple USS/UTMs are serving for different geographical areas which are along the same flight path for one UAV.

Therefore, there is a scenario where USS/UTM relocation is required due to the change in UAV mobility: for the location movement of UAV, the current USS/UTM may not be able to provide flight-related service after UAV move to the area out of its control. Considering the above scenario, the 5GS suggests to define solution to support USS/UTM relocation to guarantee the continuity of UAV flight.

2. Text Proposal

It is proposed to agree the following solution into TR 23.700-59.

# * * * * First change * * * *

6.X	Solution #X:  USS/UTM relocation

6.X.1	Key Issue mapping

This solution resolves KI#1.

6.X.2	Description

This solution addresses Key Issue #1 "Enhancement of NEF services to support service exposure and interactions between MNOs and UTM functions".

Considering the scenario of multiple USS/UTM serving different geographical areas corresponding to the UAV flight paths, there are some cases that the serving USS/UTM may need to be changed during the UAV flight due to UAV mobility. USS/UTM can trigger the re-selection of the USS/UTM based on the location change of the UAV. After the re-selection, the USS/UTM can migrate the UAV context to the new USS/UTM. The UAV Context includes subscribed events, subscriber addresses, and UAS NF addresses. For the subscribed events, new USS/UTM may re-initiate the subscription after the relocation.

6.X.3	Procedures

Figure 6.X.3-1: Procedure for USS/UTM relocation

1.	The old USS/UTM subscribes to UAV position reporting events to AMF/GMLC/LMF through UAS AF. The event reporting type could be periodic reporting. See 5.3.2 Procedure for UAV location reporting in TS 23.256.

2.	Once old USS/UTM detects that the UAV has left its service area based on the location information provided in step1, old USS/UTM may determine to trigger USS/UTM relocation to find a new USS/UTM to serve UAV.

3.	The old USS/UTM may execute the UUAA revocation procedure by USS/UTM. See 5.2.7 UUAA Revocation by USS/UTM for details in TS 23.256.

4.	The old USS/UTM triggers the process of migrating the UAV context to new USS/UTM. The UAV context includes subscribed events, subscriber addresses, and UAS NF addresses. For subscribed events, new USS/UTM may re-initiate the subscription to the subscriber address.

5.	New USS/UTM initiates UUAA Re-authorization procedure via UAS NF. For details, see 5.2.4.3 UUAA Re-authorization procedure in TS 23.256. In this process, the new USS/UTM needs to inform the UAV with the new USS/UTM address.

6.	New USS/UTM executes UUAA Re-authentication procedure, which will trigger the UAV to report the CAA level UAV ID. For details, see 5.2.4.1 UUAA Re-authentication procedure in TS 23.256.

6.X.4	Impacts on Existing Nodes and Functionality

USS/UTM:

-	Supports UAV context transfer procedure

-	Supports to inform the target USS/UTM of handover preparation and handover of UAV subscriptions.

# * * * * End of changes * * * *