3GPP SA WG2 Meeting #161	S2-2402259

Athens, February 26 – March 1, 2024

Source:	Ericsson

Title:	Architecture Assumptions and Requirements: Update to consider 5GS

Document for:	Approval

Agenda Item:	19.14

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: The contribution discusses and proposes to include 5GS into Architecture Assumptions and Requirements.

1. Introduction

In the discussion of Architecture Assumption and Requirements, there used to be the following sentences or similiar in the clause of Architecture Assumptions in some revisions of S2-2400179:

The service-based architecture as specified in TS 23.501 [X2], TS 23.502 [X3], and TS 23.503 [X4] can be taken in account for this study. But the mandatory features and definitions in those specifications are not necessarily assumed to be baseline for this study.

Also, in the clause of Architecture Requirements, there used to be the following sentences or similar:

Solutions based on 5G system should build on the 5G System architectural principles for core network as defined in clause 4.1 of TS 23.501 [2].

During the discussion, there were some views that the solution cannot be restricted by the current 5GS, especially for those mandatory architecture principles. To reach consensus, some compromises had been made to remove all the 5GS related descriptions in all contributions, including the Architecture Assumptions and Requirements as well as Key Issues.

In the clause 4.1 of TS 22.369, it clearly states Ambient IoT technology service requirements is part of 5GS.

This Technical Specification describes the Ambient IoT technology service requirements as part of the 5G system to enable new services and use cases.

In the service requirements, all the requirements are towards 5GS, e.g., :

- Communication: The 5G system shall be able to support 5G network or an Ambient IoT capable UE to communicate with a group of Ambient IoT devices simultaneously……

- Positioning: The 5G system shall support location services for Ambient IoT devices (e.g., to locate Ambient IoT devices using absolute or relative positioning methods)……

- Management: The 5G network shall support suitable management mechanisms for an Ambient IoT device or a group of Ambient IoT devices……

- Exposure: ……The 5G system shall enable an authorized 3rd party to instruct the 5G network to trigger a group of Ambient IoT devices in an specific area and which action the Ambient IoT devices need to perform when triggered (e.g. send ID, receive further information, send measurement value).

- Charging: The 5G system shall be able to collect charging information in a suitable way for using Ambient IoT services on per Ambient IoT device basis or a group of Ambient IoT devices (e.g., total number of communications per charging period).

- Security and privacy: The 5G system shall enable security protection suitable for Ambient IoT, without compromising overall 5G security protection......

In our view, all the solutions for Ambient IoT needs to be based on 5GS or at least take the 5GS into account, and reuse existing NFs when applicable (e.g., NEF for exposure, CHF for charging, etc.). The solutions need to follow the 5G System architectural principles when applicable (e.g. modularize the function design, define procedures as services, etc.)

And after the normative work, the new NFs and the new procedures will become a part of the 5GS, so that 5GS can fulfil the service requirements defined in TS 22.369.

[Proposal] Include 5GS in Architecture Assumptions and Requirements.

NOTE:	By listing 5GS we are also implicitly avoiding earlier systems like EPS to be in scope of the work.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-13 v0.1.0:

* * * Start of Change * * * *

## 4.1	Architectural Assumptions

The architecture and framework as specified in TS 23.501 [4] is assumed to be taken into account when applicable.

-	The following traffic types for Ambient IoT Device are to be studied:

-	DT: Device-terminated; and

-	DO-DTT: Device-originated - device-terminated triggered.

NOTE 1:	The final decision for including DO-A (Device-originated - autonomous) in the study depends on RAN decision.

-	The following two connectivity topologies as defined in TR 38.848 [7] are to be studied:

-	Topology 1: BS <--> Ambient IoT Device;

-	Topology 2: BS <--> intermediate node <--> Ambient IoT Device: Only a UE can act as an intermediate node which is under the network control.

-	The communication spectrum is assumed to be licensed.

-	Handover is not supported.

-	RRC states are not supported by AIoT Devices (see RP-234058 [3])

-	No mobility (i.e. at least no cell selection/re-selection-like function) supported by AIoT Devices (see RP-234058 [3])

Editor's note:	The RAN SID reference is to be updated to RAN TR when available, and the meaning of no mobility is to be clarified by RAN.

NOTE 2: 	Coordination with RAN is required to determine the Ambient IoT Device capabilities in relation to system level of functionality (considering e.g. traffic scenarios, connectivity topologies etc.).

NOTE 3:	The security aspects for Ambient IoT requires coordination with SA WG3.

NOTE 4:	The charging aspects for Ambient IoT will be studied by SA WG5.

NOTE 5:	The NAS based Congestion control is not in the scope of this study.

* * * Next of Change * * * *

## 4.2	Architectural Requirements

The following architectural requirements are applicable to this study:

- 	Solutions and architecture to support Ambient IoT need to comply with 5G System architectural principles for core network as defined in clause 4.1 of TS 23.501 [4] when applicable.

-	Support for AIoT Services needs to adhere to the nature of the AIoT Devices (e.g. ultra-low complexity, power, cost and resource-constrained).

-	Support of the security aspects needs to consider the nature of the AIoT Devices (e.g. ultra-low complexity power, cost and resource-constrained) while addressing e.g. confidentiality, integrity, etc.

* * * End of Change * * * *