SA WG2 Meeting #S2-161	S2-2402273

26 February - 3 March, 2024, Athens, Greece                                 (revision of S2-23XXXX)

Source: 	vivo

Title: 	KI#3, update solution 8

Document for: 	Approval

Agenda Item: 	19.2

Work Item / Release:	FS_NG_RTC_Ph2 / Rel-19

Abstract of the contribution: This contribution proposes to update solution#8.

- Introduction

This contribution proposes to make the solution#8 more general to fit for more DC applications and deletes one EN.

2	Discussion

Solution#8 has introduce how to perform interworking for File Transfer APP, other DC applications, such as the Location Share APP, can also use this method. For example, when the DCMTSI UE wants to share its location to MTSI UE, the DCMTSI UE sends its location to DC AS through the P2A application DC, and DC AS generates a website which can show the location to the MTSI UE, and sends the hyperlink of the website to MTSI UE by SMS, then the user of MTSI UE has the ability to access the website to see the real-time location of the DCMTSI UE if the user wants.

The interworking actions for different DC Application are different, and the detail of how to perform the interworking actions can be per-defined in the DC Application Server. The DCSF does not need to know what interworking actions the DC Application Server should be performed, the DCSF just triggers the DC Application Server to perform the interworking actions when determining the peer UE is a MTSI UE, so the EN in solution#8 can be deleted:

Editor's note:	It is FFS how the DCSF knows the DC AS can sends SMS and whether the step 9 is really needed.

3     Proposal

It is proposed to capture the following contents into TR 23.700-77.

* * * * Start of 1st Change * * * *

6.8	Solution #8: Interworking between a DCMTSI UE and an MTSI UE by using messaging

6.8.1	Description

This solution addresses Key Issue #3 "Data channel interworking with MTSI UE" and focuses on how to support interworking by using messaging.

For the MTSI UE, the legacy media types usually include video media, audio media, message etc. This solution proposes a method to perform interworking between DCMTSI UE and MTSI UE by using messaging, i.e. SMS.

For example, when sending a file from DCMTSI UE to MTSI UE, the DCMTSI UE sends the file to DC Application Server through the P2A application DC, and DC Application Server sends a hyperlink of the file to the MTSI UE by SMS, then the user of MTSI UE has the ability to download it using the hyperlink if the user wants.

Another example, when sharing location from DCMTSI UE to MTSI UE, the DCMTSI UE sends its real-time location to DC Application Server through the P2A application DC, and DC Application Server generates a website which can show the location to the MTSI UE, and sends the hyperlink of the website to MTSI UE by SMS, then the user of MTSI UE has the ability to access the website to see the real-time location of the DCMTSI UE if the user wants.

6.8.2	Procedures

Figure 6.8.2-1: Person-to-Application (P2A) Application Data Channel set up Signalling Procedure

Precondition:

UE#1 is a DCMTSI UE, UE#2 is a MTSI UE.

The user of UE#1 wants to transfer a file, e.g. a photo, to the user of UE#2 during the call or the user of UE#1 wants to share his/her real-time location to the user of UE#2.

The steps in the call flow are as follows:

0-27. Step 0-27 from clause AC.7.2.2: (Person-to-Application (P2A) Application Data Channel Setup) in TS 23.228 [5] is performed with the differences that:

In step 0, The bootstrap DC will only be established between UE#1 and MF/MRF.

UE#1 can know which DC application support interworking, e.g. the DCSF only provides the application list which support interworking to UE#1 when the DCSF determines that UE#2 or the network of UE#2 does not support data channel.

UE#1 determines that UE#2 or the network of UE#2 doesn't support data channel based on the received response.

Editor's note:	It is FFS how does the DCSF know the UE#2 or the network of UE#2 does not support interworking.

In step 1, when the user of UE#1 selects the DC application, such as File Transfer APP or Location Share APP, the UE#1 initials the P2A application data channel establishment procedure based on UE#2 or the network of UE#2 doesn't support data channel, and the DC application supports interworking.

In step 9, DCSF additionally sends the UE#2' SIP URI and an interworking indication to the DC Application Server to indicate that interworking should be performed when determines that UE#2 does not support data channel.

The UE#2's SIP URI is received by DCSF during the bootstrap DC establishment procedure.

NOTE 1:	Details on how DCSF communicates with the DC Application Server is out of scope of this Release.

28.	UE#1 sends what he/she wants to share to UE#2, such as the File or its real-time location, to DC Application Server through the P2A application DC.

29.	DC Application Server performs the predefined interworking actions based on the interworking indication, such as:

For the DC Application Server of File Transfer APP, the predefined interworking action is to generate a hyperlink associated with the received File.

For the DC Application Server of Location Share APP, the predefined interworking action is to generates a hyperlink associated with a website which can show the location to the UE#2.

30.	DC Application Server sends a SMS with the hyperlink to DCSF as specified in TS 24.341 [15].

NOTE 1:	Details on how DC Application Server performs the interworking action is out of scope of this Release.

31.	The user of UE#2 can download the File using the hyperlink or access the website to see the real-time location of UE#1 if he/she wants.

NOTE 2:	How the user of UE#2 trust the hyperlink is out of scope of this specification. E.g. the user of UE#2 can trust the hyperlink based on the oral negotiation with the user of UE#1.

6.8.3	Impacts on Existing Nodes and Functionality

This solution has some requirements on the existing IMS nodes below:

DCSF:

-	Determine that the peer UE or the network of peer UE does not support data channel.

-	Send peer UE's SIP URI and an interworking indication to DC Application Server to indicate interworking should be performed.

DC Application server:

-	Performs predefined interworking actions based on the interworking indication from DCSF.

-	Send a SMS with hyperlink to peer UE.

* * * * End of Changes * * * *