3GPP TSG WG SA2 Meeting #161	 	S2-2402385

26 February - 1 March, 2024, Athens, Greece

Title:	[draft] Reply LS on Support of interworking between SA4 RTC and IMS

Response to:	LS on Support of interworking between SA4 RTC and IMS (S2-2400070/S4-232055)

Release:	Release 18

Work Item:	FS_eiRTCW

Source:	China Mobile (will be SA2)

To:	SA4

Cc:	SA1, SA3, SA5, CT1, CT3

Contact person:	Yi Jiang

jiangyi(at)chinamobile(dot)com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

SA2 thanks SA WG4 for the LS on Support of interworking between SA4 RTC and IMS. SA2 has the following answers to the question from SA4.

Question:

Are there any potential impacts on existing IMS specifications from the above NNI-based solution (connecting the WebRTC-based RTC communication network to IMS network, by applying the interworking model specified in 3GPP TS 29.162), considering that the solution uses the interface of existing 3GPP IMS specification without any modifications?

Answer:

SA2 finds that in 3GPP TS 26.930, which is the output specification of R18 SA4 FS_eiRTCW study item, it describes that:

One of the main scopes of this document is to study the enhancements of RTC network for supporting collaboration scenario 4 specified in 3GPP TS 26.506 [12]. Collaboration scenario 4 enables inter-operable WebRTC services, which is extended collaboration scenario 3 with functions to support MNO to MNO inter-operability. This collaboration scenario 4 could include the roaming case, but this case is outside the scope of this document.

According to 3GPP TS 26.506, collaboration scenario 3 is that MNO hosts the WebRTC sessions by providing the trusted WebRTC signalling server in RTC AS, and collaboration scenario 4 is MNO to MNO inter-operability.

SA2 has identified the following questions regarding to the RTC network studied in R18 SA4 FS_eiRTCW study item:

- What’s the relationship between the RTC network and the MNO? Whether the RTC network is used by the MNO and will support roaming and interworking scenario?

If the answer of question#1 is yes, SA2 thinks that the following questions need to be clarified further:

- What are the requirements and service scenarios of the RTC network and where them came from? Have they been discussed and endorsed by 3GPP SA1?

- What’s the relationship between the RTC network and the existing IMS network? For the RTC network can provide legacy audio/video call service, can it be the evolved alternative of the existing IMS network?

- Operated by the MNO and providing legacy audio/video call service, the RTC network need to comply with regulations for telephony service in the country (e.g., high-availability, support of emergency services, support of lawful interception, high speech quality and high security-level.). Have these aspects been discussed and endorsed by 3GPP SA3?

- Have the OAM and charging aspects of the RTC network been discussed and endorsed by 3GPP SA5?

- Have the roaming and interworking scenarios of the RTC network been discussed and endorsed by GSMA?

2	Actions

To SA WG4:

ACTION: 	SA2 kindly asks SA4 group for clarifications regarding the questions above and provide feedback if any.

3	Dates of next SA WG2 meetings

SA2#162	15th – 19th Apr. 2024	Changsha, China

SA2#163	27th – 31st May. 2024	TBD, Korea