SA WG2 Meeting #161	S2-2402389

Athens, GREECE, February 26 – March 01, 2024

Source:	OPPO

Title:	KI#4, Solution#14 update

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM Ph2/Rel-19

Abstract of the contribution: This contribution proposes to update solution#14 of KI#4 to resolve ENs.

1.	Discussion

The paper proposes to update solution#14 of KI#4 to resolve ENs:

Editor's note:	How to ensure that an application can provide the stream info to the lower layers of the UE in a reliable way is FFS. Other options for the UE to receive the stream info are FFS.

For UL traffic, lower layer of the UE can read the stream info from the packet header, e.g. when QUIC is used, and if the stream info is Connection ID as defined in IETF QUIC [11], because the Connection ID is unencrypted, lower layer of the UE can directly read stream info, i.e. Connection ID from the QUIC header. If the stream info is Stream ID as defined in IETF QUIC [11], the Stream ID is encrypted, how lower layer of the UE read the stream info from the traffic header is by Interlayer interaction of the UE, i.e. by UE’s implementation, for example, lower layer of the UE may need to get a secure key from its application layer to read the Stream ID. The EN can be removed with clarifications.

Editor's note:	Whether UPF can get the stream info or part of stream info directly from the QUIC header is FFS.

Similar as description above, when QUIC is used, and if the stream info is Connection ID as defined in IETF QUIC [11], because the Connection ID is unencrypted, UPF can directly read stream info, i.e. Connection ID from the QUIC header. If the stream info is Stream ID which is encrypted and cannot be read directly from the QUIC header, the stream info needs be carried in the N6 header.

Editor's note:	The protocol used for including this information in N6 tunnelling encapsulation header is FFS.

As mentioned in Solution #22 of TR 23.700-60, the use of GTP-U on N6 is already supported in TS 29.244, thus can be used for the N6 tunnelling to carry stream info.

2.	Text proposal

It is proposed to agree the following changes to TR 23.700-70:

* * * 1st  change* * *

6.14	Solution #14: Extending Packet Filter Set to identify multiplexed traffic flows within a single transport connection

6.14.1	Key Issue mapping

This solution addresses Key Issues #4(Traffic detection and QoS flow mapping for multiplexed data flows).

6.14.2	Description

When several media streams are multiplexed on the same transport layer connection, e.g. when Transport protocol like IETF QUIC [11] is used, current Packet Filter Set can be extended by adding stream info to identify the media streams within a single transport connection. The stream info, for example, could be Connection ID or Stream ID as defined in IETF QUIC [11].

For IP PDU Session Type, the extended Packet Filter includes:

-	current Packet Filter as defined in clause 5.7.6 of TS 23.501 [2].

-	Stream info.

For downlink traffic, if the stream info which carried in the traffic header is not encrypted, e.g. the Connection ID defined in IETF QUIC [11], UPF directly read stream info from the traffic header. If the stream info carried in the traffic header is encrypted, e.g. the Stream ID defined in IETF QUIC [11], AS can provide stream info in the N6 tunnelling encapsulation header towards UPF, an example of the packet format from N6 is depicted in Figure 6.14.2-1. Based on the extended Packet Filter Set, the downlink traffic filtering can be performed by UPF. Any suitable tunnelling technology can be used for the N6 tunnel between the UPF (PSA) and the AS that can carry vendor-specific information, one example is GTP-U encapsulation over N6 as described in TS 29.244 [x].

Figure 6.14.2-1: Example of the packet format from N6

For uplink traffic, if the stream info which carried in the traffic header is not encrypted, e.g. the Connection ID defined in IETF QUIC [11], lower layer of the UE can directly read stream info from the traffic header. If the stream info carried in the traffic header is encrypted, e.g. the Stream ID defined in IETF QUIC [11], how lower layer of the UE read the stream info from the traffic header is by interlayer interaction of the UE, i.e. by UE’s implementation. The detail of UE’s interlayer interaction is not specified and limited in this solution. Then UE performs the uplink traffic filtering with extended Packet Filter Set.

The binding of traffic onto QoS Flows reuse the existing functionality specified in clause 6.1.3.2.4 of TS 23.503 [4].

6.14.3	Procedures

The procedure to identify multiplexed traffic flows is described in the Figure 6.14.3-1.

Figure 6.14.3-1: Procedure to identify multiplexed traffic flows

1.	AF provides service information to the PCF by invoking Npcf_PolicyAuthorization_Create Request or Npcf_PolicyAuthorization_Update Request service operation as described in step 1a in clause 4.16.5.2 of TS 23.502 [3]. In this step AF provides the extended Packet Filters which including stream info to PCF. NEF can be involved between the AF and PCF.

2.	PCF generates the authorized PCC rule(s) based on the AF request, and performs PCF initiated SM Policy Association Modification procedure as defined in clause 4.16.5.2 of TS 23.502 [3] to provide the PCC rule(s) to SMF. The PCC rule(s) also include the extended Packet Filters.

3.	SMF reuses the existing functionality specified in clause 6.1.3.2.4 of TS 23.503 [4] for QoS flow binding.

4.	SMF updates the UPF with N4 Rules related to new or modified QoS Flow(s), the extended Packet Filters are provided to UPF in the N4 Rules for downlink traffic filtering.

5.	N2 SM messages are exchanged between SMF and UE. In this step, UE receives QoS rule(s) with the extended Packet Filters for uplink traffic filtering.

For DL traffic, UPF performs traffic filtering with the received N4 rule(s), the DL packets are classified based on the IP/UDP/ upper layer header /N6 tunnel header and the extended Packet Filters in the N4 rule(s).

For UL traffic, UE performs traffic filtering with the received QoS rule(s). UE evaluates UL packets based on the IP/UDP/upper layer header against the extended Packet Filter Set in a QoS rule. UE can get the stream info from its upper layer by its implementation.

6.14.4	Impacts on services, entities and interfaces

PCF impact:

-	providing PCC rule(s) with extended Packet Filters to SMF.

SMF impact:

-	providing N4 rule(s) with extended Packet Filters to UPF.

-	providing QoS rule(s) with extended Packet Filters to UE.

UPF impact:

-	performs DL traffic filtering based on the IP/UDP/N6 tunnel header and the extended Packet Filters.

UE impact:

-	performs UL traffic filtering based on the IP/UDP/upper layer header and the extended Packet Filters.

AF impact:

-	provide extended packet filter to the NEF/PCF.

NEF impact:

-	Provide extended packet filter to the PCF.

AS impact:

-	include Steam info in the N6 tunneling encapsulation header towards UPF.

* * * 2nd  change* * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policies and Charging control framework for the 5G System; Stage 2".

[5]	IETF RFC 3711: "The Secure Real-time Transport Protocol (SRTP)", March 2004.

[6]	IETF RFC 6904: "Encryption of Header Extensions in the Secure Real-time Transport Protocol (SRTP)".

[7]	IETF RFC 9335: "Completely Encrypting RTP Header Extensions and Contributing Sources".

[8]	IETF draft-ietf-avtcore-rtp-over-quic: "RTP over QUIC (RoQ)".

[9]	IETF draft-ietf-moq-transport: "Media over QUIC Transport".

[10]	IETF experimental draft-ietf-avtext-framemarking: "Frame Marking RTP Header Extension".

[11]	IETF RFC 9000: "QUIC: A UDP-Based Multiplexed and Secure Transport".

[12]	TR 26.926 v18.1.0: "Traffic Models and Quality Evaluation Methods for Media and XR Services in 5G Systems".

[13]	TR 26.925 v18.1.0: "Typical traffic characteristics of media services on 3GPP networks".

[14]	IETF RFC 9330: "Low Latency, Low Loss, and Scalable Throughput (L4S) Internet Service: Architecture.

[15]	IETF RFC 9331: "The Explicit Congestion Notification (ECN) Protocol for Low Latency, Low Loss, and Scalable Throughput (L4S)".

[16]	IETF draft-ietf-tsvwg-ecn-encap-guidelines-22: "Guidelines for Adding Congestion Notification to Protocols that Encapsulate IP".

[17]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[18]	CableLabs DOCSIS MULPI: "Data-Over-Cable Service Interface Specifications DOCSIS 3.1, MAC and Upper Layer Protocols Interface Specification".

[19]	IETF RFC 9332: " Dual-Queue Coupled Active Queue Management (AQM) for Low Latency, Low Loss, and Scalable Throughput (L4S)".

[20]	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations".

[21]	IETF draft-ietf-tsvwg-udp-options: "Transport options for UDP".

[22]	IETF RFC 6363: "Forward Error Correction (FEC) Framework".

[23]	IETF RFC 6364: "Session Description Protocol Elements for the Forward Error Correction (FEC) Framework".

[24]	IETF RFC 6681: "Raptor Forward Error Correction (FEC) Schemes for FECFRAME".

[25]	IETF RFC 6682: "RTP Payload Format for Raptor Forward Error Correction (FEC) ".

[26]	IETF RFC 6695: "Methods to Convey Forward Error Correction (FEC) Framework Configuration Information".

[27]	IETF RFC 6816: "Simple Low-Density Parity Check (LDPC) Staircase Forward Error Correction (FEC) Scheme for FECFRAME".

[28]	IETF RFC 6865: "Simple Reed-Solomon Forward Error Correction (FEC) Scheme for FECFRAME".

[29]	IETF RFC 8680: "Forward Error Correction (FEC) Framework Extension to Sliding Window Codes".

[30]	IETF RFC 8681: "Sliding Window Random Linear Code (RLC) Forward Erasure Correction (FEC) Schemes for FECFRAME".

[31]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[32]	IETF RFC 8627: "RTP Payload Format for Flexible Forward Error Correction (FEC)".

[x]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes; Stage 3".

Editor's note:	References [8], [9] and [10] cannot be formally referenced until published as RFC.

* * * End of change * * *