SA WG2 Meeting #161	S2-2402422

Feburary 26 – March 1 , 2024, Athens

Source:	China Telecom

Title:	KI#1.1, New Sol: Association of the two SUPIs for DualSteer

Document for:	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS/Rel-19

Abstract of the contribution: This paper proposes a new Solution to address key issue #1.1 of the FS_MASSS.

1	Discussion

This paper proposes a new Solution to address Key Issue #1.1of the FS_MASSS.

2	Proposal

It is proposed to include the following changes in TR 23.700-54 V0.1.0.

* * * * Start of Changes – All New Text * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of DualSteer Solutions to Key Issues

|  | Key Issues for DualSteer |  |  |  |
| --- | --- | --- | --- | --- |
| Solution# | <Key Issue #1.1> | <Key Issue #1.2> |  |  |
| #X | X |  |  |  |

## 6.1	Solutions for DualSteer

### 6.1.X	Solution #X:  Association of the two SUPIs for DualSteer

#### 6.1.X.1	Description

This solution addresses Key Issue #1.1 “Subscription aspects to support DualSteer”.

To associate two subscriptions/SUPIs for DualSteer, the term “DualSteer Pair” may be introduce to indicate two SUPIs that share the same subscription profile and has corresponding subscription data stored in UDM/UDR as in Table 6.1.x.1for example:

Table 6.1.x.1: DualSteer Pair Subscription data types

| Subscription data type | Field | Description |
| --- | --- | --- |
| DualSteer Pair | DualSteer Pair Identifier | Identifies the pair of SUPIs that share the same subscription profile that supports DualSteer. |
|  | SUPI list | Corresponding SUPI list, i.e., 2 SUPIs |
| DualSteer Pair Data | DualSteer Pair Identifier | Identifiers of the DualSteer Pair that the DualSteer Pair Data belongs to. |
|  | DualSteer data | (NOTE 1) |
| NOTE 1: The content of Dualsteer data is subject to the solutions to other KIs on DualSteer, considering the subscription information related to access and mobility, session management, and policy aspects, for example the DNN an S-NSSAI specific parameters to support DualSteer. |  |  |

The benefit of introducing the DualSteer Pair is to facilitate the management of the two SUPIs when they act as one DualSteer Device. They could be managed using the share data feature, for example, the network could assign the DNN an S-NSSAI specific parameters to support DualSteer, or the PCF could update policies that targeting the pair.

At the same time, if the UE itself is not capable of dualsteer feature, it can still use the common subscription data of one SUPI, i.e., the network and UE do not need to consider any DualSteer related information.

The DualSteer Pair Subscription Data should be added as part of UE subscription data and adding the field “DualSteer Pair ID” in Table 5.2.3.3.1-1 of TS23.502 to related Subscription data type, such as session management, to indicates that the SUPI has subscribed DualSteer feature if the field is not blank.

NOTE 1: how to update the UE Subscription data types is subject to the solutions to other KIs on DualSteer, considering  the subscription information related to access and mobility, session management, and policy aspects, and will be discussed in normative phase.

#### 6.1.X.2	Procedures

This solution does not impact the current procedures, except additional parameters related to DualSteer is sent.

#### 6.1.X.3	Impacts on services, entities and interfaces

UDR/UDM:

- Providing DualSteer related subscription data.

* * * * End of Changes * * * *