3GPP TSG-WG SA2 Meeting #161	S2-2402468

Athens, Greece, Feb 26 – Mar 1, 2024

Source:	Huawei, HiSilicon

Title:	KI#3: New solution on OAM method to provide efficient mobility and service continuity when served by MWAB

Document for:	Approval

Agenda Item:	19.6

Work Item / Release:	FS_VMR_Ph2 / Rel-19

Abstract: This document proposes to a new solution on Efficient mobility and service continuity when served by MWAB.

1. Introduction

1.1	Background

There is a Key Issue on Efficient mobility and service continuity when served by MWAB:

## 5.4 Key Issue #4: Efficient mobility and service continuity when served by MWAB The following aspects need to be studied for UEs served by the MWAB in the case of mobility in the scenario A and B: - Whether and how to enhance current procedures of mobility and service continuity for a UE. The following aspects need to be considered in potential solutions: - how to reflect the change of MWAB serving cell or location in the mobility management of the UEs served by the MWAB. - how to efficiently manage the mobility of the UEs served by the MWAB, when 5GC node change is necessary. NOTE 2: Mechanisms related to mobility management and service continuity have RAN dependency and should align with the progress of RAN WGs.

In Rel-19, the content of the scope regarding mobility contains the following aspects:

-	How to reflect the change of MWAB serving cell or location in the mobility management of the UEs served by the MWAB.

-	How to efficiently manage the mobility of the UEs served by the MWAB, when 5GC node change is necessary.

The key aspect of the mobility aspect is the way for the MWAB-gNB to provide the expected cell ID or TAC to the UEs that served by the MWAB. The other aspect is to find a proper N2/N3 endpoint of the MWAB-gNB upon mobility.

In Rel-18 VMR work, the configuration of the Cell ID/TAC is configured by the OAM, and the Cell ID/TAC can be further updated by the donor-CU and OAM respectively as defined in clause 8.9.15 and 8.9.16 of TS 38.410. However, in Rel-19, there is no "fixed anchor" node (i.e., full functionalities of the gNB are onboard), and it seems that only the OAM alternative can be used for the MWAB.

It is proposed to make use of/enhance the OAM method defined in Rel-18 VMR to fulfil the requirement of the mobility for Rel-19 MWAB.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-06.

# * * * * First change * * * *

6.0	Mapping of solutions to key issues

Editor's note:	This clause describes the mapping between solutions and key issues.

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions |  |  |  | Key issue#4 |
| olution #Y: OAM method to provide efficient mobility and service continuity when served by MWAB |  |  |  | X |

# * * * * Second change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[4]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description".

[5]	3GPP TS 38.401: "NG-RAN Architecture description".

[6]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[x]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

# * * * * Third change * * * *

6.x	Solution #Y: OAM method to provide efficient mobility and service continuity when served by MWAB

6.x.1	Key Issue mapping

This solution addresses Key Issue #4.

6.x.2	Functional Description

This solution proposes to make use of the OAM method to provide efficient mobility and service continuity when served by MWAB, and consider the following aspects:

-	The address of the serving PLMN OAM is pre-configured at the MWAB.

-	MWAB obtains the MWAB configuration parameters from PLMN OAM via the PDU Session of the MWAB-UE after successfully authorized by the serving PLMN of MWAB-UE, especially the following MWAB configuration parameters should be provided by the PLMN OAM:

-	Cell ID and the associating geographical information;

-	TAC and the associating geographical information;

-	AMF ID and the associating geographical information;

NOTE 1:	The authorization of MWAB-UE is addressed by the solution of KI#1, which is out of scope of this solution.

-	The MWAB-gNB uses the Cell ID and TAC in the SIB message based on its physical location and received MWAB configuration parameters.

-	The MWAB-gNB manages (e.g., establishes, updates, releases) the NGAP connection with the AMF(s) as per the MWAB configuration parameters.

NOTE 2:	The MWAB configuration parameters can be updated by the OAM as per the physical location of MWAB.

6.x.3	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

Figure 6.x.3-1: OAM method to provide efficient mobility and service continuity when served by MWAB

1.	MWAB-UE registers to the network, and if it is authorized to operate as an MWAB-gNB, the flow continues.

2.	MWAB communicates with the OAM of the MWAB-gNB, and obtains the MWAB configuration parameters. The MWAB configuration parameters contains Cell ID(s), TAC(s), AMF ID(s) and the associating geographical information as described in clause 6.x.2.

3.	The MWAB-gNB chooses the Cell ID, TAC, AMF instance based on its physical location and received MWAB configuration parameters.

4.	The MWAB-gNB establishes the NGAP connection with the AMF as defined in TS 38.413, based on the MWAB configuration parameters.

5.	The MWAB-gNB broadcasts the Cell ID and TAC determined in step 3.

6.	In case MWAB detects it needs to update the broadcasted Cell ID/TAC, or establish the NGAP connection with a new AMF, step 7 to 9 continues.

Editor's note:	How does MWAB detects it needs to update the Cell ID/TAC/AMF connection is FFS.

7.	In case only the Cell ID/TAC needs to be updated, MWAB-gNB updates the NGAP connection with AMF as defined in TS 38.413, and the flow continues in step 9.

8.	In case the AMF needs to be re-allocated, the MWAB-gNB releases the NGAP connection with the old AMF (i.e., AMF1), and establishes the NGAP connection with Cell ID/TAC needs to be updated, MWAB-gNB updates the NGAP connection with AMF as defined in TS 38.413, and the flow continues in step 9.

9.	The MWAB-gNB broadcasts the Cell ID and TAC determined in step 6.

6.x.4	Impacts on existing services, entities and interfaces

Editor's note:	This clause captures impacts on existing services, entities and interfaces.

MWAB:

-	Obtains the MWAB configuration parameters from the OAM via the PDU Session of the MWAB-UE.

-	Uses the Cell ID and TAC in the SIB message based on its physical location and received MWAB configuration parameters.

-	Manages (e.g., establishes, updates, releases) the NGAP connection with the AMF(s) as per the MWAB configuration parameters.

OAM:

-	Provides MWAB configuration parameters to the MWAB.

# * * * * End of changes * * * *