SA WG2 Meeting #161	S2-2402483

Athens, GREECE, February 26 – March 01, 2024

Source:	OPPO

Title:	KI#2 New Solution for multi-hop UE-to-UE Relays discovery

Document for:	Approval

Agenda Item:	19.7

Work Item / Release:	FS_5G_ProSe_Ph3/Rel-19

Abstract of the contribution: This contribution proposes a solution for KI#2 for multi-hop UE-to-UE Relays discovery.

1.	Discussion

The paper proposes a new solution for KI#2 on multi-hop UE-to-UE Relays discovery.

2.	Text proposal

It is proposed to agree the following changes to TR 23.700-03:

* * * Start of change (all new text) * * *

6.X	Solution #X Layer-3 multi-hop UE-to-UE Relays discovery

6.X.1	Description

This solution applies to Key Issue #2 " Support of Layer-3 multi-hop UE-to-UE Relays". Model A discovery and Model B discovery procedures are proposed in this solution to support the Layer-3 multi-hop UE-to-UE Relays.

RSC is used to indicate the connectivity service supported by the UE-to-UE Relays, and each RSC is associated with a multi-hop authorization info. The multi-hop authorization info includes:

-	Whether multi-hop UE-to-UE Relay is allowed;

-	The maximum number of hops supported.

The RSC and the multi-hop authorization info are provided to the End UEs and UE-to-UE Relays using the Policy/Parameter provisioning as described in clause 5.1.5 of TS 23.304 [4].

For Model A, Model B discovery, the function of multi-hop UE-to-UE Relay is almost same as one-hop UE-to-UE Relay described in clause 6.3.2.4 of TS 23.304 [4], with following differences:

-	Check the multi-hop authorization info which is associated with RSC to decide whether itself can perform as a multi-hop UE-to-UE Relay.

-	Multi-hop routing info is defined as a list of User Info ID of the UE-to-UE Relay in sequence. The multi-hop routing info is included in the UE-to-UE Relay Discovery Announcement message or UE-to-UE Relay Discovery Solicitation message. When a UE decides itself can act as a multi-hop UE-to-UE Relay, it adds the User Info ID of itself into the received multi-hop routing info, and sends a UE-to-UE Relay Discovery Announcement message or UE-to-UE Relay Discovery Solicitation message. The Multi-hop routing info is also sent to discoverer End UE in the UE-to-UE Relay Discovery Response message

The UEs who monitors the UE-to-UE Relay Discovery Announcement message, or receives UE-to-UE Relay Discovery Response message can utilize the multi-hop routing info for multi-hop UE-to-UE Relay selection.

The multi-hop routing info can be provided by the source End UE in the Direct Communication Request message for the follow up multi-hop UE-to-UE Relay communication setup.

6.X.2	Procedures

Figure 6.x.2-1 is the procedure for Model A discovery.

Figure 6.x.2-1: Model A discovery

Figure 6.x.2-1 shows how End UE-2 receives the Direct discovery set from End UE-1 via multi-hop UE-to-UE relay.

The assumption is that RSC and the multi-hop authorization info have been provided to the End UEs and UE-to-UE Relays using the Policy/Parameter provisioning as described in clause 5.1.5 of TS 23.304 [4].

Step1-2 is same as Step1-2 of the UE-to-UE Relay Discovery with Model A procedure as described in clause 6.3.2.4.2 of TS 23.304 [4]. In step 2, the UE-to-UE Relay Discovery Announcement message includes RSC, Direct discovery set, and Multi-hop routing info. The Multi-hop routing info is {seq_num1 User Info ID of UE-to-UE Relay-1}.

3.	UE-to-UE Relay2 receives UE-to-UE Relay Discovery Announcement message from step 2. UE-to-UE Relay-2 checks the Policy/Parameter provisioning info and decides to act as a multi-hop UE-to-UE Relay. UE-to-UE Relay-2 sends a UE-to-UE Relay Discovery Announcement message. The UE-to-UE Relay Discovery Announcement message contains RSC, Multi-hop routing info {seq_num1 User Info ID of UE-to-UE Relay-1, seq_num2  User Info ID of UE-to-UE Relay-2}, Direct discovery set.

If m UE-to UE Relays have been involved in the multi-hop UE-to-UE Relay Model-A Discovery, the UE-to-UE Relay Discovery Announcement message contains RSC, Multi-hop routing info {seq_num1 User Info ID of UE-to-UE Relay-1, seq_num2  User Info ID of UE-to-UE Relay-2, …seq_num m User Info ID of UE-UE Relay-m }, Direct discovery set.

Figure 6.x.2-2 is the procedure for Model B discovery.

Figure 6.x.2-2: Model B discovery

The assumption is that RSC and the multi-hop authorization info have been provided to the End UEs and UE-to-UE Relays using the Policy/Parameter provisioning as described in clause 5.1.5 of TS 23.304 [4].

Step1-2 is same as Step1-2 of UE-to-UE Relay Discovery with Model B procedure as described in clause 6.3.2.4.3 of TS 23.304 [4]. In step 2, the UE-to-UE Relay Discovery Solicitation message includes RSC, Direct discovery set, and Multi-hop routing info.  The Multi-hop routing info is {seq_num1 User Info ID of UE-to-UE Relay-1}.

3.	UE-to-UE Relay-2 receives UE-to-UE Relay Discovery Solicitation message from step 2. UE-to-UE Relay-2 checks the Policy/Parameter provisioning info and decides to act as a multi-hop UE-to-UE Relay. UE-to-UE Relay-2 sends a UE-to-UE Relay Discovery Solicitation message. The UE-to-UE Relay Discovery Solicitation message contains RSC, Multi-hop routing info {seq_num1 User Info ID of UE-to-UE Relay-1, seq_num2  User Info ID of UE-to-UE Relay-2}, Direct discovery set.

If m UE-to UE Relays have been involved in the multi-hop UE-to-UE Relay Model-B Discovery, the UE-to-UE Relay Discovery Solicitation message contains RSC, Multi-hop routing info {seq_num1 User Info ID of UE-to-UE Relay-1, seq_num2 User Info ID of UE-to-UE Relay-2, …seq_num m User Info ID of UE-UE Relay-m }, Direct discovery set.

4-6. End UE (UE-2) responds to the UE-to-UE Relay-2 with a UE-to-UE Relay Discovery Response message. The UE-to-UE Relay Discovery Response message contains RSC, Multi-hop routing info {seq_num1 User Info ID of UE-to-UE Relay-1, seq_num2  User Info ID of UE-to-UE Relay-2}, Direct discovery set. The UE-to-UE Relay Discovery Response message is sent to End UE (UE-1) via UE-to-UE Relay-2 and UE-to-UE Relay-1.

If m UE-to UE Relays have been involved in the multi-hop UE-to-UE Relay Model-B Discovery, the UE-to-UE Relay Discovery Response message contains RSC, Multi-hop routing info {seq_num1 User Info ID of UE-to-UE Relay-1, seq_num2 User Info ID of UE-to-UE Relay-2, …seq_num m User Info ID of UE-UE Relay-m }, Direct discovery set.

6.X.3	Impacts on services, entities and interfaces

UE:

-	New functionality related to UE-to-UE Relay discovery.

* * * End of change * * *