3GPP TSG-WG SA2 Meeting #161	S2-2402538

Athens, Greece, February 26 - March 1, 2024

Title:	[Draft] LS on Event ID for Timing Synchronization Status and Reporting

Response to:

Release:	Release 18

Work Item:	TRS_URLLC

Source:	SA2

To:	RAN2

Cc:	-

Contact Person:

Name:	Sang-Jun Moon

E-mail Address:	moonst AT samsung DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.501 CR 5335

1. Overall Description:

SA2 would like to inform RAN2 of that SA2 discussed a missing point on Event ID for Timing Synchronization Status like the following:

Event ID is broadcasted only if NG-RAN supports and uses TRS feature. Hence, there are two cases that Event ID is not broadcasted by NG-RAN:

(1) If an NG-RAN node guesses there are no UEs which require the clock quality information, the NG-RAN node stops broadcasting Event ID in SIB9.

(2) If an NG-RAN node does not support the TSS feature, then Event ID is absent in SIB9.

A TSS-enabled UE having stored Event ID may experience the absence of Event ID (1) in the same cell or (2) after the cell reselection. In this case, the UE shall consider that the clock quality information has changed and determine whether to transit to the RRC_CONNECTED state to retrieve it or check the availability. This part is completely missing in TS 23.501, so should be clarified. In other words, as Event ID is proposed for indicating Synch Status changes, the change from getting Event ID into not getting Event ID needs to work as the same as getting a changed Event ID.

SA2 agreed a CR to clarify the UE behavior upon receiving SIB9 without Event ID after receiving SIB9 with Event ID.

2. Actions:

To RAN2:

ACTION: 	SA2 kindly asks RAN2 to take the above into account and to apply the related changes to RAN2 specifications.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #162		15-19 April 2024	Changsha, China

TSG-SA2 Meeting #163		27-31 May 2024	Korea (TBD)