3GPP TSG-WG SA2 Meeting #160E e-meeting	S2-2402542

Elbonia, January 22 – 29, 2024

Source:	Futurewei

Title:	New solution for KI#2: Procedure for VFL initiation and preparation

Document for:	Approval

Agenda Item:	19.15

Work Item / Release:	FS_AIML_CN / Rel-19

Abstract of the contribution: Proposal for a new solution for KI#2, Procedure for VFL initiation and preparation

# Discussion

Vertical Federated Learning (VFL) allows multiple FL participants (e.g., NWDAF containing MTLF) that have access to different data attributes (e.g. features and labels) of the same data samples (e.g. user) to jointly train an ML model.

This paper proposes a procedure for VFL initiation and preparation which may include steps for identifying whether VFL is needed, VFL active participant discovery, VFL passive participant discovery and feature/sample alignment. The proposed procedure can be applicable to VFL preparation for cross domain VFL (between 5GC and AF) and/or VFL within 5GC.

# Proposal

It is proposed to capture below solution in TR 23.700-84.

Start of Changes

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues and Use Cases

|  | Key Issues |  | Use cases (optional) |  |
| --- | --- | --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> | <use case #x> | <use case #y> |
| 6.x |  | x |  |  |
| #2 |  |  |  |  |

Next Change

## 6.X	Solution #X: Procedure for VFL initiation and preparation

### 6.X.1	Description

Procedure for VFL initiation and preparation may include steps of identifying whether VFL is needed, VFL active participant discovery, VFL passive participant(s) discovery and feature/sample alignment.

### 6.X.2	Procedures

#### 6.x.2.1 Procedure for VFL initiation and preparation

- ML model training consumer, which may be located in 5GC or AF domain, may send a ML model training request to NWDAF containing MTLF.

- NWDAF containing MTLF may determine VFL is needed if it is indicated in the ML training request from ML model training consumer,  corresponding analytics ID requires cross domain ML training between 5GC and AF, or ML model training corresponds to a use case for VFL within 5GC.

- NWDAF containing MTLF may discover and select VFL active participant from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytic ID of the required ML model, VFL capability Type (i.e. VFL active participant), Service Area, Data availability by the active VFL participant (active VFL participant should own the data labels). For cross domain VFL between 5FC and AF, VFL active participant may be located in AF domain.

Editor’s Note: Details for VFL active participant registration procedure are FFS.

- NWDAF containing MTLF may send a VFL preparation request to VFL active participant.

- VFL active participant may discover and select VFL passive participant(s) from NRF by invoking the Nnrf_NFDiscovery_Request service operation. For cross domain VFL between 5FC and AF, VFL passive participant(s) may be located in AF domain.

Editor’s Note: Details for VFL passive participant(s) registration procedure are FFS.

- Sample alignment and feature selection may be performed between VFL active participant and VFL passive participant(s). Sample alignment and feature selection may depend on data samples available at VFL active participant and also feature dimension of VFL passive participant(s).

Editor’s Note: Details for sample alignment and feature selection are FFS.

### 6.X.3	Impacts on services, entities and interfaces

NWDAF containing MTLF:

-	Determine VFL is needed;

-	Support VFL active/passive participant capability;

Editor’s Note: Details for impacts on other services, entities (including NRF and NEF) and interfaces are FFS.

End of Changes