SA WG2 Meeting #S2-161	S2-2402578

26 February - 1 March, 2024, Athens, Greece	(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | 0125 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the SL Positioning Server UE and LMF description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2024-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some descriptions in the clause 5.3.1 on the SL Positioning Server UE are not correct. The SL Positioning Server UE does not perform SL reference UE selection. LMF does not decide target UE, SL reference UE to execute the result calculation |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing the incorrect descriptionon the SL Positioning Server UE and LMF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | incorrect descriptionon the SL Positioning Server UE and LMF functionality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.3.1	General

Any UE supporting Ranging/SL Positioning, e.g. Target UE, SL Reference UE, Sidelink Positioning Server UE, Located UE, shall have a Ranging/SL Positioning layer, which is above the AS layer and handles service requests received from application layer to control the Ranging/SL Positioning operation. The Ranging/SL Positioning layer provides the support of authorization and provisioning as described in clause 5.1, the UE discovery and selection as described in clause 5.2, the Ranging/SL Positioning Protocol (RSPP) defined in TS 38.355 [12], and the protocols between UE and LMF for Ranging/SL Positioning. Ranging/SL Positioning layer supports the Ranging/SL Positioning service sessions for one time or periodic ranging, ranging for distance or direction measurement or both. The Ranging/SL Positioning layer provides the ranging parameters to AS layer, including one time or periodic ranging, ranging for distance or direction measurement or both.

NOTE 1:	The Ranging/SL Positioning Protocol (RSPP) mention above corresponds to Sidelink Positioning Protocol (SLPP) in RAN.

Editor's note:	The protocol used between UE and LMF will be decided by RAN and could be an extension of LPP, a new protocol or both.

The RSPP is exchanged over SR5 over PC5-U reference point between UEs (i.e. Target UE, SL Reference UE, Located UE and SL Positioning Server UE) to manage the Ranging/SL Positioning service sessions between UEs or among a group of UEs. Multiple UEs may be involved in a single Ranging/Sidelink Positioning session. When the Ranging/SL Positioning service session is among a group of UEs, the RSPP may provide group related identifier information for the PC5 transport layer handling. The transport of the RSPP over PC5-U is specified in clause 5.3.2. The RSPP supports the following functions:

-	exchange the Ranging/Sidelink Positioning capability;

-	exchange the Ranging/Sidelink Positioning assistant data;

-	exchange Ranging/Sidelink positioning measurement data/result.

NOTE 2:	Group management is out-of-scope of this specification.

NOTE 3:		RSPP signalling negotiation determines whether measurement results or Ranging/Sidelink Positioning results are exchanged over the SR5.

NOTE 4:	The Ranging/Sidelink positioning supports the scheduled location time feature.

A LMF shall be involved when at least one of the Target UE and the SL Reference UE are in the network coverage and the serving network is capable to support ranging functionalities. The details of Network based Ranging/SL Positioning operation, and Network assisted SL Positioning are described in detail in clause 5.5. When Network assisted Ranging/SL Positioning is used, the use of RSPP is necessary for Ranging/SL Positioning control.

A SL Positioning Server UE can be discovered and selected for result calculation, method determination, assistant data distribution  in case of out-of-coverage or for UE-only Operation if the serving network does not support Ranging/SL Positioning. If the LMF capable for Ranging/SL Positioning is reachable by Target UE and/or SL Reference UE, the LMF can still decide that a SL Positioning Server UE executes the result calculation. A SL Positioning Server UE can be co-located with a Target UE or SL Reference UE.

Editor's note:	Functionalities of the SL Positioning Server UE will be determined by RAN WGs.

END OF CHANGES