3GPP SA WG2 Meeting #S2-161	S2-2402664

26 February - 01 March, 2024, Athens, Greece

Source:	MediaTek Inc.

Title:	Update to KI#1.1: Subscription aspects to support DualSteer

Document for:	Agreement

Agenda Item:	19.13

Work Item / Release:	FS_MASSS/Rel-19

Abstract of the contribution: This paper proposes to update to KI#1.1

Discussion

In this paper, it proposes to update to KI#1.1: Subscription aspects to support DualSteer.

In the key issue, one bullet as captured below needs to be updated:

"Whether it is needed and what enhancements are required in subscription data to support Registration and Mobility Management and management of PDU sessions for DualSteer."

Subscription data should not have impact on the management of PDU Sessions.

Proposal: removing "management of PDU sessions"

Proposal

It proposes to update KI#1.1 to TR 23.700-54

*** Start of changes ***

5.1	Key Issues for DualSteer

5.1.1	Key Issue #1.1: Subscription aspects to support DualSteer

5.1.1.1	Description

This key issue will study the enhancements of subscription aspects to support DualSteer including the following:

-	Whether and how the 5G System identifies and associates the two subscriptions/SUPIs for DualSteer.

-	Whether it is needed and what enhancements are required in the policy subscription data to support generation of rules/policies for DualSteer.

-	Whether it is needed and what enhancements are required in subscription data to support Registration and Mobility Management for DualSteer.

*** End of changes ***