SA WG2 Meeting #161	S2-2402685

February 26 – March 1, 2024, Athens, Greece

# Title:	New solution KI#3: UE and PDU session admission to a network slice based on energy criteria

# Source:	Nokia, Nokia Shanghai Bell

# Document for:	Approval

Agenda Item:	FS_EnergySys/19.4

Abstract:

# 1. Introduction

The NSAC feature today can be used to restrict the number of UEs or PDU sessions which are admitted to the system based on a contracted # of UEs or number of PDU sessions that are allowed to be simultaneously respectively registered and established in the network slice. the NSACF is the NF that decides whether a certain UE or PDU session is admitted in the network slice.

Assuming that the energy consumption in a network slice can be linked to the admission of UEs and sessions, this solution proposes that a PLMN can configure NSAC based on energy criterion certain network slices for which the network slice customer has agreed to admit UEs or sessions only up to a certain energy threshold.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-66.

# Proposed text

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | X |
| 1 | x |  |  |  |
| 2 | x |  |  |  |
| 3 | x |  | x |  |
| 4 | x |  |  |  |
| 5 | x |  | x |  |
| 6 | x |  |  |  |
| 7 |  | x |  |  |
| 8 |  | x |  |  |
| 9 |  | x |  |  |
| 10 |  | x |  |  |
| 11 |  | x | x |  |
| 12 |  |  | x |  |
| 13 |  |  | x |  |
| 14 |  |  | x |  |
| 15 |  | x | x |  |
| X |  | x | x |  |

# Proposed text - ALL NEW TEXT

## 6.x	Solution #x: NSAC based on energy criterion.

### 6.x.1	Key Issue mapping

This is a solution for KI#3 and KI#2

### 6.x.2	Functional Description

This solution assumes a network slice customer agrees to authorize up to a certain level of energy to be consumed by its UEs in its network slice. After that it accepts that new UEs or PDU sessions may not be admitted to the 5GS. The NSACF receives the current (or estimated) consumed energy in the network slice from NWDAF analytics or from OAM.

The customer may also subscribe to exposure of the analytic of energy consumed in the network slice (based on solutions to KI#1) to top up credit to avoid its UEs/sessions not being served.

### 6.x.3	Procedures

#### 6.x.3.1	NSAC based on energy criterion

Figure 6.x.3.1-1: NSAC based on energy criterion

1.	NSACF subscribes to network slice energy consumption analytics from with the NWDAF of a S-NSSAI

2.	NWDAF provides analytics to the NSACF for the S-NSSAI of step 1

3.	UE attempts to register with the S-NSSAI from step 1, or to establish a PDU session for the Same S-NSSAI

4.	The AMF or, as applicable, the SMF detects the S-NSSAI is subject to NSAC

5.	The NSAC step is executed by updating the NSACF to increase the number of UEs or sessions as applicable.

6.	The results of the step 5 are received at the AMF or SMF as applicable and the S-NSSAI is included (or not) in the allowed NSSAI and, as applicable, the PDU session is established (or not) depending on outcome of step 5.

### 6.x.4	Impacts on existing services, entities and interfaces

NSACF: subscribe to energy consumption analytics for the network slice.

NWDAF: provide analytics for energy consumption in a network slice.

# End of proposed text