SA WG2 Meeting #161	S2-2402733

Athens, GREECE, February 26 – March 01, 2024

Source: 	vivo

Title: 	KI#4: New Solution – NAS signaling storm mitigation and prevention based on NWDAF detection and prediction

Document for: 	Approval

Agenda Item: 	19.15

Work Item / Release:	FS_AIML_CN / Rel-19

Abstract of the contribution: This contribution proposes a solution for KI#4 to support NAS signalling storm mitigation and prevention based on NWDAF detection and prediction.

1	Discussion

As depicted in use case #3, UE may be active at the same time so that a large amount of NAS signalling may be transmitted into the network, which may cause signalling storm in that area during that period of time. Thus, this contribution proposes a solution to detect, predict, mitigate and prevent the NAS signalling storm.

2. Proposal

It is proposed to add the following contents to TR 23.700-84.

Start of Change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[6]	3GPP TR 38.843: "Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface".

[yy]	3GPP TS 38.413: “NG Application Protocol (NGAP)”.

Next Change

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues and Use Cases

|  | Key Issues |  |  |  | Use cases (optional) |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> | <Key Issue #3> | <Key Issue #4> | <use case #4> | <use case #y> |
| 6.X |  |  |  | X | X |  |

Next Change (all new text)

6.X	Solution #X: NAS signalling storm mitigation and prevention based on NWDAF detection and prediction

6.X.1	Description

This solution is for Key Issue#4: NWDAF enhancements to support network abnormal behaviours (i.e. Signalling storm) mitigation and prevention.

As depicted in use case #3, UE may be active at the same time so that a large amount of NAS signalling may be transmitted into the network, which may cause signalling storm in that area during that period of time.

6.X.1.1	Input Data

To detect and predict NAS signalling storms, 3 distinct types of input data are required, namely NAS signalling feature data, NF context data, and UE behaviour data.

The NAS signalling feature data can indicate the presence of a NAS signalling storm, enabling the NWDAF to identify such storms. Additionally, by training a ML model to recognize typical NAS signalling patterns, the NWDAF can predict the occurrence of a NAS signalling storm when there is a trend of abnormal NAS signalling.

The NAS signalling storm feature is characterized by a significant increase in NAS signalling directed towards the AMF over a period of time, leading to abnormal consumption of AMF resources (e.g. over 85% CPU or memory usage). This may overwhelm the AMF's capacity to handle the NAS signalling, resulting in a high volume of NAS rejection or unhandled NAS requests. So, the following input data is needed in table 6.X.1.1-1.

Table 6.X.1.1-1: Input data: NAS signalling feature data

| Information | Source | Description |
| --- | --- | --- |
| S-NSSAI | AMF | Identifies the Network Slice for which analytics information is provided. |
| DNN | AMF | Identifies the data network name (e.g. internet) for which analytics information is provided. |
| SUPI | AMF | UE ID for the UE that sends the signalling. |
| NAS unhandled request message amount | AMF | Amount of NAS unhandled request message, e.g. Registration Request, Service Request, PDU session establishment Request. |
| NAS reject message amount | AMF | Amount of NAS Reject message, e.g. Registration Reject, Service Reject, PDU session establishment Reject. |
| NAS message amount | AMF | Amount of all the NAS message. |
| Start/end time | AMF | Start and end time of detection. |

The NF context data contains information about the current UE context and NF’s load, enabling the NWDAF to determine the cause of a signalling storm, such as massive UE access.

Table 6.X.1.1-2: Input data: NF context data

| Information | Source | Description |
| --- | --- | --- |
| Number of UEs | AMF | Total number of UEs that is served by the AMF |
| NF load | NRF | The load of specific NF instance(s) in their NF profile |
| NF resource usage | OAM | The usage of assigned virtual resources currently in use for specific NF instance(s) (mean usage of virtual CPU, memory, disk). |

The UE behaviour data is the same as the input data for abnormal behaviour related network data analytics defined in clause 6.7.5.2 of TS 23.288 [5]. This data is used to identify abnormal UE behaviour and assist the NWDAF in determining the cause of a signalling storm, such as a sudden increase in UE access.

6.X.1.2	Output Analytics

The output analytics of signalling storm detection of NWDAF is defined in Table 6.X.1.2-1.

Table 6.X.1.2-1: signalling storm detection

| Information | Description |
| --- | --- |
| Report (1..max) | List of observed signalling storm |
| > NF ID | The target of signalling storm detected by NWDAF |
| > Reason | The reason of signalling storm (e.g. massive UE access, etc.) |
| > SUPI list (1..SUPImax) | SUPI(s) of the UE(s) which cause the signalling storm |

The output analytics of signalling storm prediction of NWDAF is defined in Table 6.X.1.2-2.

Table 6.X.1.2-2: signalling storm predictions

| Information | Description |
| --- | --- |
| Report (1..max) | List of observed signalling storm |
| > NF ID | The target of signalling storm detected by NWDAF |
| > Reason | The reason of signalling storm (e.g. massive UE access, etc.) |
| > SUPI list (1..SUPImax) | SUPI(s) of the UE(s) which cause the signalling storm |
| > Confidence | Confidence of this prediction |

6.X.2	Procedures

For this new analytics, current procedures to support network data analytics as defined in clause 6 of TS 23.288 [5] can be reused. The NF consumer may be UDM or AMF.

Besides, the following existing mechanism can be reused to mitigate and prevent the signalling storm.

Table 6.X.2-1: Mechanisms to mitigate and prevent the signalling storm

| Reason | NF consumer | Actions of NFs |
| --- | --- | --- |
| Massive UE access | UDM、AMF | UDM/AMF updates UE’s S-NSSAI using update of UE network slice configuration procedure as defined in clause 5.14.4.2 of TS 23.501 [2] to split the UEs who cause the signalling storm into a specific slice. AMF sends OVERLOAD START as defined in clause 8.7.7 of TS 38.413 [yy] to start overload control with the specific S-NSSAI. |

6.X.3	Impacts on services, entities and interfaces

NWDAF:

-	New service to provide NWDAF capability for signalling storm detection and prediction.

-	Collects training data for signalling storm detection and prediction.

-	Trains ML model for signalling storm detection.

AMF/UDM:

-	Reuse existing mechanisms for the specific UE control once the report is received.

End of Changes