SA WG2 Meeting #161	S2-2402734

February 26 – March 1, 2024, Athens, Greece

# Title:	KI#1: new solution on modification of UP b/h PDU sessions

# Source:	Nokia, Nokia Shanghai Bell

# Document for:	Approval

Agenda Item:	FS_VMR_Ph2/19.6

Abstract:	this paper provides a solution for the adaptation of b/h PDU session to serve the QoS Flows of UEs served by MBSR.

1. Introduction

This paper addresses the option to efficiently support QoS for the UEs served by a MWAB by modification of the PDU session used to carry the N3 traffic for all the MWAB-gNB to AMFs NG connections.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-06.

# Proposed text

## 6.0	Mapping of solutions to key issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of solutions to key issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 |  |  |  |
| x | x |  |  |  |

# Proposed text - ALL NEW TEXT

## 6.X	Solution #x: Modification of the PDU session for b/h to efficiently support QoS for UEs served by a MWAB-gNB.

### 6.X.1	General

This is a Solution addressing KI#1: Architectural enhancements for the support of a MWAB, focused on the adaptation of b/h PDU session to the QoS Flows of UEs served by MBSR.

### 6.X.2	Functional description

Figure 6.x.2-1: mapping of QoS flows at MWAB and at B/H UPF

Figure 6.x.2-1 shows that the UE traffic for a specific QoS flow is carried in GTP-U tunnels for UL and DL that are associated with DSCP values and IP@ of UE UPF (for UL) and IP address of MWAB-gNB (for DL). The DSCP value can be identified at the MWAB-gNB based on policy related to information the MWAB-gNB has in the SM context for the PDU session. The MWAB gNB then requests the MWAB-UE to perform a modification of the b/h PDU session  with a specific 5QI an QoS parameters that are suitable to handle the SDF of the DL traffic identified by the traffic associated with the IP@ of the MWAB-gNB and the DSCP value. This provides the b/h UPF with the packet classification rules it needs to map the DL traffic from the UE UPF to the right QoS flow on the b/h PDU session.

### 6.X.3	Procedures

### 6.X.3.1	Handling of a UE PDU session establishment or modification

Figure 6.x.3.1-1: handling of a UE PDU session establishment or modification

1. 	A PDU session is established or modified for a UE, and this causes the MWAB-gNB to receive from SMF a new SM context for a PDU session including at least a QoS flow.

2.	For each QoS flow the MWAB-gNB determines the required 5QI and QoS parameters in the b/h PDU session and TNL to be used to signal to the b/h SMF the SDF for the QoS rules related to this SDF for DL.  For UL the QoS rules are also determined by MWAB and the TNL information determined at the MWAB-gNB is used to classify in UL.

3. 	The MWAB-UE modifies the b/h PDU session as instructed.

4.	The UPF of the b/h PDU session is now ready to correctly process DL traffic from the UE UPF.

5.	The MWAB-UE acks the correct modification of the b/h PDU session.

6. 	The MWAB-gNB can complete the establishment of the PDU session.

7.	The data for the UE PDU session can be sent/received with the right QoS.

### 6.X.4	Impacts on services, entities, and interfaces

MWAB: determine how to modify the b/h PDU session in response to the establishment or modification of PDU sessions for the UEs it serves.

OAM: configuration of MWAB-gNB with the correct mapping rules between the UE level an b/h level PDU session parameter.

# End of proposed text