3GPP TSG-WG SA2 Meeting #161	S2-2402749

February 26 – March 1, 2024, Athens, Greece	(Revision of S2-24xxx)

Source:	Samsung

Title:	KI#1: New solution for mobility support between CAG and CSG cells

Document for:	Approval

Agenda Item:	19.12

Work Item / Release:	FS_5G_Femto /Rel-19

Abstract of the contribution: This contribution proposes a new solution for KI#1.

- Discussion

In clause 5.1 of TR 23.700-45, the Key Issue #1 for support of UE mobility between CAG cell of 5G Femto and CSG cell is agreed as one of key issues for Rel-19 FS_5G_Femto study:

This key issue investigates any needed enhancements to support the UE moving between CAG cell of 5G Femto and CSG cell.

The KI should cover:

-  The mobility scenarios to be studied for the UE move (e.g. CSG cell to CAG cell of 5G Femto).

-  Whether and how control signalling procedures are enhanced to support the mobility scenarios.

NOTE 1:	This Key Issue assumes that the existing CAG concept defined for PNI-NPN is re-used for access control when a UE wants to access a 5G Femto cell without impacts to PNI-NPN.

NOTE 2:	This Key Issue expects that solutions avoid impacts on EPC (e.g., MME), E-UTRAN and NG-RAN.

Editor's note: It is FFS if idle mode mobility scenario needs to be considered.

This contribution proposes a new solution for the KI #1.

- Proposal

It is proposed to adopt the following changes into TR 23.700-45.

*** Start of the change ***

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues and Use Cases

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> |
| #1 |  |  |
| #X | x |  |

*** Next change (all new text) ***

6.X	Solution #X: Solution for mobility between CAG and CSG cells

6.X.1	Description

The solution aims to address the issues described in KI#1 to support mobility between a CAG cell of 5G Femto and a CSG cell of HeNB/EPC.

The solutions assumes the followings:

-  The existing CAG concept defined for PNI-NPN is re-used for access control when a UE wants to access a 5G Femto cell without impacts to PNI-NPN.

-  Impacts on EPC, E-UTRAN and NG-RAN should be avoided.

-  UE subscription information includes allowed CAG and CSG list for the UE

-  Allowed CAG information may include a mapped CSG ID for a CAG ID to support mobility between CAG and CAG cells

A UE may be provided with allowed CAG and CSG list via AMF during registration with 5GC or via a configuration server. If the UE finds a CAG cell of a 5G Femto while communicating through a CSG cell attached to the EPC, it may sends a mapped CSG ID for the corresponding CAG cell of the 5G Femto to the source HeNB. The mapped CSG ID is forwarded to the MME and AMF to check allowed CSG and CAG list based on the UE subscription data from the UDM.

6.X.2	Procedures

Figure 6.x.2-1 Procedures for UE mobility from CSG cell to CAG cell

0.  It is assumed that UE has been provisioned with the allowed CAG and CSG list for access to CAG and CSG cells. During the registration with 5GC, UE may receive the list from the AMF. The UE is currently attached to a CSG cell and EPC and set up a PDN connection(s).

- The UE discovers an available CAG cell of 5G Femto and reports to the source eNB. Based on the control of the source eNB, the UE may sends the measurement report to the source eNB which may include a mapped CSG ID corresponding to a CAG cell of a 5G Femto.

Editor’s Note:	The exact procedure for cell selection and the measurement report will be defined by RAN WGs.

- The source eNB may send Handover Required message to the MME

- The MME checks the subscription data to find if the mapped CSG ID received from the UE is included in the UE’s allowed CSG list.

- Based on the allowed CSG list included in the subscription data, the MME decides if the handover request to the target CAG cell of the 5G Femto can be accepted

4a. If the received CSG ID is not matched, the MME rejects the handover request.

4b. If the received CSG ID is included in the allowed CSG list of the UE, the MME forwards the handover request to the AMF. The AMF may check if the handover request from the MME can be acceptable based on the UE subscription data, operator policies, etc. Otherwise, the AMF may reject the handover request from the MME.

- The AMF selects the SMF+PGW-C and requests to create Session Management context for UE’s PDU session(s).

- The AMF forwards the handover request to the target 5G Femto gNB.

- The gNB sends acknowledges message to the AMF.

- Rest of the interworking handover procedures will proceed.

The same procedure may be applied for the case that the UE moves from 5GS to the EPS, i.e., from CAG cells to CSG cells.

6.X.3	Impacts on services, entities and interfaces

UE:

- Should support the receive and send the mapped CSG ID

AMF:

- Should check CAG list which maps from the received CSG ID.

*** End of the change ***