3GPP TSG-WG SA2 Meeting #161	S2-2402837

26 February - 1 March, 2024, Athens, Greece	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	KI#2, New Solution for enabling provisioning of CAG information in non-roaming scenario

Document for:	Approval

Agenda Item:	19.12

Work Item / Release:	FS_5G_Femto / Rel-19

Abstract: A new solution is proposed to enable provisioning of CAG information in non-roaming scenario.

1. Introduction/Discussion

KI#2 aims to address the following aspects:

-	How to enable the CAG owner or an authorized administrator to provision/update CAG information to the network for 5G Femto access control. The provisioning/updating of CAG info to the network that 5G Femto serves and the network that the UE has subscription will be considered.

This solution considers the non-roaming scenario where the network that 5G Femto serves and the network that the UE (of the visitor) has subscription are same. In this scenario, the AF (authorized administrator) may trigger the External Parameter Provisioning Procedure to update the CAG information stored in the UDM.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-45.

# * * * * First change * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> |
| #1 |  |  |
| #2 |  |  |
| #X |  | X |

# * * * * Second change * * * *

6.X	Solution #X: Enable provisioning of CAG information in non-roaming scenario

6.X.1	Description

This solution is aimed at KI#2. CAG information is part of subscription data which is stored in the UDM. In order to create the CAG information for the UE to access a CAG cell of 5G NR Femto, CAG owner should provide the CAG information related to the UE to the UDM. UDM can create and store the CAG information in the subscription data of the UE. This solution is assumed to be applicable to the non-roaming scenario.

6.X.2	Procedures

Figure 6.X.2-1: CAG information provisioning

1.	The UE of the CAG owner establishes a connection (e.g. via a PDU Session) with the server used by the authorized administrator. The UE of CAG owner can provide the CAG information related to the UE(s) of visitor which potentially may access to the CAG cell. The CAG information of UE(s) includes the list of UE identifier (e.g. GPSI) and the corresponding allowed CAG list. The allowed CAG list indicates the CAG ID(s) of Femto cell(s) which the UE is allowed to access to. Optionally, the UE of the CAG owner can also provide validity conditions (e.g. time period) associated with the allowed CAG list.

2.	The AF corresponding to the authorized administrator sends Nnef_ParameterProvision_Create or Nnef_ParameterProvision_Update Request to NEF to provides the CAG information to be created or updated.

3.	If the AF is authorised by the NEF to provision the parameters, the NEF requests to create, update and store the provisioned parameters as part of the subscriber data via Nudm_ParameterProvision_Create or Nudm_ParameterProvision_Update Request message.

4.	UDM may read from UDR, by means of Nudr_DM_Query, corresponding subscription information in order to validate required data updates and authorize the changes for the allowed CAG list for the list of UE for the corresponding AF.

5.	If the AF is authorized by the UDM to provision the parameters for this subscriber, the UDM resolves the GPSI to SUPI, create or update the CAG information based on the provisioned data and requests to create or update the CAG information as part of the subscriber data via Nudr_DM_Create/Update Request message.

6.	UDM responds the request with Nudm_ParameterProvision_Create/Update Response. If the procedure failed, the cause value indicates the reason.

7.	NEF responds the request with Nnef_ParameterProvision_Create/Update Response. If the procedure failed, the cause value indicates the reason.

8.	UDM triggers UE Configuration Update Procedure as specified in clause 4.2.4.2 of TS 23.502 [3] to send the Allowed CAG list to the UE of the visitor.

After the UE of the visitor obtains the Allowed CAG list, it follows the existing mechanism to perform cell (re-)selection to access to the corresponding CAG cell for Femto.

6.X.3	Impacts on services, entities and interfaces

UE (CAG owner):

- Ability to provide the identifier of visitor UE (e.g. GPSI), allowed CAG list and optional validity conditions to the authorized administrator.

AF (Authorized administrator):

- Ability to perform the External Parameter Provisioning procedure to provide the UE identifier, allowed CAG list and optional validity conditions to the UDM.

UDM:

- Ability to create and store the CAG information based on the provisioned data from AF.

# * * * * End of changes * * * *