3GPP TSG-SA2 Meeting #161	S2-2402867

# Athens, Greece, February 26 - March 1, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4721 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network slice replacement and NSAC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | there is a need to support NSAC for the case of network slice replacement as discussed in S2-2402863. this CR allows flexible policy to be applied. |  |  |  |  |  |  |  |  |
| Summary of change: |  | in case of Network Slice Replacement enable the AMF to trigger the update of the number of UEs registered to every NSACF that handles the network slices involved in the procedure. in case of Network Slice Replacement enable the SMF to trigger the update of the number of PDU sessions to every NSACF that handles the network slices involved in the procedure. Enable the Nnsacf_NSAC_NumOfUEsUpdate and/or Nnsacf_NSAC_NumOfPDUsUpdate to indicate both the replaced and alternative S-NSSAI to NSACF(s) during Network Slice Replacement. |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The number of PDU sessions is not accurately updatedaccording to operator preference. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.11.2, 4.2.11.4, 5.2.21.2.2, 5.2.21.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

## FIRST CHANGE

#### 4.2.11.2	Number of UEs per network slice availability check and update procedure

This clause applies to Non-Hierarchical and centralized NSAC architectures. The difference between the two architectures for the various steps, where applicable, is described at the end of the clause.

The number of UEs per network slice availability check and update procedure is to update (i.e. increase or decrease) the number of UEs registered with an S-NSSAI which is subject to NSAC. The AMF is configured with the information indicating which network slice is subject to NSAC.

Figure 4.2.11.2-1: Number of UEs per network slice availability check and update procedure

1.	If the AMF is not aware of which NSACF to communicate, the AMF performs NSACF discovery as described in clause 6.3.22 of TS 23.501 [2] and in clause 5.2.7.3.2. The AMF triggers the Number of UEs per network slice availability check and update procedure to update the number of UEs registered with a network slice when a network slice subject to NSAC is included in the Allowed NSSAI or Partially Allowed NSSAI (i.e. the AMF requests to register the UE with the S-NSSAI) or removed from the Allowed NSSAI or Partially Allowed NSSAI (i.e. the AMF requests to de-register the UE from the S-NSSAI) for a UE. The trigger event at the AMF also includes the change of Allowed NSSAI or Partially Allowed NSSAI in the case of inter-AMF mobility or in the case of Network Slice Replacement (see clause 5.15..19 of TS 23.501[2]),. In case of Network Slice Replacement, the AMF of the alternative network slice triggers the Number of UEs per network slice availability check and update procedure to update the number of UEs registered with the network slice towards the NSACF(s) that handle the network slice(s) involved in the replacement procedure if NSAC is applicable to any of the network slices. The procedure is triggered in the following cases:

-	At UE Registration procedure, according to clause 4.2.2.2.2 (including Registration types of Initial Registration or Mobility Registration Update in inter-AMF mobility in CM-CONNECTED or CM-IDLE state):

-	before the Registration Accept in step 21 if the EAC mode is active; or

-	after the Registration Accept message if the EAC mode is not active;

-	At UE Deregistration procedure, as per clause 4.2.2.3, after the Deregistration procedure is completed;

-	At UE Configuration Update procedure (which may result from NSSAA procedure or subscribed S-NSSAI change or Network Slice Replacement):

-	before the UE Configuration Update message if the EAC mode is active and the update flag is to increase; or

-	after the UE Configuration Update message if the EAC mode is active and the update flag is to decrease; or

-	after the UE Configuration Update message if the EAC mode is not active.

NOTE 1:	Depending on the deployment, there may be different NSACF for different S-NSSAI subject to NSAC and hence, during the registration, AMF triggers the Number of UEs per network slice availability check and update procedure to multiple NSACFs.

2.	The AMF sends Nnsacf_NSAC_NumOfUEsUpdate_Request message to the NSACF. The AMF includes in the message the UE ID, Access Type to which the Allowed NSSAI or Partially Allowed NSSAI is applied, the S-NSSAI(s), the NF ID and the update flag which indicates whether the number of UEs registered with the S-NSSAI(s) is to be increased when the UE has gained registration to network slice(s) subject to NSAC or the number of UEs registered with the S-NSSAI(s) is to be decreased when the UE has deregistered from S-NSSAI(s) or could not renew its registration to an S-NSSAI subject to NSAC. The AMF includes both the replaced and Alternative S-NSSAIs for Network Slice Replacement so that NSACF(s) can use it as specified in 3GPP TS 23.501 [2].

If this is the first time to perform NSAC procedure for the S-NSSAI towards the NSACF, the AMF includes notification endpoint for EAC Notification to implicitly subscribe the EAC notification for the S-NSSAI from the NSACF.

3.	The NSACF determines whether the Access Type provided by the AMF is configured for the NSAC based on its configuration. If the Access Type is not configured for the NSAC, the NSACF always accepts the request from the AMF without increasing or decreasing the number of UEs. If the Access Type is configured for the NSAC, the NSACF updates the current number of UEs registered for the S-NSSAI, i.e. increases or decrease the number of UEs registered per network slice based on the information provided by the AMF in the update flag parameter.

If the update flag parameter from the AMF indicates increase, the following applies:

-	If the UE ID is already in the list of UEs registered with the network slice, the current number of UEs is not increased as the UE has already been counted as registered with the network slice. The NSACF creates a new entry associated with this new update and shall also maintain the old entry associated with previous update. The multiple entries for the same UE ID in the NSACF are differentiated based on the NF ID of the NF sending the update request. The NSACF removes the entry associated with the NF ID upon reception of a request having update flag indicating decrease.

NOTE 2:	The use case of having two or more entries in the NSACF for the same UE can happen during (a) inter-AMF mobility when the new AMF request update to the NSACF before the old AMF sends request to deregister the UE; or (b) PDN connections establishment in the EPC when multiple SMF +PGW-Cs (i.e. used for different PDN Connections associated with the same S-NSSAI) send update requests for maximum number of UEs to the NSACF.

NOTE 3:	To handle AMF graceful removal, the NSACF can subscribe for unavailability notifications with the AMF (directly or via NRF) as described in clause 5.21.2.2 and act accordingly, e.g. update the NF ID with the target AMF ID.

-	If the UE ID is not in the list of UE IDs registered with the network slice and the maximum number of UEs registered with the network slice has not been reached yet, the NSACF adds the UE ID in the list of UEs registered with the network slice as a new entry associated with this new update and increases the current number of the UEs registered with the network slice. If the UE ID is not in the list of UEs registered with that S-NSSAI and the maximum number of UEs for that S-NSSAI has already been reached, then the NSACF returns a result parameter indicating that the maximum number of UEs registered with the network slice has been reached.

If the update flag parameter from the AMF indicates decrease and if there is only one entry associated with the UE ID, the NSACF removes the UE ID from the list of UEs registered with the network slice for each of the S-NSSAI(s) indicated in the request from the AMF and also the NSACF decreases the number of UEs per network slice that is maintained by the NSACF for each of these network slices. If there are multiple entries associated with the UE ID, the NSACF removes the entry associated with the NF ID but the UE ID is kept in the list of UEs registered with the S-NSSAI.

If the update parameter from the AMF indicates both the replaced and alternative S-NSSAIs due to Network Slice Replacement, the NSACF behaves as specified in 3GPP TS 23.501 [2].

The NSACF takes access type into account for increasing and decreasing the number of UEs per network slice as described in clause 5.15.11.1 of TS 23.501 [2].

The NSACF stores the notification endpoint for EAC Notification associated with the S-NSSAI if it is received from the AMF. The NSACF can use this AMF notification endpoint to update the EAC mode as described in clause 4.2.11.3.

NOTE 4:	This enables the NSACF to maintain up-to-date information about the AMFs serving the S-NSSAIs.

4.	The NSACF returns the Nnsacf_NSAC_NumOfUEsUpdate_Response message including Result indication per S-NSSAI. The Result indication includes either 'maximum number of UEs registered with the network slice reached' or 'maximum number of UEs registered with the network slice not reached'.

At UE Registration procedure, if only some of the S-NSSAIs reached the maximum number of UEs per S-NSSAI, the AMF sends a Registration Accept message to the UE in which the AMF includes the rejected S-NSSAI(s) in the rejected NSSAI list for which the NSACF has indicated that the maximum number of UEs per network slice has been reached and for each rejected S-NSSAI the AMF includes a reject cause set to 'maximum number of UEs per network slice reached' and optionally a back-off timer.

When for all the Requested S-NSSAI(s) provided in step 2 the NSACF returned the maximum number of UEs per network slice has been reached and if one or more subscribed S-NSSAIs are marked as default in the subscription data and not subject to NSAC, the AMF can decide to include these Default Subscribed S-NSSAIs in the Allowed NSSAI. Otherwise, the AMF rejects the UE request for registration. In the Registration Reject message, the AMF includes the rejected S-NSSAI(s) in the rejected NSSAI parameter and for each rejected S-NSSAI the AMF includes a reject cause to indicate that the maximum number of UEs per network slice has been reached and optionally a back-off timer.

NOTE 5:	If the use case requires the UE to remain reachable at all times with at least one slice, it is recommended that at least one of the Subscribed S-NSSAIs is marked as the default S-NSSAI which is not subject to NSAC. This will ensure the UE is able to access to services even when maximum number of UEs per network slice has been reached.

For a centralized architecture the following differences apply:

-	In step 2, the AMF additionally includes the NSAC service area the AMF belongs to, if available, as an additional parameter in the Nnsacf_NSAC_NumOfUEsUpdate_Request.

-	In step 3, based on operator configuration, the NSACF performs the validation against the maximum number of Registered UEs registered per NSAC service area defined for the network slice if applicable and available, or the maximum number of Registered UEs in the entire PLMN for the network slice. Additionally the NSACF stores the NSAC service area of AMF if available.

NOTE 6:	When a centralized NSAC architecture is deployed, NSACF does not perform any readmission at inter-AMF mobility since the UE is already admitted if the validation of maximum number of Registered UEs against the entire PLMN.

## MORE CHANGES

#### 4.2.11.4	Number of PDU Sessions per network slice availability check and update procedure

This clause applies to Non-Hierarchical NSAC and centralized NSAC architectures. The difference between the two architectures for the various steps, where applicable, is described at the end of the clause.

The number of PDU Sessions per network slice availability check and update procedure is to update (i.e. increase or decrease) the number of PDU Sessions established on S-NSSAI which is subject to NSAC. The SMF is configured with the information indicating which network slice is subject to NSAC.

NOTE 1:	EAC mode is not applicable for Number of PDU Sessions per network slice availability check and update procedure.

Figure 4.2.11.4-1: Number of PDU Sessions per network slice availability check and update procedure

1.	If the SMF is not aware of which NSACF to communicate, the SMF performs NSACF discovery as described in clause 6.3.22 of TS 23.501 [2] and in clause 5.2.7.3.2. The SMF anchoring the PDU session triggers the Number of PDU Sessions per network slice availability check and update procedure for the network slices that are subject to NSAC at the beginning of a PDU Session Establishment procedure (clause 4.3.2.2.1 and clause 4.3.2.2.2) only for new PDU Sessions to be established, when the S-NSSAI of the PDU session is updated during the PDU Session modification procedure (clause 4.3.3.2), and as a last step of successful PDU Session Release procedure (clause 4.3.4.2 and clause 4.3.4.3).

NOTE 2:	SMFs handling PDU sessions associated with UE Request Type "Existing PDU Session" for intra access handover purposes do not interact with the NSACF.

2.	The SMF anchoring the PDU session sends Nnsacf_NSAC_NumOfPDUsUpdate_Request message to the NSACF. 

In the case of Network Slice Replacement (see clause 5.15..19 of TS 23.501[2]), the SMF of the alternative network slice triggers the Nnsacf_NSAC_NumOfPDUsUpdate towards the NSACF(s) that handle the network slice(s) involved in the replacement procedure if NSAC is applicable to any of the network slices. Also, The SMF includes both the replaced and alternative S-NSSAIs for Network Slice Replacement so that NSACF(s) can use it as specified in 3GPP TS 23.501[2], if the SMF received the SM Context Update message including the Alternative S-NSSAI and the S-NSSAI and the SMF determined to retain, release or re-establish the PDU session as specified in clause 4.3.3.2 or if the SMF received the SM Context Create message including the Alternative S-NSSAI and the S-NSSAI as specified in clause 4.3.2.2.1.

The SMF includes in the message the UE-ID, the PDU session ID, S-NSSAI for which the number of PDU Sessions per network slice update is required, Access Type and the update flag. The update flag may include one of the following values:

-	'increase' which indicates that the number of PDUs established on the S-NSSAI is to be increased when the procedure is triggered at the beginning of PDU Session Establishment procedure, when the S-NSSAI of the PDU session is updated to the Alternative S-NSSAI at the PDU Session Modification procedure due to Network Slice Replacement, or when a new user plane leg is to be established for an MA PDU Session;

-	'decrease' which indicates that the number of PDU Sessions on the S-NSSAI is to be decreased when the procedure is triggered at the end of PDU Sessions Release procedure, when the S-NSSAI of the PDU session is updated to the Alternative S-NSSAI at the PDU Session Modification procedure due to Network Slice Replacement, or when an existing user plane leg is to be released for an MA PDU Session. In the case of a PDU Session Establishment failure, the anchor SMF triggers another request to the NSACF with the update flag parameter equal to decrease in order to re-adjust back the PDU Session counter in the NSACF; or

-	'update' which indicates that for existing PDU Session the Access Type is to be replaced with a new Access Type during inter access mobility.

NOTE 3:	For SSC mode 3 PDU session, the SMF of the new PDU Session invokes the NSACF to increase the number of PDU Session and adds the new PDU session ID in the NSACF. When the old PDU session is released the SMF of the old PDU session invokes the NSACF to decrease the number of PDU Session and remove the old PDU session ID in the NSACF.

NOTE 4:	An SMF anchoring an IPv6 Multi-homed PDU session does not invoke NSACF for an S-NSSAI subject to NSAC when the PDU session replaces an existing anchor according to clause 4.3.5.3.

3.	The NSACF updates the current number of PDU Sessions established on the S-NSSAI, i.e. increase or decrease the number of PDU Sessions per network slice based on the information provided by the anchor SMF in the update flag parameter.

If the update flag parameter from the SMF anchoring the PDU session indicates increase value and the maximum number of PDU Sessions established on the S-NSSAI has already been reached, then the NSACF returns a result parameter indicating that the maximum number of PDU Sessions per network slice has been reached. If the maximum number of PDU Sessions established on the S-NSSAI has not been reached, the NSACF checks the UE ID. If the UE ID is located, the NSACF, stores the PDU Session ID and the Access Type and increases the number of PDU Sessions for that S-NSSAI. If the NSACF did not locate the UE ID, it creates an entry for the UE ID, stores the PDU Session ID and Access Type and increases the number of PDU Sessions for that S-NSSAI.

If the update flag parameter from the SMF anchoring the PDU session indicates decrease value, the current number of PDU Sessions per S-NSSAI, the NSACF locates the UE ID and decreases the number of PDU Sessions for that S-NSSAI and removes the related PDU Session ID entry. If the UE ID has no more PDU sessions, after the decrease, the NSACF removes the UE ID entry.

If the update parameter from the SMF indicates both the replaced and Alternative S-NSSAIs due to Network Slice Replacement, the NSACF behaves as specified in 3GPP TS 23.501 [2].

If the update flag parameter from the SMF anchoring the PDU session indicates update value, the NSACF locates the existing entry with UE ID and PDU Session ID and replaces the Access Type in the existing entry.

The NSACF takes the Access Type parameter into account for increasing and decreasing the number of PDU Sessions per S-NSSAI as described in clause 5.15.11.2 of TS 23.501 [2]. For MA PDU Session, if the SMF received information that the UE is registered over both accesses, the SMF provides multiple Access Types to the NSACF. If the NSACF receives a request containing multiple Access Types, the NSACF provides a Result indication for each Access Type.

4.	The NSACF acknowledges the update to the anchor SMF with Nnsacf_NSAC_NumOfPDUsUpdate_Response message including a Result indication. If the NSACF returns a Result indication including 'maximum number of PDU Sessions per S-NSSAI reached', the SMF rejects the PDU Session establishment request with reject cause set to 'maximum number of PDU Sessions per S-NSSAI reached' and optionally a back-off timer and the Access Type.

For MA PDU Session Establishment, the NSACF may accept the MA PDU Session and may provide to the SMF a Result indicating 'maximum number of PDU Sessions per S-NSSAI reached' or 'maximum number of PDU Sessions per S-NSSAI not reached' associated with an Access Type. If the NSACF indicates a failure that is associated with the Access Type over which the UE sent the MA PDU Session Establishment Request, the SMF sends to the UE a PDU Session Establishment Reject with a Result indication including 'maximum number of PDU Sessions per S-NSSAI reached' ,optionally a back-off timer and the Access Type. When the SMF rejects the MA PDU Session, the SMF sets the Access Type parameter as follows:

-	If the UE is registered via both accesses and:

-	If the NSACF indicates failure for both accesses, the Access Type indicates both accesses;

-	If the NSACF indicates failure for the access over which the MA PDU Session Establishment Request is received, the Access Type indicates the access over which the MA PDU Session Request is received.

NOTE 5:	If the UE is registered in both accesses and the NSACF indicates failure for the access different from the access over which the MA PDU Session Establishment Request is received, the SMF accepts the MA PDU Session Request and does not provide back-off timer to the UE.

-	If the UE is registered via a single access, the Access Type indicates the access over which the MA PDU Session Request is received.

-	For MA PDU Session Release over single Access Type, the NSACF locates the existing entry with PDU Session ID and if founds the entry with both Access Type then it removes only the received Access Type entry while keeping the PDU Session ID.

For a centralized architecture the following differences apply:

-	In step 2, the SMF additionally includes the NSAC service area the SMF belongs to, if available, as an additional parameter in the Nnsacf_NSAC_NumOfPDUsUpdate_Request.

-	In step 3, based on operator configuration, the NSACF performs the validation against the maximum number of PDU Sessions established on the S-NSSAI per NSAC service area, if applicable and available, or maximum number of PDU Sessions established on the S-NSSAI in the entire PLMN. Additionally the NSACF stores the NSAC service area of SMF if available.

## MORE CHANGES

##### 5.2.21.2.2	Nnsacf_NSAC_NumOfUEsUpdate service operation

Service Operation name: Nnsacf_NSAC_NumOfUEsUpdate

Description: Updates the number of UEs registered with a network slice (e.g. increase or decrease) when the UE registration status for a network slice subject to NSAC has changed. Also, if the number of the UEs registered with the network slice is to be increased and the Early Admission Control (EAC) mode in the NSACF is activated for that network slice (see Nnsacf_NSAC_EACNotify service operation), the NSACF first checks whether the number of UEs registered with the network slice has reached the maximum number of UEs per network slice threshold. If the maximum number of UEs registered with the network slice has already been reached, the UE registration for that network slice via the same Access Type configured in the NSACF is rejected. If the EAC is not activated, the NSACF increases or decreases the number of UEs per network slice as per the input parameters below.

Inputs, Required: S-NSSAI(s), UE ID (SUPI), NF ID, Access Type, update flag.

Inputs, Optional: No.

Inputs, Conditional: Notification endpoint for EAC Notification for the S-NSSAI, NSAC admission mode, PLMN ID, UE already registered indication, AMF NSAC service area, replaced S-NSSAI, Alternative S-NSSAI.

The S-NSSAI(s) parameter is a list of one or more network slices for which the number of UEs registered with a network slice is to be updated and checked if the maximum number of UEs per network slice threshold has already been reached. In the roaming case, the corresponding mapped S-NSSAI(s) of the HPLMN is also included.

The UE ID parameter is used by the NSACF to maintain a list of UE IDs registered with the network slice. The NSACF also takes Access Type into account for increasing and decreasing the number of UEs per network slice as described in clause 5.15.11.1 of TS 23.501 [2].

The NF ID parameter is the NF instance ID of the NF (e.g. AMF or SMF + PGW-C) sending the request to the NSACF.

The update flag input parameter indicates whether the number of UEs registered with a network slice is to be:

-	increased when the UE registers to a new network slice subject to NSAC. If the UE ID is already in the list of UEs registered with the network slice, the number of UEs registered with the network slice is not increased as the UE has already been counted as registered with the network slice. If the UE ID is not in the list of UE IDs registered with the network slice and the maximum number of UEs registered with the network slice has not been reached yet, the NSACF adds the UE ID in the list of UEs registered with the network slice and increases the number of the UEs registered with the network slice. If the UE_ID is not in the list of UEs registered with that S-NSSAI and the maximum number of UEs per network slice for that S-NSSAI has already been reached, then the NSACF returns maximum number of UEs per network slice reached result;

-	decreased when the UE deregisters for a network slice that is subject to NSAC. The NSACF decreases the number of the UEs registered with the network slice and removes the UE ID from the list of UEs registered with the network slice;

-	updated when the UE moves the PDU sessions across Access Types as described in the clause 4.11.5.9a.

NSAC admission mode applies to inbound roamers. It is included by NF consumer to indicate applicable NSAC admission mode in VPLMN. Its values can be VPLMN NSAC admission mode, or VPLMN with HPLMN assistance NSAC admission mode.

The PLMN ID is the serving PLMN of the UE.

The UE already registered parameter indicates that the UE has already been registered in another NSACF service area.

The AMF NSAC service area is included in a centralized NSAC architecture, if available.

The NSACF may optionally return the current status of the network slice availability (e.g. a percentage out of the maximum number of UEs registered with a network slice) in the availability status parameter. This information may be used for NSACF signalling and load balancing in case multiple NSACFs are serving the same network slice.

If the operation is invoked due to Network Slice Replacement, the AMF also indicates both the replaced and alternative S-NSSAIs so that the NSACF(s) can behave as described in 3GPP TS 23.501 [2].

Outputs, Required: Result indication.

The Result indication parameter contains the outcome of the update and check operation in the NSACF and may indicate one of the values 'maximum number of UEs for the S-NSSAI not reached' or 'maximum number of UEs for the S-NSSAI reached'.

Outputs, Optional: Updated local maximum number of registered UEs, or Updated UE admission threshold.

These parameters can only be returned by a primary NSACF.

## NEXT CHANGE

##### 5.2.21.2.4	Nnsacf_NSAC_NumOfPDUsUpdate service operation

Service Operation name: Nnsacf_NSAC_NumOfPDUsUpdate

Description: Updates the number of PDU Sessions established on a network slice (e.g. increase or decrease). Also, if the number of PDU Sessions on the network slice is to be increased, the NSACF first checks whether the number of the PDU Sessions on that network slice has reached the maximum number of PDU Sessions per network slice. If the maximum number of PDU Sessions on the network slice has already been reached, the PDU Session Establishment procedure is rejected.

Inputs, Required: S-NSSAI, UE ID, PDU Session ID, Access Type, update flag.

The S-NSSAI parameter is the network slice for which the number of PDU Sessions established on a network slice is to be updated. In the LBO roaming case, the corresponding mapped S-NSSAI of the HPLMN is also included.

The UE ID parameter is used by the NSACF to maintain a list of UE IDs that has established PDU sessions with the network slice.

PDU Session ID parameter is used by the NSACF to maintain for each UE ID, the PDU Session ID(s) for established PDU Sessions.

The Access Type parameter indicates over which access network type the PDU Session is established. In the case of MA PDU Session, one or multiple Access Types may be included for a PDU Session ID.

The update flag input parameter indicates 'increase', 'decrease' or 'update' as specified in clause 4.2.11.4.

Inputs, Optional: NSAC admission mode, PLMN ID.

NSAC admission mode applies to inbound roamers. It is included by NF consumer to indicate applicable NSAC admission mode in VPLMN. Its values can be VPLMN NSAC admission mode, or VPLMN with HPLMN assistance NSAC admission mode.

The PLMN ID is the serving PLMN of the UE.

Inputs, Conditional: The SMF NSAC service area is included in a centralized NSAC architecture, if applicable, replaced S-NSSAI, Alternative S-NSSAI.

If the operation is invoked due to Network Slice Replacement, the SMF also indicates both the replaced and alternative S-NSSAIs so that the NSACF(s) can behave as described in 3GPP TS 23.501 [2].

Outputs, Required: Result indication, Access Type.

The Result indication parameter contains the outcome of the update and check operation in the NSACF and may indicate one of the values 'maximum number of PDU Sessions for the S-NSSAI not reached' or 'maximum number of PDU Sessions for the S-NSSAI reached'.

The Access Type parameter is associated with the Result indication parameter.

Outputs, Optional: Updated local maximum number of PDUs.

This parameter is returned only by a primary NSACF.

## END of CHANGES