3GPP SA WG2 Meeting #161	S2-2402881

Athens, Greece, February 26th-March 1st, 2024

Source:	Huawei, HiSilicon

Title:	KI#1 New Sol: UAV flight planning and monitoring

Document for:	Approval

Agenda Item:	19.10

Work Item / Release:	FS_ UAS_Ph3 / Rel-19

Abstract of the contribution: This paper proposes a solution for KI#1 of FS_UAS_Ph3.

# Discussion

The aim of the key issue #1 is to study whether and how to enhance NEF services to support service exposure and interactions between MNOs and UTM functions for supporting: pre-mission flight planning and in-mission flight monitoring for UAVs, C2 communication reliability and the scenario of multiple USS serving the geographical areas corresponding to the UAV flight path. This paper is about the pre-mission flight planning and in-mission flight monitoring for UAVs and proposes a solution for the referred part.

UAVs flight planning and monitoring can include the use for the A2X service data delivery in the pre-mission and in-mission flight. As specified in TS 23.256, A2X leverages 5QI 75 originally defined for V2X to deliver the A2X service data. In the V2X service, notification on QoS Sustainability Analytics to the V2X Application Server has been supported in order to adjust the application behaviour in advance with potential QoS change. This solution allows the notification on QoS sustainability analytics mechanism to be used in A2X communication so that the UTM/USS can receive the notification for potential QoS change and manage pre-mission flight planning and in-mission flight monitoring for UAVs.

A2X can leverage what is defined for V2X in clauses 5.4.5.2 in TS 23.287 with the following differences:

-	V2X is replaced by A2X.

-	if the location information is in a fine granularity as a list of waypoints (expressed as longitude, latitude and height in geographical coordinates)

# Proposal

A solution is proposed for KI#1 in the UAS Ph3 TR23.700-59.

*** Start of changes (all new text) ***

## 6.X	Solution #X: UAV flight planning and monitoring

### 6.X.1	Key Issue mapping

This solution is related to KI#1:

-	It addresses Key Issues #1 since it proposes to enhance the notification on QoS sustainability analytics mechanism to the A2X application server for pre-mission flight planning and in-mission flight monitoring.

### 6.X.2	Description

An A2X Application Server may request notifications on QoS Sustainability Analytics from the NEF for an indicated geographic area or a path of interest and a target time interval in order to adjust the application behaviour (e.g., UTM) in advance with potential QoS change. The A2X Application Server may also request past statistical information for the purposes of adjustment of the application, how UTM/USS makes use of such data is outside of 3GPP scope.

The A2X application server can be the service consumer and the request from the A2X application server contains the following parameters:

-	Analytics ID = "QoS Sustainability";

-	Target of Analytics Reporting: "any UE";

-	Analytics Filter Information:

-	Optional maximum number of objects;

-	QoS requirements:

-	5QI (standardized or pre-configured), and applicable additional QoS parameters and the corresponding values (conditional, i.e., it is needed for GBR 5QIs to know the GFBR); or

-	the QoS Characteristics attributes including Resource Type, PDB and PER and their values;

NOTE:	The A2X Application Server could provide the 5QIs as input based on SLA with the operator which authorizes the A2X Application Server to use certain 5QIs and to use the QoS Sustainability Analytics only for them.

-	Location information: An Area of Interest or a path of interest. The location information could reflect a list of waypoints:

-	if the location information is an Area of Interest, the area can be either described in a coarse granularity as list of TAIs or Cell IDs, or in a fine granularity as geographical area (that can be smaller than a cell), or both (coarse and fine granularity); if both granularities are provided, the NWDAF understands that the area of interest is the intersection between the fine granularity location and the list of TAIs or Cell IDs.

-	if the location information is a path of interest, the area can be either described in a coarse granularity as list of TAIs or Cell IDs, or in a fine granularity as a list of waypoints (expressed as longitude, latitude and height in geographical coordinates) and combined with a radius value, or both (coarse and fine granularity); if both granularities are provided, the NWDAF understands that the path of interest is the intersection between the fine granularity location and the list of TAIs or Cell IDs.

-	Threshold linear distance: The distance travelled by the UE before reporting subsequent location as described in TS 23.273 [39].

-	S-NSSAI (optional, if available in the A2X Application Server).

-	Optional UE Device and Context Information: which may contain one or more of the following:

-	Speed range, which is a range of UE speeds for which analytics is requested, where the speed range is indicated as a range of Velocity Estimate, as in clause 6.1.6.2.17 of TS 29.572 [37];

-	Device information, which may contain one of the following:

-	List of equipment types, according to clause 8.0 of GSMA TS.06 [38].

-	Analytics target period: relative time interval, either in the past or in the future, that indicates the time period for which the QoS Sustainability analytics is requested;

-	Optionally, Spatial granularity size and Temporal granularity size;

-	Reporting Threshold(s), which apply only for subscriptions and indicate conditions on the level to be reached for the reporting of the analytics, i.e. to discretize the output analytics and to trigger the notification when the threshold(s) provided in the analytics subscription are crossed by the expected QoS KPIs (more details are specified in clause 6.9.1 of TS 23.288 [20]).

-	In a subscription, the Notification Correlation Id and the Notification Target Address.

### 6.X.3	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

### 6.X.4	Impacts on services, entities and interfaces

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

*** END of changes ***