SA WG2 Meeting #SA2-161	S2-2402973

February 26 – March 01, 2024, Athens, Greece	(Revision of S2-2400740)

Source:	China Mobile

Title:	TR 23.700-70 KI#2: Solution for Supporting PDU Set information Identification for end-to-end encrypted XRM traffic

Document for:	Approval

Agenda Item: 	19.3

Work Item / Release:	FS_XRM Ph2

Abstract of the contribution: This draft proposes a solution to address how to support the PDU Set information Identification for end-to-end encrypted traffic in FS_XRM_Ph2.

# * * * * 1st change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policies and Charging control framework for the 5G System; Stage 2".

[5]	IETF RFC 3711: "The Secure Real-time Transport Protocol (SRTP)", March 2004.

[6]	IETF RFC 6904: "Encryption of Header Extensions in the Secure Real-time Transport Protocol (SRTP)".

[7]	IETF RFC 9335: "Completely Encrypting RTP Header Extensions and Contributing Sources".

[8]	IETF draft-ietf-avtcore-rtp-over-quic: "RTP over QUIC (RoQ)".

[9]	IETF draft-ietf-moq-transport: "Media over QUIC Transport".

[10]	IETF experimental draft-ietf-avtext-framemarking: "Frame Marking RTP Header Extension".

[11]	IETF RFC 9000: "QUIC: A UDP-Based Multiplexed and Secure Transport".

[12]	TR 26.926 v18.1.0: "Traffic Models and Quality Evaluation Methods for Media and XR Services in 5G Systems".

[13]	TR 26.925 v18.1.0: "Typical traffic characteristics of media services on 3GPP networks".

[14]	IETF RFC 9330: "Low Latency, Low Loss, and Scalable Throughput (L4S) Internet Service: Architecture.

[15]	IETF RFC 9331: "The Explicit Congestion Notification (ECN) Protocol for Low Latency, Low Loss, and Scalable Throughput (L4S)".

[16]	IETF draft-ietf-tsvwg-ecn-encap-guidelines-22: "Guidelines for Adding Congestion Notification to Protocols that Encapsulate IP".

[17]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[18]	CableLabs DOCSIS MULPI: "Data-Over-Cable Service Interface Specifications DOCSIS 3.1, MAC and Upper Layer Protocols Interface Specification".

[19]	IETF RFC 9332: " Dual-Queue Coupled Active Queue Management (AQM) for Low Latency, Low Loss, and Scalable Throughput (L4S)".

[20]	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations".

[21]	IETF draft-ietf-tsvwg-udp-options: "Transport options for UDP".

[22]	IETF RFC 6363: "Forward Error Correction (FEC) Framework".

[23]	IETF RFC 6364: "Session Description Protocol Elements for the Forward Error Correction (FEC) Framework".

[24]	IETF RFC 6681: "Raptor Forward Error Correction (FEC) Schemes for FECFRAME".

[25]	IETF RFC 6682: "RTP Payload Format for Raptor Forward Error Correction (FEC) ".

[26]	IETF RFC 6695: "Methods to Convey Forward Error Correction (FEC) Framework Configuration Information".

[27]	IETF RFC 6816: "Simple Low-Density Parity Check (LDPC) Staircase Forward Error Correction (FEC) Scheme for FECFRAME".

[28]	IETF RFC 6865: "Simple Reed-Solomon Forward Error Correction (FEC) Scheme for FECFRAME".

[29]	IETF RFC 8680: "Forward Error Correction (FEC) Framework Extension to Sliding Window Codes".

[30]	IETF RFC 8681: "Sliding Window Random Linear Code (RLC) Forward Erasure Correction (FEC) Schemes for FECFRAME".

[31]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[32]	IETF RFC 8627: "RTP Payload Format for Flexible Forward Error Correction (FEC)".

[X1]	IETF RFC 9298: "Proxying UDP in HTTP".

Editor's note:	References [8], [9] and [10] cannot be formally referenced until published as RFC.

# * * * * 2nd change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issue # |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | <KI #1> | <KI#2> | <KI#3> | <KI#4> | <KI#5> | <KI#6> | <KI#7> | <KI#8> | <KI#9> |
| #1: PDU Set content ratio awareness at RAN | x |  |  |  |  |  |  |  |  |
| #2: Discarding of redundant PDUs (FEC) and reporting | x |  |  |  |  |  |  |  |  |
| #3: FEC mechanism and PSI based PDU Set QoS Handling Enhancement | x |  |  |  |  |  |  |  |  |
| #4: PDU Set FEC-based PDU Set QoS Handling | x |  |  |  |  |  |  |  |  |
| #5: PDU Set Handling and Information marking …for PSDB/PSER/PSIHI | x |  |  |  |  |  |  |  |  |
| #6: Enhanced Alternative QoS Profiles for PDU set based QoS handling | x |  |  |  |  |  |  |  |  |
| #7: Enhancing alternative QoS profile …PDU set QoS parameters | x |  |  |  |  |  |  |  |  |
| #8: Consistent PDU Set Handling between AF and 5GS | x |  |  | x |  |  |  |  |  |
| #9: PDU Set information identification for encrypted traffic |  | x |  | x |  |  |  |  |  |
| #10: PDU Set information identification based on MoQ |  | x |  |  |  |  |  |  |  |
| #11: RTP over QUIC based Encrypted Traffic …QoS flows mapping |  | x |  |  |  |  |  |  |  |
| #12: Obfuscated Metadata to Classify Payload in Encrypted Media Packets |  | x |  | x | x |  |  |  |  |
| #13: Multiple DSCP markings per QoS Flow |  |  | x |  |  |  |  |  |  |
| #14: Extending Packet Filter … within a single transport connection |  |  |  | x |  |  |  |  |  |
| #15: Traffic Detection and QoS mapping for XR and Media services |  |  |  | x |  |  |  |  |  |
| #16: AS based trigger of data boost handling with reflective QoS |  |  |  |  | x |  |  |  |  |
| #17: L4S in non-3GPP access networks |  |  |  |  |  | x |  |  |  |
| #18: PDU Set handling in wireline/wireless non-3GPP access |  |  |  |  |  |  | x |  |  |
| #X |  | x |  |  |  |  |  |  |  |

# * * * * 3rd change * * * *

# * * * * (all new texts) * * * *

## 6.X	Solution#X: Use UDP-option and Proxy-UDP to address the PDU-Set info identification for E2E encrypted XRM traffic

### 6.X.1	Description

The usage of end-to-end information encryption between a media server and a subscriber is widely deployed for media service transmission. As specified in the TR 23.700-70, a media flow could be either fully encrypted – with both the media header and the media payload encrypted, or partially encrypted – with the media payload encrypted but the media header only partially encrypted. This solution proposal addresses the case of the full-encryption of an XRM media flow – evidently, the partially-encrypted case can be accommodated with the same solution.

Based on the Rel-18 XRM work, when XRM packets (of a media flow), generated at a media server, reach a UPF, the UPF needs to extract and parse the packet contents for PDU set information identification. For this scenario, either the packet contents have to be in plaintext, as required in the Rel-18 XRM Phase-1 work, for which the UPF faces no challenge to build up the PDU-set info, or if the packet is end-to-end encrypted, the UPF must have some mechanism to ‘decrypt’ the packets for PDU-set info extraction & parsing. Here, ‘end-to-end’ means the communication of a subscriber (UE) and a media server (AS), via intermediaries over a public domain. This KI#2 mandates that any solution that relies on the breaking of end-to-end encryption be unacceptable.

### 6.X.1.1	UDP-Option

As we know, if a media flow is fully encrypted end-to-end at a media server, then no intermediary, regardless of being an eavesdropper or an authorized observer as design, could peek into the media frame, including both the header and payload, except the destination receiver (i.e., UE) of the flow. So, our solution must depend on somewhat technology that will be applied in the media server and that would also carry the so-required PDU set related information outside of the (encrypted) media frame layer. Remember this PDU-set info is per-packet associated and thus the technology would be best implemented in some on-packet field(s).

With this mind, we propose to use the new IETF technology UDP-option [21] for the PDU-set information carryover. The UDP-option draft [21] extends thru the use of transport header options that are associated with the UDP protocol. Most commonly, media frames are RTP-encap’ed that would run over UDP. By using the UDP encapsulation with options, though the inside RTP-frame may be encrypted (or not; doesn’t matter), the PDU-set related information are carried in the (outside) UDP option fields. For example, according to the UDP-option IETF draft [21], we may get a code from the 'Kind' range [10...126] to identify the top-level as the type of ‘3GPP Services’. Then, we further define the sub-structures for more concrete service types. Regarding this solution, it would be the ‘XRM service’. Successively, the related PDU-set information would be defined and carried in Option fields. Moreover, another advantage is that UDP is a layer-4 protocol and its header & (newly-added) options are not normally processed by intermediary network elements (e.g., routers, switches, etc.). Not only does this relieve the processing burden off IP transport devices, but also gives a clear demarcation of the TCP/IP layer structure. The Figure 6.x.1.1 is the encap’ed UDP datagram structure with Option fields added toward the end:

Figure 6.X.1.1: UDP w/ Option for PDU Set information carryover

When this type of UDP-option encap’ed media frame reaches a UPF, the UPF can use the PDU-set information carried in the UDP-option fields for XRM service handling. At the UPF, the original RTP-encap’ed media frames are still encrypted and the UPF cannot peek into them. So, the E2E encryption is still warranted.

However, one issue we must further tackle is that, when a UDP datagram with option fields is transmitted from a media server to a subscriber (UE in a 5GS network) over a public domain, the UDP-option fields are not encrypted and that critical PDU-set information, if in plaintext, might be observed & compromised by an intermediary 3rd-party entity. So, we introduce the scheme ‘Proxying UDP in HTTP’ as in RFC 9298 [X1] to address the risk.

### 6.X.1.2	RFC-9298 Proxying UDP in HTTP

We propose to use the HTTP/3 to proxy the UDP datagram, as per the RFC 9298 [X1]. Because the UDP option fields contain sensitive PDU-set information, HTTP/3 will be used as the proxy protocol. HTTP/3 runs over QUIC, which provides the integrity and privacy protection via TLS. Since the RFC 9298 has been introduced into TS 23.501 [2] via the standard work of ATSSS, we do not foresee any significant barrier for the inclusion of the RFC 9298 technology for this XRM E2E encryption KI. The HTTP/3 proxy client runs on a media server, while the HTTP/3 proxy server is installed and activated in the mattered UPF. The Figure 6.x.1.2 shows the encap’ed structure of the UDP-proxying HTTP/3 datagram payload:

UDP Proxying HTTP Datagram Payload {

Context ID (i),

UDP Proxying Payload (..),

}

Figure 6.X.1.2: UDP-proxying HTTP datagram payload

This HTTP payload will be further encap’ed with QUIC + Proxy-UDP, and sent over to the HTTP/3 proxy server on a UPF over the public domain. The QUIC protocol helps the integrity protection. Once the HTTP/3 proxy server (on the UPF) receives the HTTP datagram, it extracts out the UDP-proxying payload (as shown in the Figure 6.x.1.2). The UDP-proxying payload, conforming to what is shown in the Figure 6.x.1.1, contains the PDU-set related information (in plaintext) in UDP-option fields. Note that the UDP payload itself, containing the original RTP header & media contents, is still encryption-protected at the moment. Then, the UPF can start processing the PDU-set information following the work that has been standardized via the Rel-18 XRM.

### 6.X.1.3	The Holistic Solution: UDP-option + RFC 9298

In summary, our solution is the integrated application of both the IETF UDP-option [21] and the RFC 9298 [x1]. The holistic solution is shown in the Figure 6.x.1.3:

Figure 6.X.1.3: Holistic solution: UDP-option + RFC 9298

In the Figure 6.x.1.3, at the side of ‘Media Sever’, the point ‘M1’ shows a UDP datagram with UDP option fields [21] carrying the XRM PDU-set information. The E2E encrypted RTP media data is contained in the UDP payload field. The point ‘M2’ shows a (UDP-proxying) HTTP datagram payload as per the RFC 9298 [X1]. With the HTTP/3 being used for proxy, the point ‘M3’ shows the final proxied UDP datagram that will be transmitted, over a public domain, toward the UPF (in a 5G network). At the UPF, the UDP proxying payload is shown after the HTTP/3 proxy decap’ed. With this solution, the original RTP contents maintain the E2E integrity & privacy, while the UPF is able to extract and parse the PDU-set information.

### 6.X.2	Procedures

The following procedure only shows the enhanced steps to the existing PDU session establishment/modification procedure in term of the AF-triggered service request.

NOTE xx:  It is reasonably assumed that AF and AS (Media server) can communicate to share info.

Figure 6.X.2: Procedure of provisioning UDP-option + HTTP/3 Proxy

1.	An AF function sends the request of provisioning both HTTP/3 proxy and UDP-option to NEF (untrusted) or to PCF (trusted).

2.	PCF generates the corresponding PCC rule and sends over to SMF.

3.	A PDU session establishment or modification procedure is triggered that will use the PCC rule.

4.	A UPF is (re-)selected that supports the HTTP/3 proxy and UDP-option.

5.	The UPF generates the URI (of the HTTP/3 proxy) and install a HTTP/3 proxy server with the URI info.

6.	UPF and SMF go thru the N4 session modification update. The SMF receives the settings of the HTTP/3 proxy server from the UPF.

7.	The SMF sends the HTTP/3 proxy server info to PCF, which further sends the info to AF (possibly via NEF for untrusted case). If the AF is different from the AS (i.e., a media server), then the AF communicates with the AS to share the info.

8. 	The media server (i.e., AS) sends the downlink data to the UPF, with the application of both ‘Proxy UDP in HTTP’ and ‘UDP option’. Note that the UDP-option accommodates the PDU-set related information.

9.	The UPF extracts and parses the PDU-set information from the ‘UDP Proxying Payload’, and then continues the processing as per the Rel-18 XRM standard.

### 6.X.3	Impacts on services, entities and interfaces

AF:

-	Send the request of provisioning HTTP/3 proxy and UDP option.

PCF:

-	Receive the HTTP/3 proxy and UDP option provisioning request.

-	Generate the corresponding PCC rule

-	Upon receiving the information of a HTTP/3 proxy server from SMF, send the info to AF (possibly via NEF for untrusted case)

SMF:

-	Receive the PCC rule for PDU session establishment/modification

-	(Re-)select a UPF that supports the HTTP/3 proxy and UDP option

-	Upon receiving the information of a HTTP/3 proxy server from UPF, send the info to PCF

UPF:

-	Generate the URI (of the HTTP/3 proxy) and install a HTTP/3 proxy server with the URI

-	Send the HTTP/3 proxy server information to SMF

-	Extract & parse PDU-set information from the ‘UDP Proxying Payload’

# * * * * End of changes * * * *