SA WG2 Meeting #161	S2-2403055_was_2990

February 26 – March 1, 2024, Athens, Greece

Source:	InterDigital Inc.

Title:	New use case for Vertical Federated Learning

Document for:	Approval

Agenda Item: 	19.15

Work Item / Release:	FS_AIML_CN / Rel-19

Abstract of the contribution: This paper proposes a new use case on WT#2 for Vertical Federated Learning.

# 1	Discussion

Based on the last SA#102 Plenary meeting of the approved R19 FS_AIML_CN (SP-231800), this paper proposes a use case related to WT2.

- WT2: Study whether and what potential enhancements are needed to enable 5G system to assist in collaborative AI/ML operation involving 5GC/NWDAF and/or AF for “Vertical Federated Learning (VFL)”. The work will be based only on and limited to the scope of justified use cases.

# 2. Proposal

It is proposed to adopt the following text in TR 23.700-84.

* * * * Start of Changes (All new text)  * * * *

## 5.1.X	Use Case #X: Vertical Federated Learning for Member UE Selection Assistance.

### 5.1.X.1	Description

In Release 18, the 5GC supports a new service for member UE selection assistance to help Application Functions (AF) with determining candidate UEs based on filtering criteria provided by the AF. The procedure is described in detail in clause 4.15.13in TS 23.502.

Figure 1 illustrates some aspects of the member UE selection procedure.

The AF provides UE1-UE7 as the target UEs, along with some filtering criteria to the 5GC.

The 5GC uses the filtering criteria to determine a list of candidate UEs that can be potentially used by the application function. In the example, 5GC determines that candidate UEs are UE1-UE5.

The application may then select UEs from the candidate UE list. In the example, the application function selects UE1, UE2 and UE3.

Figure 1. Example 1 of Member UE Selection Assistance by 5GC.

In this UC, it is assumed that the NEF may use ML models to infer the member UE candidates to be provided to the requesting AF. E.g., the 5GC and one or more AFs may collaborative train ML models used to infer member UE selection, to improve the performance of these models, e.g., when sensitive data cannot be shared.

An example is provided where ML models may be trained for specific sample, e.g., an S-NSSAI, an AoI or a group of UEs, and the 5GC and AF may collect local training data using distinct Member UE selection criteria.

This may be useful for e.g., an AF, e.g., to determine when new filtering criteria needs to be provided to the 5GC, for an ongoing Member UE Selection subscription.

The 5GC and the AF may use Vertical Federated Learning to further refine the member selection assistance procedure, by collaboratively training ML models to infer member UE candidates. The 5GC (e.g., an NWDAF) and the AF may build a ML model that uses intermediate data from one another, , and train this model to optimize the member UE selection assistance service.

In a second example shown in Figure 2, two application functions use the member UE selection assistance service with the 5GC to help select UEs for their application traffic.

Figure 2. Example 2 of member UE selection assistance by 5GC.

The application functions AF1 and AF2 request assistance with member UE selection from the 5GC, providing each a list of target UEs and one or more filtering criteria. AF1 and AF2 may have a subset of UEs from the two target UE lists, that are common in both member UE selection assistance requests. In the example, UE5, UE6 and UE7 are common UEs provided in the requests from AF1 and AF2.

In this case, the 5GC may provide these common UEs in the candidate UE list, which can be selected by both Application functions afterwards.

In this example, the AFs and the 5GC may avail of a VFL mechanism, and use common UEs (e.g., UE5-UE7 in the example) as a sample, but different filtering criteria, which is part of the feature space, and they may collaboratively train ML models, to infer member UE candidates, out of the list of target UEs..

Note 1: The required functionality needed at the NEF to train ML models used to derive the Member UE candidates, may be obtained by collocating NWDAF functionality with MTLF and AnLF capabilities, within the NEF.

Note 2: Although this UC suggests that VFL may be used amongst 5GC, and AFs, to derive Member UE candidates, the UC may apply to other mechanisms that involve 5GC (e.g., NWDAF) and AFs, and that may avail of VFL to e.g., mitigate privacy concerns.

* * * * End of Changes * * * *