SA WG2 Meeting #161	S2-2403060

# Athens, February 26 – March 1, 2024		       (revision of S2-2401970)

Source: 	Samsung

Title: 	KI#1: New Solution on LMF Support for AI/ML Direct Positioning

Document for: 	Approval

Agenda Item: 	19.15

Work Item / Release:	FS_AIML_CN / Rel-19

Abstract of the contribution: This pCR proposes a new solution on LMF Support for AI/ML Direct Positioning.

# 1	Discussion

This pCR proposes a new solution to KI#1 on LMF Support for AI/ML Direct Positioning

# 2. Proposal

It is proposed to adopt the following text in TR 23.700-84.

*** Start of 1stchange ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[6]	3GPP TR 38.843: "Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface".

[Z]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS)".

*** Start of 2nd change ***

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues and Use Cases

|  | Key Issues |  | Use cases (optional) |  |
| --- | --- | --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> | <use case #x> | <use case #y> |
| #X | X |  |  |  |

*** Start of 3rd change (all new text) ***

## 6.X	Solution #X: Support for AI/ML Direct Positioning Training, Inference and Data Collection with LMF-side models

### 6.X.1	Description

This solution addresses KI#1. The solution presents three different aspects regarding 5GC support for AI/ML Direct Positioning with LMF-side models, namely training, inference and data collection procedures. The solution addresses Case 2b and 3b as defined in TR 38.843 [6].

The main features of the solution are as follows:

-	LMF requests training data from NWDAF.

-	NWDAF or LMF may perform the AI/ML training.

-	NWDAF may share a trained model with LMF.

-	When a UE location is requested, LMF performs AI/ML inference on the trained model.

In addition, this solution allows LMF to be co-located with NWDAF containing MTLF/AnLF for model training/model inference.

### 6.X.2	Procedures

Clauses 6.X.2.1, 6.X.2.2 and 6.X.2.3 show step-by-step procedures for training, inference and data collection aspects of AI/ML Direct Positioning with LMF-side models, respectively.

NOTE:	In all procedures of this solution, LMF may be a standalone NF or co-located with NWDAF containing MTLF/AnLF for model training/model inference (i.e. no interaction between the LMF with the co-located NWDAF needs to be defined).

#### 6.X.2.1	Training procedure for AI/ML Direct Positioning with LMF-side models

Figure 6.X.2.1-1: Training procedure for AI/ML Direct Positioning with LMF-side models

The training procedure for AI/ML Direct Positioning with LMF-side models in Figure 6.X.2.1-1 is described step by step below.

1.	LMF subscribes to training data from NWDAF for an AI/ML model to be used for direct positioning. The training data subscription may be a request for analytics using the existing services, or a request for a model, or for other type of data relevant for training. Several optional parameters as defined in TS 23.288 [5] may be provided as service operation inputs.

NOTE 1:	The training data subscription may be triggered by LMF receiving a previous request from another network entity. The trigger itself is not depicted in Figure 6.X.2.1-1 as it is not considered part of the training procedure itself.

Editor’s Note: The list of required and optional inputs for LMF training data subscription to NWDAF is FFS.

2.	Data collection is performed to train the model(s) for direct positioning.

NOTE 2:	 The data collected from UE and NG-RAN is in RAN WGs scope.

Editor’s Note: Whether and how any data collection for training from NFs is performed in the 5GC is FFS.

3.	[OPTIONAL] If the training data subscription from LMF in step 1 requires a trained model, NWDAF containing MTLF may train the AI/ML model.

4.	NWDAF containing MTLF notifies LMF that the requested AI/ML model for direct positioning has been trained using the Notification service operation of the same service used in step 1.

5.	[OPTIONAL] If the AI/ML model for positioning has not been trained, LMF trains the AI/ML model.

#### 6.X.2.2	Inference procedure for AI/ML Direct Positioning with LMF-side models

Figure 6.X.2.2-1: Inference procedure for AI/ML Direct Positioning with LMF-side models

This procedure assumes that a trained AI/ML model is available in LMF for Direct Positioning, i.e. the procedure in Figure 6.X.2.1-1 has already taken place. The inference procedure for AI/ML Direct Positioning with LMF-side models in Figure 6.X.2.2-1 is described step by step below.

1.	Steps 1 to 11 in cl. 6.1.2 of TS 23.273 [Z] are followed.

NOTE:	In this solution, it is assumed that the location services consumer need not be aware that AI/ML direct positioning is used by the network to estimate the UE location.

2.	AI/ML Direct Positioning preparations are performed to support Case 2b and Case 3b defined in TR 38.843 [6]. This step is out of SA2 scope.

3.	Data collection is required to perform inference for AI/ML Direct Positioning and estimate the UE location according to Cases 2b and 3b.

Editor’s Note: Whether and how any data collection for inference from NFs is performed in the 5GC is FFS.

4.	AI/ML Direct Positioning inference is performed at LMF.

5.	Steps 13 to 24 in cl. 6.1.2 of TS 23.273 [Z] are followed to deliver the estimated UE location to the consumer.

### 6.X.3	Impacts on services, entities and interfaces

NWDAF:

-	Enhance support for model training services to enable AI/ML positioning training and inference at LMF.

LMF:

-	Support for training/inference for AI/ML Direct Positioning and data collection.

*** End of 3rd change ***