SA WG2 Meeting #161	S2-2403197

# February 26th- March 1st, 2024, Athens, Greece

Source: 	Intel

Title: 	New solution for KI#3: Energy Information Analytics for network energy saving support by NWDAF

Document for: 	Approval

Agenda Item: 	19.4 (FS_EnergySys)

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: This contribution proposes a new for KI#3 in TR 23.700-66.

# 1	Discussion

This document addresses KI#3 in TR 23.700-66 with a new solution proposal.

‐	Whether and how to enhance network analytics for network energy saving and network energy efficiency.

‐	What, if any, energy related information (e.g. per QoS flow/PDU session/UE/NF) is required and how it is collected to support 5GS enhancement.

# 2 Proposal

It is proposed to include the following solution in TR 23.700-66 addressing KI#3.

* * * * First Change * * * *

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | X |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| x |  |  | X |  |

* * * * Next Change * * * *

## 6.x	Solution #X: Energy Information Analytics for network energy saving support by NWDAF

### 6.x.1	Key Issue mapping

This solution addresses key issue#3 described in clause 5.2.

### 6.x.2	Functional Description

The functional description here focuses on enabling Solution 10 defined in clause 6.10 where the EECF uses the services provided by the NWDAF to obtain the required analytics on energy consumption patterns. The EECF supports Energy Evaluation request from other NFs such as SMF. The Energy evaluation request from SMF includes PDU session ID, Energy Data Profile data for the UE, policy information provided by the PCF for the PDU session which includes energy policy recommendation for the PDU session. The EECF evaluates these input parameters and provides and provides session modifications recommendations in PDU session parameters to optimize energy usage. To evaluate the input parameters and provide session modification recommendations, the EECF may consider the historical data of the energy consumption for the network entities or average energy consumption of the NFs involved in the PDU session while determining the evaluation response provided to the SMF. The EECF may obtain the EnergyInformation analytics defined in clause 6.x.2.1 corresponding to the historical energy consumption or prediction for the energy consumption over a certain period, NF load analytics, NF performance analytics using the NWDAF services.

#### 6.x.2.1	Energy Information Analytics

#### The Energy Information Analytics defined in Solution#12 is used in this solution.

### 6.x.3	Procedures

Figure 6.x.3-1: Procedure for EnergyInformation Analytics provided to an NWDAF consumer

1a. The consumer NF (EECF, SMF, PCF) sends a request to the NWDAF for EnergyInformation analytics using either the Nnwdaf_AnalyticsInfo or Nnwdaf_AnalyticsSubscription service. The Analytics ID is set to "EnergyInformation", the Target of Analytics Reporting and the Analytics Filter Information are set as described in clause 6.x.2.

1b. The consumer NF can request statistics or prediction and the analytics can be requested with the filter information. The consumer NF (EECF, SMF, PCF) sends a request to the NWDAF for NF load analytics using either the Nnwdaf_AnalyticsInfo or Nnwdaf_AnalyticsSubscription service. The Analytics ID is set to "NF load".

1c. The consumer NF (EECF, SMF, PCF) sends a request to the NWDAF for NF Performance analytics using either the Nnwdaf_AnalyticsInfo or Nnwdaf_AnalyticsSubscription service. The Analytics ID is set to "NF Performance".

1d. The consumer NF (EECF, SMF, PCF) sends a request to the NWDAF for NF Performance analytics using either the Nnwdaf_AnalyticsInfo or Nnwdaf_AnalyticsSubscription service. The Analytics ID is set to "DN Service performance".

2. The NWDAF collects data required as described in clause 6.x.2 from OAM, AF, other NFs in 5GC to generate the EnergyInformation analytics, NF load analytics and NF performance analytics requested in step 1.

3. The NWDAF derives the requested analytics.

4. The NWDAF sends Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request response (output for requested analytics in step1, [Subscription Correlation Id], Confidence).

NOTE: This solution differs from Solution 12 based on the entity which is the consumer of the NWDAF EnergyInformation analytics. In this solution, the EECF is the consumer of the NWDAF EnergyInformation analytics. The EECF may obtain the EnergyInformation analytics corresponding to the historical energy consumption or prediction for the energy consumption over a certain period when determining the evaluation response provided to the SMF i.e., provides session modifications recommendations in PDU session parameters. In solution 12 clause 6.12.3, the EECF is one of the data sources for data collection by the NWDAF required to generate the requested Energy Information analytics.

### 6.x.4	Impacts on existing services, entities and interfaces

Editor's note:	This clause captures impacts on existing services, entities and interfaces.

* * * * Next change * * * *

#### 6.12.2.1	General Description

This clause describes how NWDAF can provide energy related analytics, in the form of statistics or predictions or both, to another NF.

The service consumer may be an AF, 5GC NF (e.g. NEF, PCF, AMF, SMF, EECF), or the OAM.

The consumer of these analytics shall indicate in the request:

-	Analytics ID = "Energy information";

Editor's note:	The Analytics ID is FFS.

-	Target of Analytics Reporting:, network slice level, NF Instance/NF set level, PDU session level, QoS flow level, and/or UE level, e.g. a single UE (SUPI/GPSI) or a group of UEs or any UE, a QoS flow or a set of QoS flows or a PDU session(s).

Editor's note:	Target of Analytics Reporting is FFS.

-	Analytics Filter Information:

-	S-NSSAI;

-	NSI ID;

-	DNN;

-	DNAI;

-	Application ID;

-	QoS parameters (e.g. 5QI, QoS Characteristics);

-	NF Set ID;

-	NF Instance ID;

-	NF services ID(s);

-	Area of Interest (AoI);

  - -  Energy saving levels

-  Application ID. Certain applications contribution towards more energy consumption example UEs sending periodic data to an application server as part of monitoring or data collection for ML training. If energy consumed for communication with such applications contribute towards max energy credits assigned to NWDAF service consumer (example AF) then the AF may decide to deprioritize monitoring or data collection for ML training for a given period from a given UE or group of UEs.

-  Maximum Energy Credits for a given UE subscription

-	an optional list of analytics subsets that are requested;

-	optionally, preferred granularity of energy related information: PDU Session level, QoS flow level or application level;

-	Optional preferred level of accuracy of the analytics;

-	Optional preferred level of accuracy per analytics subset;

-	Optional Reporting Thresholds;

-	An Analytics target period indicates the time period over which the statistics or predictions are requested;

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

-	Optionally, Spatial granularity size and Temporal granularity size.

-	Indication whether to return the estimated energy consumption required to provide these analytics.

NOTE 1:	It is FFS whether and how to estimate/predict the required energy consumption for providing energy related analytics.

NOTE 2:	This indication can instruct the NWDAF to a) only return the predicted energy consumption for providing analytics for this analytics request/subscription (without actually executing the analytics), b) return in the analytics (subscription) response an estimated energy consumption for providing analytics for this analytics subscription, c) return an estimation of the energy consumption for the analytics generation also in the analytics notify, or a combination of b) and c).

Editor's note:	Whether to extend this indication to analytics request/subscription in general (i.e. also for all existing analytics) and whether other input parameters are needed is FFS.

#### 6.12.2.2	Input Data of Energy Related Analytics

The NWDAF may collect the service data from OAM, AF and/or the 5GC network function in charge of energy monitoring and management of the energy consumption and energy efficiency (i.e. EMF, if any) as listed in Table 6.12.2.2-1 to Table 6.12.2.2-4.

Editor's note:	Whether there is a need for a new 5GC NF such as EMF or whether proposed functionality can be provided by existing NFs (e.g. NWDAF) will be evaluated during evaluation and conclusion phase, and need to consider the progress of KI#1.

Editor's note:	How EMF collects data from other entities is FFS.

Table 6.12.2.2-1: Data collected by NWDAF for energy related analytics from OAM

| Information | Source | Description |
| --- | --- | --- |
| Time window | OAM | The time window or the time duration for the collected parameters. The window is between the window start time and stop time. |
| Energy Consumption at 5GC, NF, Network Slice, gNB or NG-RAN level. | OAM | Energy consumption (EC) at various granularities, according to output analytics to be derived based on consumer request. The EC can be collected from OAM or EMF (if available). For the Energy Consumption parameters collected from OAM: - 5GC Energy Consumption is obtained by summing up the Energy Consumption of all the Network Functions (ECNF) that compose the 5G core network as defined in clause 6.7.3.2 of TS 28.554 [6]. - NF energy EC is the sum of the energy consumption of PNF(s) and/or VNF(s) which compose the NF, as defined in clause 6.7.3.1 of TS 28.554 [6]. - Network Slice Energy Consumption is obtained by summing up the Energy Consumption of all the Network Functions (ECNF) that compose the network slice as defined in clause 6.7.3.3 of TS 28.554 [6]. - The Energy Consumption (EC) of the gNB is obtained by summing up the Energy Consumption of all the Network Functions (NF) that constitute the gNB, as defined in clause 6.7.3.4.2 of TS 23.554. - Energy Consumption of the NG-RAN is obtained by summing up the Energy Consumption of all the gNB that constitute the NG-RAN, as defined in clause 6.7.3.4 of TS 28.554 [6]. |
| Identifiers (i.e. NF ID/ NF Set ID, S-NSSAI, gNB ID, etc.) | OAM | Identifiers of 5GC/NF instance or NF set/ slice/RAN node correspond to the collected EC or EE. |
| Energy efficiency of NG-RAN or network slice | OAM | The energy efficiency of NG-RAN or network slice as defined in clauses 6.7.1 and 6.7.2 of TS 28.554 [6]. |
| NF energy usage per slice | OAM | Total energy consumed for different slice (eMBB, Massive IoT) based on clause 6.7.3.3 of TS 28.554 [6] Network Slice energy consumption (EC). |
| Datacenter energy efficiency ratio | OAM | OAM or Datacenter energy efficiency, e.g. it can be Site Energy Efficiency (SEE) in ETSI ES 203 228 V1.4.1 (2022-04) or the PUE of the datacenter hosting the NF(s) (see ETSI TS 105 174-2-2). |
| Traffic load trend of cells | OAM | The predictions of traffic load trend of certain cells, as defined in TS 28.104 |
| Energy Saving recommendation for NG-RAN | OAM | Energy Saving recommendation for NG-RAN, as defined in TS 28.104 |
| Energy Saving recommendation for 5GC | OAM | Energy Saving recommendation for 5GC, as defined in TS 28.104 |
| Energy Saving recommendation for NF | OAM | Energy Saving recommendation for NF, as defined in TS 28.104 |
| Energy saving level for a given NF | OAM/PCF | The energy saving levels may be set to low, medium or The ener;gy saving level may be set to percentage of time the network function spends in energy saving state |

The NWDAF may also collect data from MDAF. For example, the EnergyEfficiencyProblemType provided by MDAF can help NWDAF with identifying and verifying the derived prediction or statistics of the energy efficiency, i.e. at NF level.

Table 6.x.2.1-1: Input data collection from AF

| Information | Source | Description |
| --- | --- | --- |
| Application ID | AF | To identify the service and support analytics per type of service (the desired level of service). |
| UE ID | AF | The list of UE ID(s) that are associated with the Service |
| Application Server Instance |  | The IP address or FQDN of the Application Server that the UE had a communication session when the Energy consumption measurement was made. |
| IP filter information | AF | Identify a service flow of the UE for the application. |
| UL/DL performance data | AF | The performance associated with the communication session of the UE with an Application Server that includes Average/Maximum Packet Delay, Average/Maximum Loss Rate and Average/Minimum/Maximum Throughput |
| Timestamp | AF | A time stamp associated to the Performance Data provided by the AF. |

Table 6.12.2.2-2: Input data collected from OAM (MDAF)

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | MDAF | A time stamp associated with the collected information. |
| EnergyEfficiencyProblemType | MDAF | Indication of type of the energy efficiency issues. The allowed value is one of the enumerated values: HighEnergyConsumption, LowEenergyEfficiency, Other, Unknown, as defined in clause 8.4.4 of TS 28.104 [16]. |
| EnergyEfficiencyProblematicObject | MDAF | Indication of NR cells or NFs where the energy efficiency issues occurred or potentially occur, as defined in clause 8.4.4 of TS 28.104 [16]. |

In order to determine the EC and EE, in particular the EC and EE at relatively finer granularities (i.e. UE level, PDU session, and QoS flow level), the NWDAF also needs to collect data from other NFs. For example, based on the principle of computing the EC of UE, PDU session or QoS level, the NWDAF needs to determine or estimate the percentage of resources used by the UE, PDU session or QoS level among the overall usage. For example, by acknowledging the number of UEs for a SMF or a slice and the EC of the SMF or slice, the NWDAF is able to determine the average UE level EC. By acknowledging the traffic volume of the UE and that of the associated SMF, and the EC of the SMF, the NWDAF is able to determine the EC of this specific UE. Using similar methods, the NWDAF is able to derive the prediction and statistics of the EC and EE at finer granularity by collecting the informative data.

Table 6.12.2.1-3: Data collected by NWDAF for energy related analytics from NFs

| Information | Source | Description |
| --- | --- | --- |
| Time window | UPF, SMF, or AF | The time window or the time duration for the collected parameters. The window is between the window start time and stop time. |
| UL/DL/overall data volume | UPF, SMF, or AF | UL/DL/overall (UL and DL) data volume of a slice, a UE, a PDU session, Qos flow, UPF, RAN node, 5GC, according to the consumer request. Could be Volume Measurement and/or Throughput Measurement. |
| NF Load information | NRF | Load per NF |
| NF status | NRF | The status of a specific NF instance(s) (registered, suspended, undiscoverable) as defined per TS 29.510 [15]. |
| NF resource usage | OAM | The usage of assigned virtual resources currently in use for specific NF instance(s) (mean usage of virtual CPU, memory, disk) as defined in clause 5.7 of TS 28.552 [10]. |
| NF resource configuration | OAM | The life cycle changes of specific NF resources (e.g. NF operational or interrupted during virtual/physical resources reconfiguration) |
| NF energy state | NRF or NF | Current NF energy state or NF energy state schedule/planning. |
| NF energy consumption estimate | OAM | NF energy consumption estimation (including IT and non-IT resources). This can be a combination of the estimated energy consumption of the NF instance(s) (considering only IT equipment) with the energy efficiency ratio (e.g. PUE) of the datacenter hosting the NF instance(s). The NF energy consumption (considering only IT equipment) can be estimated based on one of the proposed methods in clause 4.1 of TR 28.913 or clauses 6.7.3.1.4, 6.7.3.1.5, 6.7.3.1.6 and 6.7.3.1.7 of TS 28.554 [6]. (NOTE 1) |
| number of QoS flows | OAM, SMF, UPF | The Max/mean number of Qos flows. The measurement can be split into per S-NSSAI at the SMF as defined in clause 5.3.2.1 of TS 28.552 [10]. Or the number of the QoS flow per PDU session at the SMF collected from SMF. |
| number of PDU sessions | OAM, SMF, UPF | The Max/mean number of PDU sessions established by SMF. The measurement can be split into per S-NSSAI as defined in clause 5.3.1 of TS 28.552 [10]. Or the number (active) PDU sessions collected from SMF. |
| Number of UEs | OAM, AMF | The Max/mean number of UEs register to AMF collected from AMF or the number of subscribers per slice as defined in clause 5.2.1 of TS 28.552 [10] collected from OAM. |

In order to derive the output analytics of the renewable energy related information, the NWDAF needs collected the ratio of renewable energy in the total energy supply and the Coefficient of carbon from OAM.

Table 6.12.2.1-4: Input service data related to renewable energy

| Information | Source | Description |
| --- | --- | --- |
| ratio of renew energy in the total energy supply | OAM | The ratio of renew energy in the total energy supply |
| Coefficient of carbon or carbon emission | OAM | Coefficient of carbon is Used to derive the carbon emission, i.e. the statistically observed grams per mill watthour for any use of energy in the country or region where the service is provided. |
| Datacenter Carbon Emission Effectiveness KPI (KPICEE) | OAM | Ratio of CO2eq to the energy consumption by the datacenter hosting the NF, e.g. it can be calculated based on ETSI GS OEU 020 V1.1.1, clause 4.1.2. |

Editor's note:	Coordination with SA WG5 is needed regarding network energy related data collection from OAM.

Editor's note:	Data collected by NWDAF for energy related analytics needs further justification.

#### 6.12.2.3	Energy Related Output Analytics

According to the consumer request, the NWDAF determines the prediction and statistics of energy consumption and energy efficiency along with other corresponding information at different granularities using the input data.

Table 6.12.2.3-1: Statistics of the output analytics

| Information | Description |
| --- | --- |
| List of energy-related information(1…max) | Observed statistics during the Analytics target period in the consumer request. The energy-related information includes statistics of maximum, minimum, variance or average value of energy consumption and energy efficiency at the consumer required granularity over the time slot(s). The energy consumption includes the EC of renewable energy. |
| > Time slot entry (1..max) | List of time slots within the Analytics target period. The time slots of within which the statistics of energy-related information is provided. |
| >>Time slot start | Time slot start within the Analytics target period. |
| >>Duration | Duration of the time slot. If a Temporal granularity size was provided in the request or subscription, the Duration is greater than or equal to the Temporal granularity size. |
| >>Application ID | Identifiers of the one or more applications in use during the time slot within which the EE, EC and the Carbon emission is derived. |
| >> Identifiers (i.e. S-NSSAI/ NSI ID, NF ID/ NF Set ID, gNB ID, , PDU session ID, QFI, UE ID(s) etc.) | Identifiers of slice or slice instance/NF instance or NF set/ RAN node/5GC/PDU session/QoS flow/UE that correspond to the EE, EC and the Carbon emission derived by the NWDAF, based on consumer request. |
| >> NF type | Type of the NF instance. |
| >> energy consumption at required granularities (NOTE 1) | The maximum/ minimum / average / variance value of EC at the consumer required granularity over a period of time, i.e. at RAN level, Core Network level, network slice level, UE level, PDU session level, and/or QoS flow level, or of a requested AOI. This information can be a value (e.g. in Wh) or a class (e.g. low, medium, high). (NOTE 2) |
| >> energy consumption of renewable energy at required granularities (NOTE 1) | The maximum/ minimum / average / variance value of renewable energy consumption at the consumer required granularity over a period of time, i.e. at RAN level, Core Network level, network slice level, UE level, PDU session level, and/or QoS flow level, or of a requested AOI. |
| >> Carbon emission at required granularities (NOTE 1) | The maximum/ minimum / average / variance of Carbon emission at the consumer required granularity over a period of time, of a requested AOI, or of a datacentre hosting the NF. |
| >> energy efficiency at required granularities (NOTE 1) | The maximum/ minimum / average/ variance value of energy efficiency at the consumer required granularity over a period of time, i.e. at RAN level, Core Network level, network slice level, UE level, PDU session level, and/or QoS flow level, of a requested AOI, or of a datacentre hosting the NF. |
| >> maximum energy credit (limit) (NOTE 1) | The upper bound of energy used by the 5G system to provide services provided to one or more subscribers, e.g. the EC of per service per UE level. |
| >> NF energy state history | Indicates the statistics of energy state of relevant NF(s). The information might also include information on the time/schedule when the NF instance was observed to be in a specific energy state. |
| >> traffic volume (NOTE 1) | The traffic volume used to derive the statistics of the energy related information at the required granularity, i.e. the energy consumption or energy efficiency. |
| >> traffic/packet rate (NOTE 1) | The traffic/packet rate associated to the statistics of the energy related information at the required granularity or the traffic/packet rate used to derive the EE and EC. |
| >> traffic latency/ packet delay (NOTE 1) | The traffic latency/ packet delay associated to the statistics of the energy related information at the required granularity or the traffic/packet rate used to derive the EE and EC. |
| >> Datacenter Carbon Emission Effectiveness KPI (KPICEE) | Indicates the statistics of ratio of CO2eq to the energy consumption by the datacenter hosting the NF. |
| >> Datacenter energy efficiency ratio (1..max) | Indicates the statistics of Datacenter energy efficiency, e.g. it can be the PUE of the datacenter hosting the NF(s) (see ETSI TS 105 174-2-2). |
| >> Analytics energy consumption (NOTE 1) | Indicates the observed energy consumptions derived for this analytics request/subscription. |
| >> Spatial Validity | Area where the analytics applies. If a Spatial granularity size was provided in the request or subscription, the number of elements of the list is smaller than or equal to the Spatial granularity size. |
| Energy Saving recommendation for NG-RAN | Energy Saving recommendation for NG-RAN over Analytics target period |
| Energy Saving recommendation for 5GC | Energy Saving recommendation for 5GC over Analytics target period |
| Traffic load trend for cells | Traffic load trend of certain cells over the Analytics target period |
| NF energy saving level | The energy saving status of the NF over the Analytics target period, expressed as a percentage of time per status value (low, medium, high) |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested", "Preferred level of accuracy per analytics subset" and "Reporting Thresholds". NOTE 2: The operator can define the range of energy consumption corresponding to each class of energy. |  |

Table 6.12.2.3-2: Predictions of the output analytics

| Information | Description |
| --- | --- |
| List of energy-related information(1…max) | Predictions during the Analytics target period in the consumer request. The energy-related information includes predictions of maximum, minimum, variance or average value of energy consumption and energy efficiency at the consumer required granularity over the time slot(s). The energy consumption includes the EC of renewable energy. |
| > Time slot entry (1..max) | List of time slots within the Analytics target period. The time slots of within which the predictions of energy-related information is provided. |
| >>Time slot start | Time slot start within the Analytics target period. |
| >>Duration | Duration of the time slot. If a Temporal granularity size was provided in the request or subscription, the Duration is greater than or equal to the Temporal granularity size. |
| >>Application ID | Identifiers of the one or more applications in use during the time slot within which the EE, EC and the Carbon emission. |
| >> Identifiers (i.e. S-NSSAI/ NSI ID, NF ID/ NF Set ID, gNB ID, PLMN ID, PDU session ID, QFI, UE ID(s) etc.) | Identifiers of slice or slice instance/NF instance or NF set/ RAN node/5GC/PDU session/QoS flow/UE that correspond to the EE, EC and the Carbon emission derived by the NWDAF, based on consumer request. |
| >> NF type | Type of the NF instance. |
| >> energy consumption at required granularities (NOTE 1) | The maximum/ minimum / average / variance value of EC at the consumer required granularity over a period of time, i.e. at RAN level, Core Network level, network slice level, UE level, PDU session level, and/or QoS flow level, or of a requested AOI. This information can be a value (e.g. in Wh) or a class (e.g. low, medium, high). (NOTE 2) |
| >> energy consumption of renewable energy at required granularities (NOTE 1) | The maximum/ minimum / average / variance value of renewable energy consumption at the consumer required granularity over a period of time, i.e. at RAN level, Core Network level, network slice level, UE level, PDU session level, and/or QoS flow level, or of a requested AOI. |
| >> Carbon emission at required granularities (NOTE 1) | The maximum/ minimum / average / variance of Carbon emission at the consumer required granularity over a period of time, of a requested AOI, or of a datacentre hosting the NF. |
| >> energy efficiency at required granularities (NOTE 1) | The maximum/ minimum / average/ variance value of energy efficiency at the consumer required granularity over a period of time, i.e. at RAN level, Core Network level, network slice level, UE level, PDU session level, and/or QoS flow level, of a requested AOI, or of a datacentre hosting the NF. |
| >> maximum energy credit (limit) (NOTE 1) | The upper bound of energy used by the 5G system to provide services provided to one or more subscribers, i.e. the EC of per service per UE level. |
| >> NF energy state planning | Indicates the prediction of energy state of relevant NF(s). The information might also include information on the time/schedule when the NF instance is predicted to be in a specific energy state. |
| >> traffic volume (NOTE 1) | The traffic volume used to derive the predictions of the energy related information at the required granularity, e.g. the energy consumption or energy efficiency. |
| >> traffic/packet rate (NOTE 1) | The traffic/packet rate associated to the predictions of the energy related information at the required granularity or the traffic/packet rate used to derive the EE and EC. |
| >> traffic latency/ packet delay (NOTE 1) | The traffic latency/ packet delay associated to the predictions of the energy related information at the required granularity or the traffic/packet rate used to derive the EE and EC. |
| >> Datacenter Carbon Emission Effectiveness KPI (KPICEE) | Indicates the prediction of ratio of CO2eq to the energy consumption by the datacenter hosting the NF. |
| >> Datacenter energy efficiency ratio (1..max) | Indicates the prediction of Datacenter energy efficiency, e.g. it can be the PUE of the datacenter hosting the NF(s) (see ETSI TS 105 174-2-2). |
| >> Analytics energy consumption (NOTE 1) | Indicates the estimated energy consumptions derived for this analytics request/subscription. |
| >> Spatial Validity | Area where the analytics applies. If a Spatial granularity size was provided in the request or subscription, the number of elements of the list is smaller than or equal to the Spatial granularity size. |
| Confidence | Confidence of this prediction. |
| Energy Saving recommendation for 5GC | Energy Saving recommendation for 5GC over Analytics target period |
| Energy Saving recommendation for NG-RAN | Energy Saving recommendation for NG-RAN over Analytics target period |
| Traffic load trend for cells | Traffic load trend of certain cells over the Analytics target period |
| NF energy saving level | The energy saving status of the NF over the Analytics target period, expressed as a percentage of time per status value (low, medium, high) |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested", "Preferred level of accuracy per analytics subset" and "Reporting Thresholds". NOTE 2: The operator can define the range of energy consumption corresponding to each class of energy. |  |

Editor's note:	Whether the analytics of energy consumption, energy efficiency and Carbon emission at NF level and or slice level or those of a datacentre will be provided by NWDAF or OAM is FFS and may require coordination with SA WG5.

Based on energy consumption information,

-	If the consumer is AF, the AF may decide the energy related limits for a service.

-	If the consumer is PCF, the PCF may make policy decision and adjustments.

-	If the consumer is OAM, the OAM may configure the NF to enter an energy-saving mode.

-	If the consumer is AMF, the AMF may perform cell-based paging and provides a list of recommended cells.

-	If the consumer is SMF, the SMF may select a suitable UPF for the UE based on the energy consumption information.

-	If the consumer is CHF, the CHF may record the energy consumption information for charging.

-	The energy consumption of generating analytics can be used by the consumer NF to determine whether to continue subscribing the analytics.

-	The consumer of NWDAF based Energy analytics can use the analytics to define time-varying reporting periodicity (e.g. for event exposure), if supported by the producer NF.

-	If the consumer is EECF, the EECF may obtain the Energy Information analytics corresponding to the historical energy consumption or prediction for the energy consumption over a certain period, NF load analytics, NF performance analytics using the NWDAF services as described in Solution 10.

Editor's note:	The details of whether and how the analytics of EE can be provided at NF service level is FFS.

Editor's note:	What the consumer will do upon reception of energy related analytics provided by NWDAF is FFS.

* * * * End of Change * * * *