3GPP SA WG2 Meeting #S2-161	S2-2403198

26 February - 01 March, 2024, Athens, Greece

Source:	MediaTek Inc., China Mobile

Title:	Green QoS profile and Green Alternative QoS profile provisioning

Document for:	Agreement

Agenda Item:	19.4

Work Item / Release:	FS_EnergySys/Rel-19

Abstract of the contribution: This paper proposes a solution to resolve KI#2

Proposal

It proposes to include the below solution to TR 23.700-66

*** Start of changes ***

6.0	Mapping of solutions to Key Issues

6.x	Solution #X: Green QoS profile and Alternative QoS profile provisioning

6.X.1	Key issue mapping

In this solution, it addresses the following issue:

Key Issue #2: Subscription and policy control to support energy efficiency and energy saving as service criteria

6.X.2	Functional Description

6.X.2.1	General

To reduce the carbon emission, the MNO may deploy the NG-RAN(s) powered by (partial or fully) renewable energy. Furthermore, the MNO may modify the network configuration to make more UE(s) to access those NG-RAN(s) using renewable energy or some service providers may require the communication service provided by MNO to use less non-renewable energy.

This may lead to a condition that the NG-RAN(s) powered by renewable energy will need to support more UEs camping on it, which then may lead to more UEs to share same amount of resource from these green NG-RAN(s), causing fewer resources can be granted to per UE.

The users of these UEs may observe lower service data rate compared to other UEs, with a pre-agreement with the MNO that they are willing to tolerate this situation as long as the communication service provided is more eco-friendly energy. To achieve this, a Green QoS Profile (GQP) and/or Green Alternative QoS Profile are provided by the network to the NG-RAN(s).

6.X.2.2	Green QoS profile (GQP)

In addition to the legacy QoS Profile (QP), additionally a Green QoS Profile (GQP) is also provided to the NG-RAN based on the energy usage condition of the NG-RAN from OAM. GQP is used for the NG-RAN(s) when it is using the renewable energy (or when renewable energy ratio > X%), and when NG-RAN(s) is not using renewable energy (or when renewable energy ratio <= X%) it fallbacks to use the legacy QP. Compared to the existing QoS Profile (QP), the main difference is

- Certain QoS parameter(s) in GQP is inferior to the one in QP, e.g., the Maximum bit rate of UL (MBR-UL) in GQP is less than the Maximum bit rate of UL in QP. The MNOs and Service providers can negotiate for the number of QoS parameters in GQP that are inferior to those in QP; or the parameter in GQP can be determined based on green-subscription/green-contract between subscriber and MNO.

The user, network and the service providers consider the GQP and QP with the same satisfaction but using in different NG-RAN conditions, e.g., when serving by non-green NG-RAN, the user demands 21Mbps MBR-UL, but when serving by green NG-RAN, the user can tolerate 14 Mbps MBR-UL.

The SMF can get the information when the NG-RAN can be powered by renewable energy from the OAM, and the SMF can provide QP and GQP to the NG-RAN based on the information.

When the NG-RAN gets the availability information of the renewable energy from the OAM, e.g., time validity information of the solar power, the NG-RAN can decide to switch to use the renewable energy based on the time validity and at that time, it can also decide to use the GQP provided by SMF.

The NG-RAN can also get the availability information of the renewable energy from the OAM for its neighbour NG-RAN(s), the NG-RAN can decide whether to re-direct the UE(s) based on the energy related information analytics from NWDAF.

Below are examples how the GQP and QP file are provided by the network:

Figure 6.X.2.2-1: example 1 for two NG-RANs (NG-RAN 1 using renewable energy and NG-RAN 2 using non-renewable energy)

In Figure 6.X.2.2-1, since NG-RAN 2 can get the conditions of the neighboring NG-RAN(s) from OAM, the NG-RAN 2 may decide to redirect the UEs to a NG-RAN using renewable energy. Furthermore, the SMF can also know the energy usage conditions from OAM for NG-RAN 1 and NG-RAM 2, the SMF can provide the QP and GQP to NG-RAN 1. Then NG-RAN 1 can treat the UE’s QoS flows based on QP and GQP.

Figure 6.X.2.2-2: example 2 for NG-RAN 1 using non-renewable energy at T1 and then using the renewable energy at T2. More UEs are redirected to access the NG-RAN 1

In Figure 6.X.2.2-2, since the NG_RAN 1 will change to use the renewable energy at time T2 and it is possible that more UEs will be redirected to NG-RAN 1. Also, the SMF can get the energy usage condition from OAM, the SMF may update the GQP for NG-RAN 1 at time T2. The NG-RAN can determine whether to use QP or GQP based on the condition of using renewable energy.

6.X.2.3	Green Alternative QoS profile (GAQP)

The concept of Green Alternative QoS profile is similar, it is provided to NG-RAN(s) using the renewable energy at specific time periods and/or location(s). Same concept applied:

- Certain QoS parameter(s) in GAQP is inferior to the one in legacy AQP. The users (e.g., during contract signing), MNOs and Service providers can negotiate for the number of QoS parameters in GAQP that are not equal to those in AQP.

The users, network and the service providers consider the GAQP and AQP as same level (e.g., same satisfaction) but using in different NG-RAN condition, e.g., GAQP is used by the NG-RAN using the renewable energy while AQP is used by the NG-RAN using non-renewable energy. A NG-RAN can be provided both GAQP and AQP since the NG-RAN may be powered by renewable energy at specific time and be powered by non-renewable energy at the other time.

The SMF can get the information when the NG-RAN can be powered by renewable energy from the OAM, and the SMF can provide QP and GQP to the NG-RAN based on the information.

When the NG-RAN gets the availability information of the renewable energy from the OAM, e.g., time validity information of the solar power, the NG-RAN can decide to switch to use the renewable energy based on the time validity and at that time, it can also decide to use the GQP provided by SMF.

The NG-RAN can also get the availability information of the renewable energy from the OAM for its neighbour NG-RAN(s), the NG-RAN can decide whether to re-direct the UE(s) based on the energy related information analytics from NWDAF.

Below are examples how the GQP, QP, GAQP and AQP files are provided by the network:

Figure 6.X.2.3-1: example 1 for two NG-RANs (NG-RAN 1 using renewable energy and NG-RAN 2 using non-renewable energy)

In Figure 6.X.2.3-1, since NG-RAN 2 can get the conditions of the neighboring NG-RAN(s) from OAM, the NG-RAN 2 may decide to redirect the UEs to a NG-RAN using renewable energy. Furthermore, the SMF can also know the energy usage conditions from OAM for NG-RAN 1 and NG-RAM 2, the SMF can provide the QP,GQP,AQP and GAQP to NG-RAN 1. Then NG-RAN 1 can treat the UE’s QoS flows based on the QP,GQP,AQP and GAQP.

Figure 6.X.2.3-2: example 2 for NG-RAN 1 using non-renewable energy at T1 and then using the renewable energy at T2. More UEs are redirected to access the NG-RAN 1

In Figure 6.X.2.3-2, since the NG_RAN 1 will change to use the renewable energy at time T2 and it is possible that more UEs will be redirected to NG-RAN 1. Also, the SMF can get the energy usage condition from OAM, the SMF may update the GQP and GAQP for NG-RAN 1 at time T2. The NG-RAN can determine whether to use QP or GQP and AQP or GAQP based on the condition of using renewable energy.

6.X.3	Procedures

For QP:

-	Whenever the QoS profile (QP) is provided/updated by the SMF to the AN via the AMF over the N2 reference point, additionally Green QoS profile (GQP) is also provided. QP can be include more QoS value which can be used when the NG-RAN switches to use the renewable energy

For AQP:

-	Whenever the SMF provides/updates a prioritized list of Alternative QoS Profile(s) to the NG-RAN, SMF also additionally provide Green AQP (GAQP). AQP can be include more QoS value which can be used when the NG-RAN switches to use the renewable energy

For NG-RAN:

-	If GQP is available, monitor current NG-RAN energy source (e.g., renewable, non-renewable) state, and determine to use either QP or GQP. If GQP is used, all logic is the same as if regard the GQP as QP.

-	If GAQP is also available, monitor current NG-RAN energy source (e.g., renewable, non-renewable) state, and determine to use either AQP or GAQP based on measured current energy source state. If GAQP is used, all logic is the same as if regard the GAQP as AQP.

Furthermore, the existing Notification control needs to be enhanced as follows:

- 	When the NG-RAN switches to use the renewable energy based on the availability information of the renewable energy from the OAM, and the NG-RAN is provided with GQP and/or GAQP by SMF, the NG-RAN will send a notification towards SMF for informing "the renewable energy- is applied/used" to notify the SMF it will apply the GQP or GAQP profiles for QoS handling.

-	When the NG-RAN applies GQP or GAQP, the same logic of the notification control for QP and AQP is applied to GQP and GAQP as specified in clause 5.7.2.4 in TS 23.501 [2]

6.X.4	Impacts on existing services, entities and interfaces

OAM:

- Needs to provide the energy usage condition of NG-RANs to 5GC NFs and also provides the energy usage condition of the neighbouring NG-RANs for a specific NG-RAN

NG-RAN:

- Needs to support GQP and optionally GAQP QoS profiles

- Needs to know its current energy source (e.g., renewable, non-renewable) state

- Needs to notify SMF which profile is used

- Needs to redirect the UEs to the NG-RAN using renewable energy

SMF:

- Needs to provide GQP and optionally GAQP to the NG-RAN

PCF:

- Needs to generate PCC rules to SMF for the GQP and optionally GAQP QoS profiles

*** End of changes ***