SA WG2 Meeting #S2-161	S2-2403207

26 Feb - 1 March, 2024, Athens, GR	(revision of S2-2402878)

Source:	Apple

Title:	KI#3: New Sol: User Identity profile exposure

Document for:	Approval

Agenda Item: 	19.8

Work Item / Release:	FS_UIA_ARC /Rel-19

Abstract of the contribution: This paper proposes a new solution to the KI#3 in 23.700-32 v0.1.0

# 1	Discussion

The Rel-19 User Identities and Authentication Architecture study has agreed on the Key Issue #3 pertaining to “Exposure of User Identity Profile Information”.

This pCR proposal aims to address the following questions of Key Issue #3

- what and how User Identity Profile information and functionality are exposed (e.g., exposure of the content of the User Identity Profile, exposure of authorization/authentication results, authenticating users, and linking a User Identifier with a 3GPP subscription).

# 2	Proposal

It is proposed to adopt the following changes into TR 23.700-32 v0.1.0.

## * * * * 1st Change (revision marked)

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | <Key Issue #1> | <Key Issue #2> | <Key Issue #3> | <Key Issue #4> | <Key Issue #5> |
| #X |  |  | X |  |  |

* * * * 2nd Change (all text new) * * * *

## 6.X	Solution #X: User Identity profile exposure

### 6.X.1	Key Issue mapping

This solution addresses KI#3 “Exposure of User Identity Profile Information”.

### 6.X.2	Description

This solution assumes the following:

- The procedures for configuring and provisioning User Identifiers to UE are out of SA2 scope (for example, obtaining Blinded tokens through application layer, provisioning though USIM Application).

- How the user instructs UE to use a particular User Identifier is up to UE implementation (for example, biometrics based identification)

The solution is based on the following principles:

The UE subscription data in the UDM is enhanced with the following:

- One or more User Identifiers, optionally associated with one or more GPSI (if not explicitly associated, User Identifiers are applicable for all GPSIs in the UE subscription data)

- Each User Identifier can be associated with

- one or more aliases, optionally associated with one or more AF Identifiers.

The same User Identifier could be configured as part of multiple UE subscription data.

A User Identifier could be for one-time use or can be reused multiple times.

PCC Rules may additionally contain:

-  one or more user identifiers.

UE policy may additionally contain:

- information specific to user identifiers.

### 6.X.3	Procedures

#### 6.X.3.1 Verification of User Identifier associated with a subscription

An AF requests NEF to verify contents of User Identity Profile and NEF would respond with a ‘Verified Successfully’ or ‘Not Verified’ answer.

### Pre-conditions: A user has registered a User Identifier with a UE subscription in 5GC. The UDM has marked the User Identifier as the active User Identifier for the UE subscription.  User, through the application client in the UE, has provided User Identifier to the Application Function (this could be either a User Identifier linked to their 3GPP subscription during registration in 5GC or an AF specific alias associated with the User Identifier).

The intention of AF is to use 5GC as an identity provider. User Identifier (or an associated alias specific to the AF) acts as the unique identifier provided by the identity provider after the UE has successfully registered with the 5GC and linked the User Identifier to the UE’s 3GPP subscription.

Figure 6.X.3.1-1 User Identifier verification by 5GC

0. User Identifier is marked as active for the UE subscription in the UDM (through one of the solutions for Key Issue#1).

1. AF invokes NEF Nnef_VerifyIdentifier service with the identifier (i.e., a AF specific alias or User Identifier) and UE address information (e.g., GPSI or SUPI).

2. NEF performs the necessary authorization and translation of internal-external information (e.g. the mapping between GPSI and SUPI if AF is in the untrusted domain).

3. NEF invokes the UDM Nudm_VerifyIdentifer service with AF provided identifier and SUPI.

4. UDM checks whether the AF provided identifier is currently linked to the UE’s 3GPP subscription. If the AF has provided an alias, UDM checks whether this is one of the configured aliases associated with User Identifier currently active for the UE’s 3GPP subscription.

5. UDM responds to the NEF with the result of the verification.

6. NEF responds with a success or failure indication indicating whether 5GC was able to successfully confirm if the AF provided identifier is indeed active for UE’s 3GPP subscription.

#### 6.X.3.2 Exposure of Authentication Results

In addition to the procedure described in clause 6.X.3.1, the authentication results may be exposed by using existing Nnef_EventExposure service operations as described in this clause.

  - 1. AF subscribes to the following event notifications, and provides the associated notification endpoint of the AF by sending Nnef_EventExposure_Subscribe Request to NEF:

  - Authorization/authentication results.

The AF includes the impacted UE information (e.g. GPSI, or SUPI) in the subscribe request.

2. NEF performs the necessary authorization control and translation of internal-external information (e.g. the mapping between GPSI and SUPI if AF is in the untrusted domain).

3. The NEF subscribes to notifications about User Identifier authorization/authentication results, and provides the associated notification endpoint of the NEF to UDM by sending Nudm_EventExposure Subscribe request to UDM. The impacted UE information (e.g., SUPI) is included in the subscribe request.

4. UDM detects the event occurs and sends the event report, by means of Nudm_VerifyIdentifier Notify message to the associated notification endpoint of the NEF. The UDM includes User Identifier and corresponding authentication results in the event report.

5. NEF forwards the received the event report to AF.

Editor’s Note: whether the service can have “any UE” as an input parameter is FFS.

Editor’s Note: Exposure of other information related to User Identity profile is FFS.

### 6.X.4	Impacts on services, entities and interfaces

NEF: Provides a service to

Verify whether a User Identifier or components of User Identity Profile is linked to 3GPP subscription of the UE (identified by a UE IP address or GPSI).

monitor authentication results of a User Identifier with the given UE.

Subscribes to notifications about the authentication, authorization of one or more User IDs.

Forwards notifications about authentication, authorization of one or more User IDs to AF.

UDM: Provides a service to

verify an identifier or components of user identity profile is linked to the UE’s (identified by SUPI) subscription.

monitor and send authentication results of a User Identifier with the given UE.

AF:

Subscribes to notifications about authentication, authorization of one or more User IDs

* * * * End of Changes * * * *