SA WG2 Meeting #161	S2-2403277

February 26 - March 1, 2024, Greece	 (revision of S2-2402253)

Source:	China Mobile

Title:	New Solution on Network support for NTZ management

Document for:	Approval

Agenda Item:	19.10

Work Item / Release:	FS_UAS_Ph3 / Rel-19

Abstract of the contribution: This contribution proposes a new solution for Key Issue #1 "Support of No Transmit Zones".

1. Discussion

This contribution is related to KI#1: Support of No Transmit Zones where including the content "whether mechanisms are needed to differentiate aerial UEs that support functions defined for NTZs in Rel. 19 and aerial UEs that don't". Here we want to discuss about the network support to manage the aerial UEs those don't support functions defined for NTZs.

Therefore, we should consider the scenario that the UE doesn't support functions defined for NTZs wants to perform flight mission via 5G network in NTZ. There is a need to align NTZ setting with 5G network. so 5G network can manage UE based on the right aerial information(NTZ). Meanwhile, the scenario that some areas are set to NTZ for temporary use should be considered. The 5GS must have solution to support NTZ to guarantee the security of NTZ.

2. Text Proposal

It is proposed to agree the following changes into TR 23.700-59.

# * * * * First change * * * *

4.1	Architectural Assumptions

The following architectural assumptions apply:

-	For solutions to enable network-assisted/ground-based mechanism for DAA (Detect And Avoid),

-	co-existence with and leveraging, to the extent possible, Direct DAA solutions specified in TS 23.256 [2] shall be considered.

-	sensing related information is out of scope of this study.

-	Regarding C2 communication reliability aspects in KI#1, only C2 over the Uu interface is considered in this study, and UAV using multiple-PLMN connectivity to support C2 communication reliability will not be considered in this release.

The following assumptions apply to the support of NTZ:

-	NTZ may be defined according to regional/national requirements;

-	an NTZ may map to one or more cells or a fraction of a cell, or overlap different cells in a mobile operator network.

NOTE:	NTZ scope and mapping to cell(s) need to be coordinated with RAN WGs.

Editor's note:  How to define the non-NTZ-support UE behaviour in NTZ is FFS.

# * * * * Second change * * * *

6.X	Solution #X: Network support for NTZ management

6.X.1	Key Issue mapping

This solution resolves KI#3.

6.X.2	Description

This solution addresses Key Issue #1 "Support of No Transmit Zones".

Considering the scenario that the UE doesn't support functions defined for NTZs wants to perform flight mission via 5G network in NTZ. There is a need to align NTZ setting with 5G network, so 5G network can manage aerial UEs based on the right aerial information (NTZ). In order to avoid the impact on non-aerial UEs, AMF needs to determine whether the UE is an aerial UE, and if yes, AMF can compare the NTZ information with the UE's local information to determine whether the user can initiate registration via 5G network.

6.X.3	Procedures

Figure 6.X.2-1: Procedure for network support for NTZ

0.	The AMF can store the NTZ information which is configured locally or obtained from the UTM/USS via UAS-NF/NEF.

1.	The UE, which doesn't support functions defined for NTZs, sends a Registration request message and, and it shall provide a CAA-level UAV ID of the UAV if configured with one.

2.	AMF could determine whether the UE is an aerial UE according to the CAA-level UAV ID or aerial subscription, and if yes, AMF can compare the NTZ information with the TA where the UE is to determine whether the UE can continue registration.

3.	AMF sends Registration Accept/Registration Reject message to UE.

4.	The AMF can update and store the new NTZ information configured locally or obtained from the UTM/USS via UAS-NF/NEF.

5.	The UE sends a mobility registration message due to the UE mobility.

6.	AMF could determine whether the UE is an aerial UE according to the CAA-level UAV ID or aerial subscription, and if yes, AMF can compare the NTZ information with the TA where the UE will move to, to determine whether the UE can continue mobility registration.

7.	AMF sends mobility registration Success/mobility registration Failure message to UE.

Editor’s Note: Applicability of whether an UAV can access the network while moving into or in a NTZ needs to be confirmed and any solution associated with such assumption will be further evaluated once clarification of this issue has been performed.

6.X.4	Impacts on Existing Nodes and Functionality

AMF:

-	Obtain data from UTM/USS referring to NTZ and store it as NTZ information.

# * * * * End of changes * * * *