3GPP TSG-SA WG2#161	S2-2403314

Athens, Greece, 26 February-1 March 2024	(was S2-2402706)

Source:	Qualcomm Incorporated

Title:	Network initiated IMS Data Channels

Document for:	Discussion/Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_Ph2

Abstract of the contribution: This solution provides a mechanism for the supporting of network initiated IMS DC.

# Discussion

The use case of network initiated IMS data channel is identified by GSMA NG UPG, and highlighted to 3GPP for supporting. More details are available in the LS to SA2 (S2-2400034).

This solution resolve the requirement of “UE is not pre-provisioned to initiate IMS Data Channel and network-initiated capability is necessary for supporting UEs”. In this solution, DC-AS in the enterprise is applied to trigger the establishment of IMS data channel(s).

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-77:

# >>>>BEGINNING OF CHANGES<<<<

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | X |  |  |  |  |  |  |  |
| 2 | X |  |  |  |  |  |  |  |
| 3 | X |  |  |  |  |  |  |  |
| 4 | X |  |  |  |  |  |  |  |
| 5 | X |  |  |  |  |  |  |  |
| 6 |  | X |  |  |  |  |  |  |
| 7 | X | X |  |  |  |  |  |  |
| 8 |  |  | X |  |  |  |  |  |
| 9 |  |  | X |  |  |  |  |  |
| 10 |  |  |  | X |  |  |  |  |
| 11 |  |  |  | X |  |  |  |  |
| 12 |  |  |  | X |  |  |  |  |
| 13 |  |  |  | X |  |  |  |  |
| 14 |  |  |  |  |  | X |  |  |
| 15 |  |  |  |  |  | X |  |  |
| 16 |  |  |  |  |  | X |  |  |
| 17 |  |  |  |  |  |  |  | X |
| Y |  | X |  |  |  |  |  |  |

# >>>>NEXT CHANGE<<<<

## 6.Y	Solution #Y: Supporting of network initiated IMS Data Channels

### 6.Y.1	Description

The use case of network initiated IMS data channel is identified by GSMA NG UPG, and highlighted to 3GPP for supporting. Following clarification is contained in the LS to SA2 (S2-2400034):

GSMA NG UPG#08 has identified the use case when network-initiated IMS Data Channel mid-call setup is required.

In some cases, e.g customer calsto retailer’s Sales Support or to the Customer Care number and service representative or automatic IVR wants to initiate interactive menu during the call. In this scenario UE is not pre-provisioned to initiate IMS Data Channel and network-initiated capability is necessary for supporting UEs..

In the current 23.228 clause AC.7.1 describing bootstrapping procedure Re-INVITE is already allowed for mid-call IMS Data Channel addition but network-initiated Re-INVITE may need additional clarifications.

GSMA expects this use case to be frequently used in B2C scenarios and the support of this capability could be crucial for initial IMS Data Channel deployment decisions.

This solution resolve the requirement of “UE is not pre-provisioned to initiate IMS Data Channel and network-initiated capability is necessary for supporting UEs”. In this solution, DC-AS in the enterprise is applied to trigger the establishment of IMS data channel(s).

### 6.Y.2	Procedures

Figure 6.Y.2 -1 depicts IMS DC establishment procedures requested by a DC AS, including the establishment of both bootstrap data channel and application data channel.

In this procedure, UE-A represents the customer of an enterprise. UE-B represents service representative or automatic IVR in the enterprise network. DC AS is deployed by the enterprise and accesses operator’s IMS network via NEF. The IMS session can be originated from UE-A or UE-B. This procedure only demonstrates a UE-A originated IMS session.

Figure 6.Y.2 -1: Network initiated IMS DC establishment procedures

0a. The DC AS subscribes to IMS AS for the event of IMS Session (e.g., session establishment, update, release, media update) to or from UE-B based on a list of IMPU.

Editor’s Note: How DC AS subscribes to IMS AS for the IMS session, and the details of  events notification are FFS.

0b.  UE-A successfully establishes an IMS Session to UE-B.

1.	The IMS AS notifies DC AS the IMS session event, including the negotiated media description and the IMS DC capability indication of the UEs.

NOTE 1:	Step 0 and 1 may be optional. In this case, DC AS trigger the DC request based on local policy.

2.	If the DC AS  learns from the notification that bootstrap data channel between UE-A and the DCSF is not established and IMS DC is supported by the UE-A, the DC AS may initiate a request to establish bootstrap data channel according to the local policy or interworking with UE-B. The DC AS sends the network initiated bootstrap data channel request to the NEF, together with the target IMS AS address received in step 1. If the DC AS request is authorized, the NEF forwards the request to the IMS AS. If the bootstrap data channel between UE-A and the DCSF is established and notified by the IMS AS, step 2-4 can be skipped.

NOTE 2:	This procedure only presents the bootstrap data channel between UE-A and the DCSF. Whether a bootstrap data channel between UE-B and the DCSF is required should be determined by DC AS.

NOTE 3: DC AS may trigger the establishment of application data channel together with bootstrap data channel in this step. If so, step 5-7 is skipped.

Editor’s Note: The interworking and services among DC AS, NEF and IMS AS are FFS.

3.	The IMS AS validates the user subscription data  and sends a notification to the DCSF for the policy of handling the requested data channel. The IMS AS instruction MF/MRF to allocate the resources in MF/MRF and sends the SDP Offer to UE-A.

Editor’s Note: More details of this step are FFS.

4.	The IMS AS replies DCS AS the successful bootstrap data channel establishment via NEF.

5.	The DC AS determines to establish data channel(s) for a specific application and sends the request with application binding information to IMS AS via NEF.

6. The IMS AS triggers the establishment of application data channel(s) between UE-A and the DC AS. Step 2 – 26 of clause AC.7.2.2 in TS 23.228 applies for the procedure.

NOTE:	This procedure only presents the P2A application data channel between UE-A and the DC AS. Whether other type of application data channel (e.g., P2P, P2A2P) is required should be determined by DC AS based on the selected application.

7. The IMS AS replies DCS AS the successful application data channel establishment via NEF.

### 6.Y.3	Impacts on existing services, entities and interfaces

NEF impacts:

-	Receive data channel request from AFs for establishment, update or release of bootstrap data channel and application data channel.

-	Send data channel request to IMS AS.

-	Receive notifications of requested IMS events from the IMS AS.

-	Send notifications of requested IMS events to the AF.

IMS AS impacts:

- Receive subscription of IMS session event.

- Send notification of IMS session event.

- Receive data channel request from NEF for establishment, update or release of bootstrap data channel and application data channel.

- Send response of data channel request.

# >>>>END OF CHANGES<<<<