SA WG2 Meeting #161	S2-2403321

Athens, Greece, Feb. 26 – Mar. 1, 2024	(revision of S2-2402560, 0901)

Source:	ZTE

Title:	KI #8, New Sol: Transition Between Avatar and Audio/Video Communication

Document for:	Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_Ph2 / Rel-19

Abstract of the contribution: A new solution for key issue #8 is proposed to address transition between Avatar Call and Audio/Video Call.

1. Discussion

The paper is proposed to address transition between an MMTel session using audio/video codec and IMS Avatar based communication in the Key Issue #8:

## 5.8	Key Issue #8: Support of IMS Avatar Communication

### 5.8.1	Description

This key issue aims to study enhancements of IMS architecture, interfaces, and procedures to support IMS based Avatar communication. This includes studying following aspects:

-	Define and identify the impacts from Avatar communication between two or more users in the context of IMS.

-	Study the identifiers required for IMS Avatar communication, e.g., identifier for an Avatar representation in IMS, and the association of an Avatar representation with a user.

-	Study whether and how Avatar objects such as an Avatar representation are stored and accessed by the authenticated and authorized UE and/or IMS network nodes avoiding fraud and ensuring privacy.

-	Study whether and how to authorise the use of an Avatar representation in an IMS Avatar communication.

-	Study whether and how to enable service/capability negotiation between UE and IMS network. This includes service/capability negotiation to enable transition, transcoding and rendering of media in an Avatar communication.

-	Study how to enable transition and transcoding between a MMTel session using audio/video codec and IMS Avatar based communication which may use a special Avatar codec.

-	Study how to enable transcoding between speech and gesture in an IMS Avatar communication.

-	Study how to enable UE based and network based rendering in case of IMS Avatar communication.

NOTE 1:	Transition, transcoding and rendering is based on UE/network capabilities and user preferences.

NOTE 2:	Transition, transcoding, rendering and service/capability negotiation aspects require coordination with SA4.

NOTE 3:	Security and privacy aspects require coordination with SA3.

This paper is revised and resubmitted to SA2 161 meeting based on the group discussion of S2-2400901 and the comments received in the last SA2 meeting, the changes in this version include:

Add architecture for Avatar communication and transition between audio/video call and Avatar call.

Remove the alternative transport of Avatar media through MRF and video channel.

Revision in SA2#161 with highlight text:

2. Proposal

It is proposed to capture the following changes into TS 23.700-77.

* * * * Start of Changes * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | X |  |  |  |  |  |  |  |
| 2 | X |  |  |  |  |  |  |  |
| 3 | X |  |  |  |  |  |  |  |
| 4 | X |  |  |  |  |  |  |  |
| 5 | X |  |  |  |  |  |  |  |
| 6 |  | X |  |  |  |  |  |  |
| 7 | X | X |  |  |  |  |  |  |
| 8 |  |  | X |  |  |  |  |  |
| 9 |  |  | X |  |  |  |  |  |
| 10 |  |  |  | X |  |  |  |  |
| 11 |  |  |  | X |  |  |  |  |
| 12 |  |  |  | X |  |  |  |  |
| 13 |  |  |  | X |  |  |  |  |
| 14 |  |  |  |  |  | X |  |  |
| 15 |  |  |  |  |  | X |  |  |
| 16 |  |  |  |  |  | X |  |  |
| 17 |  |  |  |  |  |  |  | X |
| X |  |  |  |  |  |  |  | X |

* * * * Second Change * * * *

6.X	Solution #X: Transition Between Avatar and Audio/Video Communication

6.X.1	Description

Two UEs can communicate with each other through either audio/video communication or Avatar communication. If one UE wants to switch from one of the communication modes to another, the UE triggers the negotiation with DCSF and peer UE for the Avatar media and establishes data channel. Media Function fetches Avatar representation from Avatar Repository. If peer UE is responsible for the rendering of Avatar representation, Media Function sends Avatar representation to peer UE.

UE captures sensing data of the user e.g., facial expressions data, body motion data and sends Avatar data (i.e. sensing data) to Media Function through data channel. Media Function renders the Avatar representation for the user based on the Avatar data received and sends rendered Avatar representation to peer UE. Peer UE displays the rendered Avatar representation to user. When peer UE is responsible for the rendering of Avatar representation, Media Function transfers Avatar data to peer UE; and peer UE renders the Avatar representation and displays to user.

6.X.2	Architecture for Avatar Communication

Figure 6.X.2-1 depicts architecture for Avatar communication.

Figure 6.X.2-1 Architecture for Avatar communication

Avatar communication architecture is based on the Data Channel architecture as specified in clause AC.2.1 in TS 23.228 [5].

In the architecture, new added functional entities include:

Media Function:

-	Support rendering Avatar representation based on received Avatar data;

-	Support retrieving Avatar representation from Avatar repository.

Avatar Repository:

-	Support storing Avatar representation.

In the architecture, enhanced functional entities include:

DCSF:

-	Manage bootstrap data channel and (if applicable) application data channel resources at Media Function via the IMS AS for Avatar communication.

IMS AS:

-	Interact with the DCSF and accordingly interact with the Media Function for data channel media resource management in Avatar communication.

In the architecture, the new added interfaces include:

Interface between DCSF and Avatar Repository.

6.X.3	Procedures

6.X.3.1	Transition from audio/video communication to Avatar communication

Figure 6.X.3.1-1 depicts a flow diagram for transition procedure from audio/video communication to Avatar communication.

Figure 6.X.3.1-1 Transition procedure from audio/video communication to Avatar communication

Editor's Note:  The Figure 6.X.3.1-1 may be restructured to reflect different Avatar rendering options.

The steps in the transition procedure are as follows:

0. UE A has audio/video communication with UE B.

1. UE A exchanges audio/video media with UE B through a Media Resource Function (MRF) or directly over RTP.

2. UE A decides to switch the communication from audio/video to Avatar communication mode.

3. UE A interacts with DCSF and UE B to negotiate the Avatar media and establish data channel.

This step is similar to Person-to-Person application Data Channel set up procedure as specified in clause AC.7.2.1 of TS 23.228.

Editor’s Note: The detail interaction between UE and DCSF for establishment of Avatar communication through Data Channel is FFS.

4. DCSF fetches Avatar representation from Avatar Repository and sends it to Media Function.

Avatar representation is specific for the user of UE A; it can be pre-configured in Avatar Repository or uploaded by the user.

4'. Media Function sends Avatar representation to UE B.

This step (4') is optional. Step 4' is performed only when UE B is responsible for the rendering of Avatar representation. The Avatar representation is a raw Avatar representation sent from Media Function to UE B.

NOTE 1:	The negotiation of Avatar rendering responsibility (Media Function or UE) will be specified by SA4.

5. UE A captures sensing data of the user e.g., facial expressions data, body motion data.

6. UE A sends Avatar data (i.e. sensing data) to Media Function through data channel.

7a. Media Function uses the Avatar representation received at Step 4 to render the Avatar representation based on the Avatar data received.

8a. Media Function sends rendered Avatar representation to UE B through data channel.

If Step 4' is performed, Step 7a and 8a will not be performed because UE B will render the Avatar representation of the user based on the Avatar data (which the Media Function sends to UE B) and based on the raw Avatar representation received by UE B at Step 4'. In this option, UE B is responsible for the rendering of Avatar representation and Step. 4' is performed, then:

7b. Media Function transfers Avatar data to UE B.

8b. UE B renders the Avatar representation for user based on the Avatar data received.

9. UE B displays rendered Avatar representation to user.

10. UE B also sends its Avatar representative to UE A through Media Function; and the video between two UEs is stopped.

6.X.3.2	Transition from Avatar communication to audio/video communication

Figure 6.X.3.2-1 depicts a flow diagram for transition procedure from Avatar communication to audio/video communication.

Figure 6.X.3.2-1 Transition procedure from Avatar communication to audio/video communication

The steps in the transition procedure are as follows:

0. UE A has Avatar communication with UE B.

1. UE A exchanges Avatar media (i.e. Avatar representative) with UE B transferred by Media Function through data channel.

2. UE A decides to switch the communication from Avatar to audio/video mode.

3. UE A negotiates with UE B for exchanging video media between UE A and UE B.

4. UE A exchanges video media with UE B through MRF or directly over RTP.

The video media in step. 3 and 4 is the real video of users and is captured by the camera of UEs, rather than Avatar media.

5. UE A negotiates with DCSF and UE B to stop exchanging Avatar media and may release data channel.

6.X.4	Impacts on Existing Nodes and Functionality

UE:

-	Support capturing sensing data of users and sending it as Avatar data to Media Function;

-	Support displaying Avatar representation.

-	Optionally support rendering Avatar representation.

Media Function:

-	Support rendering Avatar representation based on received Avatar data;

-	Support retrieving Avatar representation from Avatar repository.

Avatar Repository:

-	Support storing Avatar representation.

DCSF:

-	Manage bootstrap data channel and (if applicable) application data channel resources at Media Function via the IMS AS for Avatar communication.

IMS AS:

-	Interact with the DCSF and accordingly interacts with the Media Function for data channel media resource management in Avatar communication.

* * * * End of Changes * * * *