SA WG2 Meeting #161	S2-2403425

Athens, GREECE, February 26 – March 01, 2024	(revision of S2-2402487)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4758 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Several corrections of URSP rule delivery in EPS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eUEPO |  |  |  |  |  | Date: |  |  | 2024-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This paper fixes some of the errors in URSP rule delivery in EPS. Firstly, in 4.11.0a.2a.1, this is the change of SM policy association establishment from 5GS to EPS. But it is incorrect because the this is not the UE attachment in EPS and the SMF+PGW-C doesn’t aware whether UE supports the URSP rule provision. If UE doesn’t support the indication, the PCF for session doesn’t establish the UE policy association in EPS. Secondly, in 4.11.0a.2a.2.3, it is misleading to say “When the UE reports the result of URSP rule update”. It should be UE request URSP rule if the UE policy container is included in bearer resource modification. Also, it is incorrect that “the PCF for the PDU Session selects one of the ongoing PDU Sessions for this UE that supports URSP Rule delivery in EPS”. It should be the PDN connection that supports URSP rule delivery in both UE and network. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Fix some of the incorrect part listed in reason of changes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some of the details in URSP rule provision in EPS is misleading. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.11.0a.2a.1, 4.11.0a.2a.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

4.11.0a.2a.1	SM Policy Association Establishment

The following additions are applicable to clause 4.16.4 (SM Policy Association Establishment procedure):

-	In addition to step 4 the PCF determines whether the PDN connection supports the provisioning of URSP rules in EPS by checking URSP delivery in EPS support indication provided by the SMF+PGW-C. The SMF+PGW-C provides that indication based on local capabilities and operator policies. The PCF may receive the URSP delivery in EPS support indication provided by the SMF+PGW-C and store the indication for the following use (for example, URSP rule provision to UE during 5GS to EPS mobility).

* * * Start of Change 2 * * *

4.11.0a.2a.2.3	SMF initiated SM Policy Association Modification for UE requests to update URSP rule

When the UE requests to update URSP rule that UE requested bearer resource modification without QoS update procedure and includes the UE Policy Container ePCO in the Request Bearer Resource Modification message, the following enhancements are applicable to clause 4.16.5.1 (SM Policy Association Modification procedure):

In addition to step 4 in clause 4.16.5.1 if the PCF for the PDU Session receives the "UE Policy" Policy Control Request Trigger and associated UE Policy Container, steps 4a to 4b below are executed:

-	Step 4a: The PCF for the PDU Session determines that a UE Policy association has been already established towards the PCF for the UE for the SM Policy Association as described in clause 4.11.0a.2a.7.

-	Step 4b: The PCF for the PDU Session forwards the UE Policy Container to the PCF for the UE in Npcf_UEPolicyControl_Update Request.

* * * End of Change * * *